| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_NBR-ADV message (16.2.3.12) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Feng Xie, Yang Liu, Lin Chen, Zhaohua Lu, ZTE Corporation | E-mail: {xie.feng2, liu.yang8}@zte.com.cn |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.3.12 & 16.2.6.4.2.1” |  |
| Abstract | This contribution proposes the texts for AAI_NBR-ADV (16.2.3.12) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_NBR-ADV message (16.2.3.12)

Feng Xie, Yang Liu, Lin Chen, Zhaohua Lu

ZTE Corporation

# 1. Introduction

# LB31 cmt #A0153 is accepted in the last session to modify the AAI_NBR-ADV message. However, some details need further clarification.

# In addition, LB31 cmt A0174 is accepted-modified with the solution of #A0153. How the new added parameters in AAI_NBR-ADV message should be also included.

# 2. Text Proposal

Remedy 1:

Accept the following Modifications to 16.2.3.12 AAI_NBR-ADV:

Line 21 on Page 110

======================== Start of Proposed Text =====================

For each cell type, shared configuration parameters are carried as follows

•Operator ID

•IDcell SA-Preamble Index range

•FA Physical carrier range

The Operator ID, IDcell SA-Preamble index range and FA Physical carrier ranges are optional fields, which can be used when the serving ABS chooses not to broadcast configuration information for each individual ABS within the cell type.

Within each cell type, if serving ABS chooses to broadcast configuration information for each individual ABS instead of specifying IDcell SA-Preamble index range and FA physical carrier range, a list of ABSs are provided and the following parameters are carried for each ABS

• 48bit BS-ID

• ABS IDcell SA-Preamble Index

============================== End of Proposed Text ===============

Remedy 2:

Accept the following Modifications to 16.2.3.12 AAI_NBR-ADV:

In the P802.16m/D5, page 111, line 1, replace Table 693 to the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 693 — AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | NA |
| M | Cell type | 34 | Cell type in this message 0b0000 macro 0b0001 micro 0b0010 macro hotzone 0b0011 pico femto 0b0100 OSG femto 0b0101 CSG-Open femto without CSGID 0b0110 CSG-Closed femto without CSGID 0b0111 CSG femto with CSGID 0b100 0b1000 relay 0b1001-0b1111 reserved | NA |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | NA |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | NA |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | NA |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. | NA |
| O | MSB of BSID | variable | Prefix of BSID which may include NAP ID | For ABS type of shared configuration information |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| O | IDCell SA-preamble index range start | 10 | Lower bound of SA-preamble index range | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| O | IDCell SA-preamble index range end | 10 | Upper bound of SA-preamble index range | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| MO | BSID | 48 | - | Shall be included for each neighbor BS when MSB of BSID is not present |
| M | MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 10 defined for 16m. | Shall be included for each neighbor BS |
| M | CP time | 2 | CP time of the BS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| M | SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |
| M | S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS | Shall be included for each carrier of each neighbor BS |
| M | SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the bitmap shall be either 0b00 or 0b01 or 0b11. | Shall be included for each neighbor BS |
| M | Control_bitmap | 4 | Each bit maps to one SFH subpacket Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: reserved | Shall be included for each neighbor BS |
| O | SFH Subpkt 1 | Variable depending on FFT size | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #0 is set to 1 |
| O | SFH Subpkt 2 | Variable depending on FFT size | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #1 is set to 1 |
| O | SFH Subpkt 3 | variable | - | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #2 is set to 1 |
| O | Delta information | Var. | Delta encoding, w.r.t. the reference carrier the current carrier transmitting this message if SFH_encoding format = 01, the preceding carrier if SFH_encoding_format=11) | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b01 or 0b11 |
| O | Neighbor-specific trigger TLVs | Var. | Optional neighbor-specific triggers with encoding defined in Table 744 | May be included for each neighbor BS |
| O | MOB_NBR-ADV period | 10 | The period periodicity of MOB_NVR-ADV MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | MOB_NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | MOB_NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS are indicated in AAI_NBR-ADV. |

============================== End of Proposed Text ===============

Remedy 3:

Accept the following Modifications to 16.2.6.4.2.1:

Line 27 on Page 288

======================== Start of Proposed Text =====================

16.2.6.4.2.1 Network Topology Acquisition

16.2.6.4.2.1.1 Network Topology Advertisement

…...

An AMS, by reading the indication of existence of neighbor R1 BS or LZone of ABS in AAI_NBR-ADV, always obtains the neighbor R1 BS information from the MOB_NBR-ADV message transmitted in its serving ABS LZone. The indication is the existence of MOB NBR-ADV period, MOB_NBR-ADV offset, and MOB_NBR-ADV interval contained in AAI_MOB-ADV. AMS listens for MOB_NBR-ADV in LZone in frame N which meet:

mod(N, MOB_NBR-ADV period)= MOB_NBR-ADV offset+i, where i=0,…,MOB_NBR-ADV interval-1, .

The AMS’s operation in MZone should not be impacted by the switch for MOB_NBR-ADV in LZone.

============================== End of Proposed Text ===============