| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Identification and Selection of Femtocells for AMS w/o CSG White List(16.4) |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu ZTE Corporation | E-mail: {xie.feng2, chen.lin23, lu.zhaohua, liu.yang8}@zte.com.cn |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.4” |  |
| Abstract | This contribution proposes the texts for clarification on AMS’s identification and selection of femtocells w/o CSG white list (16.4) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Identification and Selection of Femtocells for AMS w/o CSG White List (16.4)

Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu

ZTE Corporation

# 1. Introduction

CSG white list is critical for an AMS who subscribed to CSG femtos to identify and select accessible CSG femtos with or without UL signaling. On the other hand, there are no mechanisms in D5 to support accessible CSG femtos selection by those AMSes if without CSG white list.

Generally there are two ways to remedy the spec. One is to mandate CSG white list, which is preferrable. Alternatively, prohibit an AMS who subcribed to CSG femtos to select CSG femtos during INE or preferred ABS selection in idle mode. That means those AMS shall follow the instruction of macro ABSs or open femtos to handover to their subscribed femtos.

# 2. Text Proposal

Remedy 1:

Accept the following Modifications to 16.4.4.2 CSG White list:

Page 695, Line 54:

======================== Start of Proposed Text =====================

16.4.4.2 CSG White list

The CSG white list, is a list of Femto ABSs to which the AMS is subscribed and can access. These femto ABSs are distinguished based on the common identifier defined in 16.4.4.

AMS's local white list may contain the allowable BS IDs or common identifiers of CSGs and relevant information to help derivation of the allowable BS IDs from common identifier.

An AMS subscribed to CSG(s) should be configured with a CSG White List for its accessibility check. The CSG white list may be provided to the AMS by the Femto service provider through the network using messaging that is outside the scope of this standard.

============================== End of Proposed Text ===============

Remedy 2:

Accept the following Modifications 16.4.7.3 Femto ABS access restrictions:

Page 773, Line 40

======================== Start of Proposed Text =====================

16.4.7.3 Femto ABS access restrictions

AMS should not attempt to access or initiate handover to a CSG-closed Femto ABS, which is not contained in the CSG whitelist or the AMS does not have CSG white list at all, except in case of emergency call or manual Femto BS selection (see 16.4.7.2). If cell bar bit in CSG-Open Femto ABS is set to 1, non member AMS shall not attempt network entry or handover to the CSG-Open Femto ABS.

============================== End of Proposed Text ===============

Remedy 3:

Accept the following Modifications to 16.4.9 Idle Mode:

Page 702, Line 40

======================== Start of Proposed Text =====================

16.4.7.1 Idle Mode

Femto ABS shall support idle mode.

The Femto ABSs operate like macro ABSs in Idle mode.

An AMS with CSG white list shall not attach to an unsubscribed CSG-Closed Femto ABS in Idle mode. And an AMS without CSG white list shall not attach to any CSG-Closed Femto ABS in Idle mode.

============================== End of Proposed Text ===============