| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning of Neighbor Femto ABSs (16.4.8.1.2) |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Feng Xie, Lin Chen, Yang Liu, Zhaohua Lu, ZTE Corporation | E-mail: {xie.feng2, chen.lin23, liu.yang8, lu.zhaohua}@zte.com.cn |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.4.8.1.2” |  |
| Abstract | This contribution proposes the texts for AMS scanning procedures for neighbor Femto ABSs (16.4.8.1.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Scanning of Neighbor Femto ABSs (16.4.8.1.2)

Feng Xie, Lin Chen, Yang Liu, Zhaohua Lu

ZTE Corporation

# 1. Introduction

In current P802.16m/D5, the procedures of AMS scanning of femto ABSs need clarifications.

In this proposal, the procedures of AMS scanning of femto ABS are described.

# 2. Text Proposal

Insert the following paragraph to Line 18 of Page 775

16.4.8.1.2 AMS scanning of neighbor femto ABSs:

======================== Start of Proposed Text =====================

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 16.2.6.1.2 with exceptions described in this subsection. An AMS may scan femto ABSs according to the FAs included in the broadcast AAI_NBR-ADV message. AMS may scan femto ABSs that are not included in AAI_NBR-ADV based on SA-preamble partitioning information (see 16.4.4). In addition, an AMS may scan allowed femto ABSs based on the CSG White List. Based on location information, AMS may initiate the scanning procedure (See 16.2.6.1.2). The location information may include absolute/relative location information of CSG Femto ABS, such as GPS info, overlay Macro ABS BSID. The AMS may request additional scanning opportunity by sending AAI-SCN-REQ including the detected SA-preamble index and FA information. AAI_SCN-REQ message may not include those CSG femto ABSs if the detected CELL_IDs do not match the CELL_IDs maintained in AMS CSG white list. Upon reception of the AAI_SCN-REQ, the ABS shall respond with an AAI_SCN-RSP including neighbor accessible Femto ABS list based on the SA-preamble index.

============================== End of Proposed Text ===============