| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Usage of AAI_NBR_REQ message (16.2.6.1.1) |  |
| Date Submitted | 2009-04-30 |  |
| Source(s) | Lin Chen, Feng Xie, Yang Liu, Zhaohua Lu, ZTE Corporation | E-mail: {chen.lin23, xie.feng2, liu.yang8, lu.zhaohua, }@zte.com.cn |
| Re: | Call for LB #31b on “ P802.16m/D5”: Target topic: “16.2.6.1.1” |  |
| Abstract | This contribution proposes the texts for the usage of AAI_NBR-REQ message (16.2.6.1.1) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The usage of AAI_NBR-REQ message (16.2.6.1.1)

Lin Chen, Feng Xie, Yang Liu, Zhaohua Lu

ZTE Corporation

# 1. Introduction

In current P802.16m/D5, the AAI_NBR-REQ message may contain the request type indication to request the neighbor BS type information with the specified type, especially the CSG femto ABS and the OSG femto ABS. However, when and how to use this request BS type indication to request CSG/OSG femto ABS info is not described in the section 16.2.6.1.1 of network topology advertisement. In this contribution, we present the detailed procedure of the request of CSG/OSG femto ABS information through the AAI_NBR-REQ message.

# 2. Proposed text with change marks

Accept the following Modifications for section 16.2.6.1.1 in page 270, line 3:

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.2.6.1.1 Network topology advertisement

An ABS shall periodically broadcast the system information of the neighboring ABSs using an AAI_NBR-ADV message. A broadcast AAI_NBR-ADV message may include the information of Open Subscriber Group Femto and/or macro hotzone ABSs, but shall not include information of neighbor Closed Subscriber Group (CSG) Femto ABSs. A broadcast AAI_NBR-ADV message may omit information of neighbor cells with cell bar information=1 in its S-SFH.

A serving ABS may unicast the AAI_NBR-ADV message to an AMS upon reception of the AAI_NBR-REQ message or in unsolicited manner. When the AMS need to obtain the neighbor information of CSG or OSG femto ABS, it may indicate it through the request BS type in the AAI_NBR-REQ message. Upon receiving such AAI_NBR-REQ message, the serving ABS sends the neighboring CSG or OSG femto ABS information through the AAI_NBR-ADV message to the AMS. The AAI_NBR-ADV message may include parameters required for cell selection e.g., cell load and cell type. The ABS may broadcast different fragments of AAI_NBR-ADV message over multiple MAC PDU’s.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]