| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Network Discovery and Selection Procedures for femto ABS (16.4.7.1) |  |
| Date Submitted | 2009-05-07 |  |
| Source(s) | Lin Chen, Feng Xie, Yang Liu, Zhaohua Lu, ZTE Corporation | E-mail: {chen.lin23, xie.feng2, liu.yang8, lu.zhaohua, }@zte.com.cn |
| Re: | Call for LB #31b on “ P802.16m/D5”: Target topic: “16.4.7.1” |  |
| Abstract | This contribution proposes the texts for the clarification of the network discovery and selection procedures for femto ABS (16.4.7.1) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Network Discovery and Selection Procedures for femto ABS (16.4.7.1)

Lin Chen, Feng Xie, Yang Liu, Zhaohua Lu

ZTE Corporation

# Introduction

The current P802.16m/D5 lacks the ending condition for the network discovery and selection procedure of femto ABSs.

In this proposal, the ending condition for the femto ABS detection, identification and selection procedure are described.

# 2. Proposed text with change marks

Accept the following Modifications for section 16.4.7.1 in page 771, line 19:

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

16.4.7.1 Femto ABS detection, identification and selection

Figure 581 shows the procedure AMS may follow for discovery of and association with Femto ABS. The figure only serves an illustration of the procedure inside AMS.

An AMS may prefer its subscribed CSG Femto ABS while other ABSs may also be chosen as candidates.

If the detected Cell_ID belongs to the set of CSG-open Femto ABSs and the AMS is not a member of any CSG, the AMS may continue the network entry with this ABS, if the AMS cannot be served by other neighboring macro or OSG Femto ABSs.

During network entry, the AMS shall acquire the DL PHY synchronization by A-Preamble. The AMS identifies the type of ABS based on the detected Cell_ID and the Cell_ID partitioning information. The AMS may select the ABS depending on the preference on the ABS types of the AMS. Cell_ID and associated carrier index, and the LSB of BS-ID (or the MSB of BS-ID if SP2 is decoded early prior to SP1) may help the AMS to quickly exclude the CSG Femto ABSs to which it is not subscribed. Full BSID (48 bits) or the derived CSGID is the exact identifier for the AMS to determine whether it is authorized to access to the target ABS. The AMS have the common identifiers in its White List (i.e., a local table in the AMS containing the identities of all the CSG femto base stations to which the AMS is subscribed and is authorized to access) in order to be authorized to access to the target Femto ABS. If the femto ABS is excluded, the AMS continue the scanning until a suitable ABS is detected.

The procedure illustrated in Figure 581, provides the procedures that the AMS may follow to complete cell selection/re-selection when Femto and Macro base stations are deployed on the same or a different FA. If the Femto base stations are deployed in a different FA (inter-FA), the same procedure may be applicable except that the AMS scans a different frequency band and conducts RF measurements during scanning in that frequency. The other procedures remain intact and the detected Cell_IDs will only belong to open and/or closed subscriber group femto base stations.

If the Femto and Macro base stations are deployed in the same FA (intra-FA), the same procedure may be applicable and the AMS conducts RF measurements during scanning in the same frequency. Combination of the inter-FA and intra-FA scanning is also possible where the neighbor macro base stations operate in the same frequency band and Femto base stations operate in a different frequency band(s).

[-------------------------------------------------End of Text Proposal----------------------------------------------------]