| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The clarification of dynamic ranging channel (16.3.6) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Jinyoung Chun, Inuk Jung, Jin Sam Kwak LG Electronics Chiwoo Lim, Jeongho Park Samsung Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31a |  |
| Abstract | This contribution provides the clarification of dynamic ranging channel. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of Dynamic Ranging Channel (16.3.6)

Jinyoung Chun, Inuk Jung, Jin Sam Kwak

LG Electronics

Chiwoo Lim, Jeongho Park

Samsung Electronics

- Introduction

Dynamic ranging channel is allocated by Broadcast Assignment A-MAP IE. But the subframe index and frequency location for dynamic ranging channel is not clarified. Therefore I suggest the proposed text to clarify it.

- Text proposal in the P802.16m/D5

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 34 of page 591 as below:]

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 862 describes the fields in a broadcast assignment A-MAP IE.

Table 862 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers indicates the existence of dynamic ranging channel in the (n+1)th frame when this IE is transmitted in (n)th frame. allocation information |
| If (Function Index == 0) { |  |  |
| Burst Size | 6 | Burst size isas indicated in the first 44 entries in Table 956 |
| Resource Index | 11 | 5MHz: 0 in first 2 MSB bits + 9 bits for resource index 10MHz: 11 bits for resource index 20MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst will be transmitted one more time 0b10: the same burst will be transmitted two more time 0b11: the same burst will be transmitted three more time |
| Reserved | TBD | Reserved bits |
| } else if (Function Index == 1) { |  |  |
| Number of Ranging Opportunities (N) | 2 | TBD |
| for (i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |
| } |  |  |

[Modify the text in line 1 of page 665 as below:]

16.3.8.3.3 Resource allocation and tile permutation for control channels

The distributed LRUs in each of uplink frequency partition may be further divided into data, bandwidth request, and feedback regions. A feedback region consists of feedback channels can be used for both HARQ ACK/NACK and fast feedback. In a multicarrier system with active DL-only secondary carriers, the primary UL carrier contains multiple feedback regions for the primary DL carrier and the active DL-only secondary carriers. The primary UL carrier contains one feedback region corresponding to the primary DL carrier and each active DL-only secondary carrier. The allocation order of data channels and UL control channels are UL HARQ feedback channels, UL fast feedback channels, UL bandwidth request channels and UL data channels as Figure 541.

When frame structure is supporting the WirelessMAN-OFDMA with FDM-based uplink PUSC zone, the ranging channel is located in the lowest DLRU index, which is followed by UL HARQ feedback channels, UL fast feedback channels, UL bandwidth request channels, and UL data channels as Figure 540. If there is no regular ranging channel in a subframe, the uplink control channel starts from lowest index as Figure 541. If there is a UL dynamic ranging channel in a frame, it is located right after the UL control channels.

[Modify the text in line 44, page 695 as below:]

The NS-RCH for handover ranging can also be allocated by Broadcast Assignment A-MAP based on ABS scheduling decision any OSFth AAI subframe, except the AAI subframe that has already been used for a regular allocation.

-----------------------------------------------   Text End  -----------------------------------------------------------