| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Service Specific Scheduling Control Header (16.2.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Yongho Kim and Jin Sam Kwak LG Electronics | E-mail: giwon.park@lge.com |
| Re: | Call for LB #31a on “ P802.16m/D6”: Target topic: “16.2.2” |  |
| Abstract | This contribution proposes clarification text for Service Specific Scheduling Control Header |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on Service Specific Scheduling Control Header (16.2.2)

Giwon Park, Kiseon Ryu, Yongho Kim and Jin Sam Kwak

LG Electonics

- Introduction

D5 does not clearly describe the Request/Response mechanism and SCID switching mechanism for SSSCH.

- Request/Response indication shall be included in SSSCH to clear request/response operation.

- Start frame offset for new sleep cycle configuration shall be included in SSSCH for AMS adopting the switched SCID.

- If the AMS/ABS sends the signaling header to ABS/AMS for the purpose of changing the parameters, AMS/ABS should send an acknowledgement through the same signaling header.

- When the AMS sends the SSSCH to switch AMS's sleep cycle setting, the ABS should modify the AMS’s sleep cycle setting switching request with SSSCH including a different SCID to the AMS.

Therefore, we propose the clarification of the Service Specific Scheduling Control Header.

- Proposed Test

--------------------------------Text start-----------------------------------------

[Modify the sentence and table 670 (from line 27 on page 49) of P802.16m/D5 as follows]

16.2.2.1.3.3 Service Specific Scheduling Control Header

Service Specific Scheduling Control Header is sent either by AMS through dedicated UL resource or by ABS, which needs to change or acknowledge the scheduling parameters of its service flow. If the Service Specific Scheduling Control Header is sent from AMS, BR type shall be included to indicate whether BR type is increment or aggregate, and it indicates Adaptation Request when it is sent for aGP service flow.  In this case, an AMS shall set the Response indication in the SSSCH to 0 (i.e. indicating a request). The ABS may acknowledge or modify the QoS parameters for aGP service flow by sending the Service Specific Scheduling Control Header in downlink, which indicates Adaptation Response. In this case, an ABS shall set the Response indication in the SSSCH to 1 (i.e. indicating a response). Service Specific Scheduling Control header format is defined in Table 659. If SCID change indicator is set to 1, the ABS shall send an acknowledgement to confirm the change of sleep mode configuration. If the ABS accepts the change of sleep mode configuration, the ABS shall send a MAEH as acknowledgement. In case that a SSSCH is used for the response to the SSSCH, an AMS and an ABS shall set the Response indication in the SSSCH to 1 (i.e. indicating a response). The ABS may modify the AMS's sleep cycle setting switching request. In this case, the ABS shall reply with SCH or SCEH SSSCH including a different SCID to the AMS.

Table 659—Service Specific Scheduling Control BR without STID Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Service Specific Scheduling Control Header() {. |  |  |
| FID | 4 | Flow Identifier. Set to 0001 |
| Type | 4 | MAC signaling header type = 0b0010 |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response to the request |
| BR Type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: aggregate 1: incremental |
| … |  |  |
| if (scheduling type == aGP Service) { |  |  |
| QoS parameter change indicator | 1 | QoS parameter change indicator 0: no change in QoS parameter 1: having changes in QoS parameter |
| SCID change indicator | 1 |  |
| if (QoS parameter change indicator = 1){ |  |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new GPI (frames) to use for future allocation. |
| }else{ |  |  |
| QoS parameter set switch | 1 | 0: primary QoS parameter set. 1: secondary QoS parameter set |
| } |  |  |
| Iif(SCID change indicator==1) |  |  |
| { |  |  |
| SCID | 4 |  |
| Start Frame Offset for new sleep cycle configuration | 3 | Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } |  |  |
| … |  |  |
| } |  |  |

--------------------------------text end-----------------------------------------