| 2Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications on DSx MAC control message (D5 - 16.2.3.45) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution proposes some modification on section 16.2.3.45 for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications on DSx MAC control message

(D5 - 16.2.3.45)

Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for clarifications for DSx MAC control message in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D5, page 183, line 1, make the following changes to subclause 16.2.3.45.1]

16.2.3.45.1 AAI_DSA-REQ

Table 740—AAI_DSA-REQ

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | QoS parameters | Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
| O |  | Unsolicited Grant Interval parameter | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O |  | Unsolicited Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O |  | Primary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O |  | Primary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O |  | Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  | Secondary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  | Adaptation Method | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only | Present when UL scheduling type is aGP Service |
| O |  | Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. | Present for UL |
| O |  | Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) | Present for UL |
| O |  | Quick Access Predefined BR Index Parameter | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O |  | MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |  |
| O |  | Vendor Specific QoS Parameter | variable |  |  |
| O | Type of Data Delivery Services parameter |  | 8 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |  |

[On the IEEE P802.16m/D5, page 189, line 33, make the following changes to subclause 16.2.3.45.2]

Table 741—AAI_DSA-RSP

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Physical Carrier Index |  | 6 | Target carrier which the AMS switches or is redirected by ABS to |  |
| O | SFID |  | 32 |  | Only included in response to AMS initiated DSA-REQ. |
| O | FID |  | 4 |  | Only included in response to AMS initiated DSA-REQ. |
| O | QoS parameters | Access Class | 2 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |  |
| O |  | Differentiated BR Timer | 6 | The mapping of predefined BR index used in quick access message to BR size and BR actions | Only included in response to AMS initiated DSA-REQ for UL. |
| O |  | Quick Access Predefined BR Index Parameter | variable | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |  |

[On the IEEE P802.16m/D5, page 190, line 18, make the following changes to subclause 16.2.3.45.4]

—Predefined BR index: Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure.

[On the IEEE P802.16m/D5, page 190, line 52, make the following changes to subclause 16.2.3.45.4]

Table 743—AAI_DSC-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | NA |
| O M | SFID | 32 |  | NA Present when an ABS initiates AAI_DSA-REQ |
| O | FID | 4 |  | Present when an ABS initiates AAI_DSA-REQ |

[On the IEEE P802.16m/D5, page 193, line 1, make the following changes to subclause 16.2.3.45.4]

Table 743—AAI_DSC-REQ

| M/O | Attributes / Array of attributes |  | Size (bits) |  | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | QoS parameters | Tolerated Jitter parameter | 32 | Milliseconds |  | Present when needed |
| O |  | Unsolicited Grant Interval parameter | 16 | Milliseconds |  | Present when needed |
| O |  | Unsolicited Polling Interval parameter | 16 | Milliseconds |  | Present when needed |
| O |  | Primary Grant Polling Interval parameter | 16 | Milliseconds |  | Present when needed |
| O |  | Primary Grant Size parameter | 16 | Number of bytes. |  | Present when needed |
| O |  | Secondary Grant Polling Interval parameter | 16 | Milliseconds |  | Present when needed |
| O |  | Secondary Grant Size parameter | 16 | Number of bytes. |  | Present when needed |
| O |  | Adaptation Method | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only |  | Present when needed |
| O |  | Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. |  | Present when needed |
| O |  | Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) |  | Present when needed |
| O |  | Quick Access Predefined BR Index Parameter | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions |  | Present when needed |
| O |  | MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved |  |  |
| O |  | Vendor Specific QoS Parameter | Variable |  |  |  |
| O | Type of Data Delivery Services parameter |  | 8 | 0: Unsolicited Grant Service 1: Real-Time Variable Rate Service 2: Non-Real-Time Variable Rate Service 3: Best Effort Service 4: Extended Real-Time Variable Rate Service |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------