| 2Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications on AAI_RNG-ACK (D5 - 16.2.3.3) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution proposes some modification on section 16.2.3.3 for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications on AAI_RNG-ACK (D5 - 16.2.3.3)

Heejeong Cho, Ki Seon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for clarifications for AAI_RNG-ACK in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D5, page 84, line 62, make the following changes to subclause 16.2.3.3]

16.2.3.3 AAI_RNG-ACK

When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID, and it shall not contain the fields, Number of frame identifiers, Frame identifier, RANRNG-ACK Bitmap, Number of received codes, and Ranging preamble code index. When the AMS receives an unsolicited AAI_RNG-ACK message, it shall reset the periodic ranging timer and adjust its PHY parameters as notified in the AAI_RNG-ACK message.

[On the IEEE P802.16m/D5, page 85, line 13, make the following changes to subclause 16.2.3.3]

Table 682—AAI_RNG-ACK message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note |  | Conditions |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M | Unicast indication |  | 1 | 0b0 : for broadcast 0b1: for unicast |  | N.A. |  |
| O | Number of frame identifiers (N_Frames) |  | 3 |  | The number of frame identifiers included in this RNG-ACK message. |  | It sShall be included only when the Unicast indication is 0b0. |
| O | Frame identifier |  | 4 | Frame which contains the ranging opportunities to which this message refers. Shall be included only when the AAI_RNG-ACK is sent with broadcast STID. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2. The frame index within the superframe (ranging from 0b00 to 0b11). |  | It shall be included only when the Unicast indication is 0b0. |  |
| O | RNG-ACK Bitmap |  | 2 | The size of this bitmap is decided by the number of ranging opportunities in the referenced frame Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. |  | It shall be included only when Unicast indication = 0b0 |  |
| O | Number of received codes |  | 6 | The number of ranging preamble code indices detected in this corresponding ranging opportunity. |  | It shall be included for each bit = 0b1 in the RNG-ACK Bitmap only when Unicast indication = 0b0 |  |
| O | Ranging responses[0..32] | Ranging preamble Code Index | 6 | Ranging preamble code index received in this ranging opportunity. |  | It shall be included for the specific user each received ranging preamble code. only when Unicast indication = 0 and RNG-ACK bitmap=1 and number of received codes > 0 |  |
|  |  | Dedicated ranging preamble code indicator | 1 | Used to indicate whether the received ranging preamble code is one of RP codes for the purpose of dedicated ranging or not. 0b0 = RP code for the purpose of contention-based normal ranging 0b1 = RP code for the purpose of dedicated ranging |  | It shall be included for each received ranging preamble code. |  |
|  |  | Ranging Status | 2 | Used to indicate whether ranging premable preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue |  | It shall be included for each received ranging preamble code index. the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or it shall be included for the specific user when Unicast indication =0b1 |  |
|  |  | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |  | It shall be included for each received ranging preamble code index when Ranging status is 0b00 or 0b10. the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or it shall be included for the specific user when Unicast indication =0b1 and Ranging status is 0b00 or 0b10. In this case, Bit #2 is always set to 0. |  |
|  |  | Timing offset adjustment | 5 | Time offset adjustment Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs |  | It shall be included for each received ranging preamble code index the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and and when API bit #0 = 0b1. Or it shall be included for the specific user when Unicast indication = 0b1 and API bit #0 = 0b1 |  |
|  |  | Power level adjustment | 4 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the AMS shall apply to its current transmission power for initial ranging. The bit size represents power level correction from -8 dB to 8dB (except 0dB) which corresponds 0b0000 to 0b1111. |  | It shall be included for each received ranging preamble code index when API bit #1 = 0b1. Or it shall be included for the specific user when Unicast indication = 0b 1 and API bit #1 = 0b1 |  |
|  |  | Frequency offset adjustment | 9 | Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (Δf) multiplied by the 9-bit number interpreted as a signed integer. |  | It shall be included for each received ranging preamble code index when API bit #2 = 0b1. Or it shall be included for the specific user when Unicast indication = 0b1 and API bit #2 = 0b1 |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------