| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Change for AAI_SingleBS_MIMO_FBK (16.2.3.37) |  |
| Date Submitted | 2010-04-28 |  |
| Source(s) | Wookbong Lee LG Electronics Jisoo Park, Wooram Shin ETRI Luciano Sarperi Fujitsu | wookbong.lee@lge.com |
| Re: | Call for LB #31a on “ P802.16m/D5” Proposed text changes to P802.16m/D5 16.2.3.37 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D6 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for AAI_SingleBS_MIMO_FBK (16.2.3.37)

Wookbong Lee

LG Electronics

Jisoo Park, Wooram Shin

ETRI

Luciano Sarperi

Fujitsu

- Introduction

This contribution change the format of AAI_SingleBS_MIMO_FBK message.

- References

[1] Draft Amendment, P802.16m/D5

Remedy #1:page 165,  modify AAI_SingleBS_MIMO_FBK Message as follows;

-------------------------------------------------Text Change #1 Start -----------------------------------------------------

16.2.3.37 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table 723. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback one or multiple feedback contentsthe subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas. Variables MaxMt, Codebook_subset, Codebook_mode, Num_best_subbands, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. Best_subbands_index is encoded as described in section 16.3.9.3.1.4. The feedback information reported in the AAI_SingleBS_MIMO_FBK message depends on currently assigned feedback processes and reporting times.

Table 705 - AAI_SingleBS_MIMO_FBK message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI-SingleBS-MIMO-FBK | N.A. |
| O | matrix2x2 | 6 | 2Tx correlation matrix as defined in section 16.3.7.2.5.6 | Presents when (Nt==2) and ((MFM == 3) or (MFM == 4) or (MFM == 6) or (MFM == 7)) and (q > 0) |
| O | matrix4x4 | 28 | 4Tx correlation matrix as defined in section 16.3.7.2.5.6 | Presents when (Nt==4) and ((MFM == 3) or (MFM == 4) or (MFM == 6) or (MFM == 7)) and (q > 0) |
| O | matrix8x8 | 120 | 8Tx correlation matrix as defined in section 16.3.7.2.5.6 | Presents when (Nt==8) and ((MFM == 3) or (MFM == 4) or (MFM == 6) or (MFM == 7)) and (q > 0) |
| M | mfmBitMap | 8 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 1 LSB #2: MFM 2 LSB #3: MFM 3 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 | N.A. |
| O | Mfm0InfoSet |  | MFM 0 | Presents when LSB #0 in mfmBitMap == 0b1 |
| O | - StcRateSet | 1-3 | Wideband STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | Shall be omitted if MaxMt == 1 |
| M | - widebandCqi | 4 | Wideband CQI | N.A. |
| O | Mfm1InfoSet |  | MFM 1 | Presents when LSB #1 in mfmBitMap == 0b1 |
| M | - widebandCqi | 4 | Wideband CQI | N.A. |
| O | Mfm2InfoSet |  | MFM 2 | Presents when LSB #2 in mfmBitMap == 0b1 |
| O | - BestSubbands | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| O | - StcRateSet | 1-3 | STC rate If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt == 1. |
| M | - SubbandCqiSetChoice | variable | The subbands CQIs are sorted in order of increasing logical index. MCS of subbands indicated by BestSubbands. | N.A. |
| O | Mfm3InfoSet |  | MFM 3 | Presents when LSB #3 in mfmBitMap == 0b1 |
| O | - BestSubbands | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| O | - StcRateSet | 1-3 | STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | Shall be omitted if MaxMt == 1 |
| M | - SubbandCqiSetChoice | variable | The subbands CQIs are sorted in order of increasing logical index. MCS of subbands indicated by BestSubbands. | N.A. |
| M | - SubbandPmiSetChoice | variable | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits The subbands PMIs are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. | N.A. |
| O | Mfm4InfoSet |  | MFM 4 | Presents when LSB #4 in mfmBitMap == 0b1 |
| O | - StcRateSet | 1-3 | Wideband STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | Shall be omitted if MaxMt == 1 |
| M | - widebandCqi | 4 | Wideband CQI | N.A. |
| O | - widebandPmi | 3-6 | Wideband PMI Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits | Presents when q == 0 |
| O | Mfm5InfoSet |  | MFM 5 | Presents when LSB #5 in mfmBitMap == 0b1 |
| O | - BestSubbands | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| M | - SubbandCqiSetChoice | variable | The subbands CQIs are sorted in order of increasing logical index. MCS of subbands indicated by BestSubbands. | N.A. |
| M | - SubbandStreamSetChoice | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit The subbands Best stream indexes are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. | N.A. |
| O | Mfm6InfoSet |  | MFM 6 | Presents when LSB #6 in mfmBitMap == 0b1 |
| O | - BestSubbands | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| M | - SubbandCqiSetChoice | variable | The subbands CQIs are sorted in order of increasing logical index. MCS of subbands indicated by BestSubbands. | N.A. |
| M | - SubbandPmiSetChoice | variable | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits The subbands PMIs are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. | N.A. |
| O | Mfm7InfoSet |  | MFM 7 | Presents when LSB #7 in mfmBitMap == 0b1 |
| M | widebandCqi | 4 | Wideband CQI | N.A. |
| O | widebandPmi | 3-6 | Wideband PMI Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits | Presents when q == 0 |

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type | 8 |  |
| If ( ((MFM == 3) and (q > 0)) or ((MFM == 6) and (q > 0)) or ((MFM == 4) and (q > 0)) or ((MFM == 7) and (q > 0))){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |
| For (i=1; i <= Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.7.2.5.6 |
| For (j=i+1; j <= Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| } |  |  |
| MFM_bitmap | 4 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling IE. LSB #0: MFM 2 LSB #1: MFM 3 LSB #2: MFM 5 LSB #3: MFM 6 |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| } |  |  |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| STC_Rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt = 1. |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Stream index | variable | Best stream index of m-th subband indicated by Best_subbands_index If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_index | variable |  |
| For (m=0; m < Num_best_subbands; m ++){ |  | The subbands are sorted in order of increasing logical index |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |
| Subband PMI | variable | PMI of m-th subband indicated by Best_subbands_index Nt = 2: 3 bits Nt = 4 and Codebook_Subset = 0b0: 6 bits Nt = 4 and Codebook_Subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------------------------Text Change #1 END -----------------------------------------------------

Remedy #2:page 866,  modify AAI_SingleBS_MIMO_FBK Message as follows;

-------------------------------------------------Text Change #2 Start -----------------------------------------------------

Mfm0InfoSet ::= SEQUENCE {

wbstcRate                      StcRateSet OPTIONAL,

widebandCqi                    INTEGER (0..15)}

Mfm1InfoSet ::= SEQUENCE {

widebanCqi                     INTEGER (0..15)}

Mfm2InfoSet ::= SEQUENCE {

bestSubbandIndex                  BestSubbands OPTIONAL,

stcRate                        StcRateSet OPTIONAL,

subbandCqi                     SubbandCqiSetChoice}

Mfm3InfoSet ::= SEQUENCE {

bestSubbandIndex                  BestSubbands OPTIONAL,

stcRate                        StcRateSet OPTIONAL,

subbandCqi                     SubbandCqiSetChoice,

subbandPmi                      SubbandPmiSetChoice}

Mfm4InfoSet ::= SEQUENCE {

wbstcRate                          StcRateSet OPTIONAL,

widebandCqi                        INTEGER (0..15)

widebandPmi                        WidebandPmiSet OPTIONAL}

Mfm5InfoSet ::= SEQUENCE {

bestSubbandIndex                  BestSubbands OPTIONAL,

subbandCqi                         SubbandCqiSetChoice,

subbandStream                     SubbandStreamSetChoice}

Mfm6InfoSet ::= SEQUENCE {

bestSubbandIndex                  BestSubbands OPTIONAL,

subbandCqi                         SubabndCqiSetChoice,

subbandPmi                         SubbandPmiSetChoice}

Mfm7InfoSet ::= SEQUENCE {

widebandCqi                        INTEGER (0..15)

widebandPmi                        WidebandPmiSet OPTIONAL }

WidebandPmiSet ::= CHOICE {

nt2                                  BIT STRING (SIZE(3)),

nt4CodeBook0                       BIT STRING (SIZE(6)),

nt4CodeBook1                       BIT STRING (SIZE(4)),

nt8                                  BIT STRING (SIZE(4))}

BestSubbands ::= CHOICE {

full                                 NULL,

fiveM                                BestSubbandForFiveM,

tenM                                 BestSubbandForTenM,

twentyM                              BestSubabndForTwentyM}

-------------------------------------------------Text Change #2 END -----------------------------------------------------