| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the AAI_SCD message Update (D5 - 16.2.3.30) |  |
| Date Submitted | 2010-05-13 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution proposes AAI_SCD message update. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the SCD message Update (D5 - 16.2.3.30)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the SCD message update in the IEEE 802.16m DRAFT Amendment [1]. ABS transmits and changes system parameters in the AAI_SCD message as well as SFH (Super Frame Header). The changed S-SFH contents are applied in the super-frame which is indicated by S-SFH apply offset in P-SFH. However, there is no description of when ABS applies the system parameters associated with the current Configuration Change Count in the AAI_SCD message.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

[Remedy #1 : Adopt the following modification in page 155, line 45, section 16.2.3.30]

The updated AAI_SCD starts to be applied at the superframe when the S-SFH SP3 including changed SCD Configuration Change Count is applied. If the SCD Configuration Change Count is different from that was saved at AMS, the AMS shall wait to receive the updated AAI_SCD message.

Table 717—AAI_SCD Message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Configuration Change Count | 4 | The value is increased whenever the contents of this message is changed The value rolls over from 0 to 15 | N.A. |

[Remedy #2 : Adopt the following modification in page 534, line 55, section 16.3.6.5.1.2]

Table 839—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| fpPowerConfig | 3 | The power boosting/de-boosting values are listed in Table 840 |
| SCD Configuration Change Count | 4 | Indicates Configuration Change Count which is considered for applying the system configuration of the AAI_SCD. |