| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to MIMO extended header (16.2.2) |  |
| Date Submitted | 2010-04-28 |  |
| Source(s) | Wookbong Lee LG Electronics Jisoo Park, Wooram Shin ETRI Taeyoung Kim Samsung Electronics Huaning Niu Intel Corp | wookbong.lee@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0032, “IEEE 802.16 Working Group Working Group Letter Ballot Recirc #31a” Target topic: “IEEE P802.16m/D5 section 16.2.2”. |  |
| Abstract | Modifications are proposed for the MIMO extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to MIMO extended header (16.2.2)

Wookbong Lee

LG Electronics

Jisoo Park, Wooram Shin

ETRI

Taeyoung Kim

Samsung Electronics

Huaning Niu

Intel Corp

# Introduction

Resolution of A0692 in 80216-10_0018r5 is not completed.

We need to change MFEH and CMFEH to MFH and CMFH (extended header to signaling header.)

- References

[1] Draft Amendment, P802.16m/D5, April. 2010

# Text proposal for inclusion in the 802.16m amendment

<Remedy #1: Add two signaling header in Table 656, on the page 48 as follows >

-------------------------------  Start of Text Proposal-1 ---------------------------------------------------

Table 656—Type field encodings for MAC signaling header type

| Type field (4 5bits) | MAC Signaling Header Type |
| --- | --- |
| 00000 | BR with STID |
| 00001 | BR without STID |
| 00010 | Service Specific Scheduling Control Header |
| 00011 | Sleep Control |
| 00100 | AMS Battery Level Report |
| 00101 | Uplink Power Status Report |
| 00110 | Correlation Matrix Feedback |
| 00111 | MIMO Feedback |
| 010101000 – 11111 | Reserved |

-------------------------------  End of Text Proposal-1 ---------------------------------------------------

<Remedy #2: Delete two MIMO extended header (0b0011 Correlation matrix feedback extended header and 0b100 MIMO feedback extended header) in Table 664, on the page 56>

<Remedy #3: Modify the section 16.2.2.2.7 and 16.2.2.2.8 on line 16 the page 64 as follows >

-------------------------------  Start of Text Proposal-3 ---------------------------------------------------

16.2.2.1.3.x2.7 Correlation Matrix Feedback Extended Header (CMFEH)

This Correlation matrix feedback extended header format is defined in Table 672X. This signaling extended header may be used by AMS as a response to a Feedback Polling A-MAP IE requesting the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3.

Table 672X - CMFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CMFEH() { |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 4 5 | MAC signaling header type = 0b00110CMFEH type |
| Length | 3 | Indicates the length of the signaling header: |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.7.2.5.65.1 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.7.2.5.65.1 |
| } |  |  |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

16.2.2.1.3.y2.8 MIMO feedback extended hHeader (MFEH)

This MIMO feedback extended header format is defined in Table 673Y. This header is used by AMS to send only the wideband information for any combinations of MFM 0, 4, 7 with q=0. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one subband for MFM 2, 3, 5 or 6. Variables MaxMt, Codebook_subset, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section <<16.3.9.3.1.5>>. The MFEH shall be padded by zeros for byte alignment to 3 bytes.

The decimal value of wideband STC rate is encoded with 2 or 3 bits in binary representation, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of subband and wideband PMI is encoded with 4 or 6 bits in binary representation, but its allowed values depend on Nt and Codebook_subset. The decimal value of subband stream index is always encoded with 2 bits, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of logical subband index is always encoded with 5 bits, but its allowed values are limited to the interval [0, YSB-1].

Table 673Y –MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 4 5 | MAC signaling header type = 0b0111MFEH type |
| Length | 3 | Indicates the length of the signaling header: |
| MFM_bitmap MFM | 3 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 4 LSB #2: MFM 7 |
| If (LSB#0 in MFM_bitmap == 1 MFM == 0){ |  | MFM 0 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Zero padding | 10 |  |
| } |  |  |
| If (MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Zero padding | 13 |  |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Zero padding | 5 |  |
| } |  |  |
| If (MFM == 3 and Nt == 2 or 4){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 2 |  |
| Subband PMI | 6 |  |
| } |  |  |
| If (MFM == 3 and Nt == 8){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Subband PMI | 4 |  |
| Zero padding | 1 |  |
| } |  |  |
| If (LSB#1 in MFM_bitmap == 1 MFM == 4){ |  | MFM 4 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Wideband PMI | 6 |  |
| Zero padding | 4 |  |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband stream index | 2 |  |
| Zero padding | 6 |  |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband PMI | 6 |  |
| Zero padding | 2 |  |
| } |  |  |
| If (LSB#2 in MFM_bitmap == 1 MFM == 7){ |  | MFM 7 |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 |  |
| Zero padding | 7 |  |
| } |  |  |
| Reserved | variable | For byte alignment |
| } |  |  |

-------------------------------  End of Text Proposal-3 ---------------------------------------------------