| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Avoidance and Mitigation to Support Femtocells (16.4.11) |  |
| Date Submitted | 2010-04-26 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak LG Electronics, Inc. | +82-31-450-1879 jin1.lee@lge.com kiseon.ryu@lge.com ronnyyoungho.kim@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution is to propose text changes for 16.4.11. |  |
| Purpose | Update P802.16m/D5 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Avoidance and Mitigation to Support Femtocells (16.4.11)

Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak

LG Electronics, Inc.

- Introduction

The section of interference avoidance and interference mitigation needs to be clarified and cleaned up.

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

16.4.11 Interference avoidance and interference mitigation

Large interference from an inaccessible Femto ABS may trigger a nearby AMS to report the interference to the serving ABS, and the report information should include system information of the inaccessible Femto ABS (e.g., BS_ID of the Femto ABS).

An AMS may initiate or be requested to report the signal strength measurements of its neighbor ABSs to its serving macro ABS via AAI_SCN-REP. The reported information can be used by the serving macro ABS to coordinate interference mitigation with its neighbor ABSs. Upon receiving the interference coordination sent by the serving  macro ABS, the interfering ABS (e.g., CSG-closed Femto ABS or the Femto ABS that AMS unable to enter due to mobility) may perform interference mitigation (e.g. reducing transmission power and/or blocking resource regions and/or changing cell type to CSG-Open Femto ABS temporarily, etc.). 
When the interfering ABS notifies the serving macro ABS of converting CSG-Closed to CSG-Open Femto ABS, the serving macro ABS may initiate handover of the AMS to the CSG-Open Femto ABS by sending AAI_HO-CMD including the BS_ID of the Femto ABS .

If the AMS is connected to macro ABS and the AMS is interfered by a CSG-Closed Femto ABS where it is not a member, it may indicate this problem to the macro ABS.

---------------------------------------------------------------Text End -----------------------------------------------------------

- Reference

[1] IEEE P802.16m/D5, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” April 2010.