| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Avoidance and Mitigation to Support Femtocells (16.4.11) |  |
| Date Submitted | 2010-04-26 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak LG Electronics, Inc. | +82-31-450-1879 jin1.lee@lge.com kiseon.ryu@lge.com ronnyyoungho.kim@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution is to propose text changes for 16.4.11. |  |
| Purpose | Update P802.16m/D5 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Avoidance and Mitigation to Support Femtocells (16.4.11)

Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak

LG Electronics, Inc.

- Introduction

The section of interference avoidance and interference mitigation needs to be clarified and cleaned up.

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

16.4.11 Interference avoidance and interference mitigation

[Remedy 1 : Modify texts as following, page 778 line 21]

An AMS may initiate or be requested to report the signal strength measurements of its neighbor ABSs to its serving macro ABS via AAI_SCN-REP. The reported information can be used by the serving macro ABS to coordinate interference mitigation with its neighbor ABSs. Upon receiving the interference coordination sent by the serving macro ABS, the interfering ABS (e.g., CSG-closed Femto ABS or the Femto ABS that AMS unable to enter due to mobility) may perform interference mitigation (e.g. reducing transmission power and/or blocking resource regions and/or converting to CSG-Open Femto ABS, etc.).

If the AMS is connected to macro ABS and the AMS is interfered by a CSG-Closed Femto ABS where it is not a member, it may indicate this problem to the macro ABS.

[Remedy 2: Modify texts as following, page 778 line 48]

Upon communication with the network entity, the CSG-Closed Femto ABS may convert to a CSG-Open Femto ABS or reduce its transmit power as directed by the network entity or refrain from transmitting on certain resource regions as indicated by network entity. In case CSG-Closed Femto ABS converts to CSG-Open Femto ABS, the AMS served by macro ABS may perform handover to the CSG-Open Femto ABS.

---------------------------------------------------------------Text End ------------------------------------------------------------------------

- Reference

[1] IEEE P802.16m/D5, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” April 2010.