| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Trigger condition related to Femto IM (16.4.11) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak LG Electronics, Inc. | +82-31-450-1879 jin1.lee@lge.com kiseon.ryu@lge.com ronnyyoungho.kim@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution is to propose text changes for 16.4.11. |  |
| Purpose | Update P802.16m/D5 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Trigger Condition related to Femto IM (16.4.11)

Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak

LG Electronics, Inc.

- Introduction

The section of interference avoidance and interference mitigation needs to be clarified and cleaned up.

- Proposed Texts

[Modify texts as following, page 779 line 25]

The serving ABS may send the interfering BS_ID(s) for which the resource reservation is coordinated via AAI_SCN-RSP to the AMS and it may request the AMS to scan the femto ABS performed coordinated IM via unsolicited AAI_SCN-RSP. The scan can be periodical. AMS may scan and report the scanning result to the serving ABS via AAI_SCN-REP, if IM termination trigger certain predefined conditions defined in 16.4.11.1 are met,. The report may be sent periodically. The serving ABS may terminate IM with the interfering femto ABS via backhaul.

[Insert the section after line 42 on page 779 as follows:]

16.4.11.1 Interference Mitigation Trigger condition definitions

IM-termination trigger definitions are encoded using the description in Table y1.

Table y1—IM-termination Trigger; Type/Function/Action Description

| Name | Legnth (bits) | Value |
| --- | --- | --- |
| Type/Function/Action | 3 | See Table y2 for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |

Table y2—IM-termination Trigger; Type/Function/Action Description

| Name | Size (bits) | Value |
| --- | --- | --- |
| Type | 1 | Trigger metric type: 0x0: RSSI metric 0x1: CINR metric |
| Function | 1 | Computation defining trigger condition: 0x0: Metric of neighbor ABS is less than absolute value 0x1: Reserved |
| Action | 1 | 0x0: Respond on trigger with AAI_SCN-REP to terminate IM. 0x1: Reserved |

---------------------------------------------------------------End text change ---------------------------------------------------

- Reference

[1] IEEE P802.16m/D5, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” April 2010.