| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to scanning for assigned secondary carriers (16.2.3.52) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Kiseon Ryu LG Electronics | E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com, |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to scanning for assigned secondary carriers (16.2.3.52)

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG Electronics

- Introduction

In this contribution, we propose that the ABS indicates in the AAI_MC-RSP message whether the AMS needs to perform the scanning procedure for each carrier or not.

- Discussions

In the multi-carrier operation, the ABS should assign the secondary carriers to an AMS right after the network (re)entry. After then, the ABS can activate or deactivate the secondary carrier(s) for the efficient data communication of an AMS in multi-carrier aggregation mode.

In here, we can consider a scenario that the several assigned carriers have smaller coverage than primary carrier or active secondary carrier and the AMS is located in the outside of the smaller carrier. In this case, the ABS which doesn’t know the exact location information of the AMS may direct to activate the secondary carrier of smaller coverage and the AMS would fail the activation of the secondary carrier. Under this scenario, the channel quality information for the inactive secondary carrier(s) will be beneficial to the carrier management of ABS. However, if an assigned secondary carrier has the same coverage as the primary carrier, the AMS does not need to perform the scanning because the ABS can refer to the measurement result of the primary carrier. With these reasons, it seems to be inefficient to perform the scanning for all assigned carriers from the AMS.

So, we propose to indicate in the AAI_MC-RSP message whether the AMS needs to perform the scanning procedure for each assigned carrier or not.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Insert the texts at line 10 in Table 754 and line 44 in Table 755 on page 206 as follows:]

16.2.3.52 AAI_MC-RSP (Multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

Table 754 —AAI_MC-RSP message format

| Field | Size (bit) | Note |
| --- | --- | --- |
| Global_Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |
| If(Global_Assign == 0){ |  |  |
| Number of Assigned Carriers (N) | 3 | Number of Assigned Secondary Carriers. |
| For (i=0; i<N; i++){ |  |  |
| Physical carrier index | 6 | The index refers to a Physical carrier index in AAI_MC-ADV message. Logical carrier index is assigned implicitly in the order of assigned physical carrier index. |
| Scanning required indicator | 1 | 0: The AMS doesn’t need to perform scanning when the carrier is inactive. 1: The AMS may perform scanning without any direction from ABS when the carrier is inactive. |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0 = not support 1= support |

Table 755 —Contents of AAI_MC-RSP message

| M/O | Attributes/Array of attributes | Size (bit) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the MS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 6 | Indicates the carriers to be assigned to the MS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index. | Shall be present when the Global Assign is zero |
| O | Scanning required indicator | 1 | Indicates whether the AMS needs to perform the scanning procedure for the assigned carrier when the carrier is inactive. 0: The AMS doesn’t need to perform scanning. 1: The AMS may perform scanning without any direction from ABS. | Shall be present when the Global Assign is zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported |  |

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------