| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to scanning for assigned secondary carriers (16.2.3.52) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Kiseon Ryu LG Electronics | E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com, |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to scanning for assigned secondary carriers (16.2.3.52)

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG Electronics

- Introduction

In this contribution, we propose that the ABS indicates in the AAI_MC-RSP message whether the AMS needs to perform the scanning procedure for each carrier or not and the CM-specific trigger TLVs for the carrier are also transmitted in this AAI_MC-RSP message if the scanning required indicator for a carrier is set to 1.

- Discussions

In the multi-carrier operation, the ABS should assign the secondary carriers to an AMS right after the network (re)entry. After then, the ABS can activate or deactivate the secondary carrier(s) for the efficient data communication of an AMS in multi-carrier aggregation mode.

In here, we can consider a scenario that the several assigned carriers have smaller coverage than primary carrier or active secondary carrier and the AMS is located in the outside of the smaller carrier. In this case, the ABS which doesn’t know the exact location information of the AMS may direct to activate the secondary carrier of smaller coverage and the AMS would fail the activation of the secondary carrier. Under this scenario, the channel quality information for the inactive secondary carrier(s) will be beneficial to the carrier management of ABS. However, if an assigned secondary carrier has the same coverage as the primary carrier, the AMS does not need to perform the scanning because the ABS can refer to the measurement result of the primary carrier. With these reasons, it seems to be inefficient to perform the scanning for all assigned carriers from the AMS.

So, we propose to indicate in the AAI_MC-RSP message whether the AMS needs to perform the scanning procedure for each assigned carrier or not, and the CM-specific trigger TLVs for the carrier are also transmitted in this AAI_MC-RSP message if the scanning required indicator for a carrier is set to 1. Defining the CM-specific trigger TLVs is beneficial to reduce the signaling overheads for Type/Function/Action.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Insert the texts at line 10 in Table 754 and line 44 in Table 755 on page 206 as follows:]

16.2.3.52 AAI_MC-RSP (Multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

Table 754 —AAI_MC-RSP message format

| Field | Size (bit) | Note |
| --- | --- | --- |
| Global_Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |
| If(Global_Assign == 0){ |  |  |
| Number of Assigned Carriers (N) | 3 | Number of Assigned Secondary Carriers. |
| For (i=0; i<N; i++){ |  |  |
| Physical carrier index | 6 | The index refers to a Physical carrier index in AAI_MC-ADV message. Logical carrier index is assigned implicitly in the order of assigned physical carrier index. |
| Scanning required indicator | 1 | 0: The AMS doesn’t need to perform scanning when the carrier is inactive. 1: The AMS may perform scanning without any direction from ABS when the carrier is inactive. |
| If (scanning required indicator ==1){ |  |  |
| CM-specific Trigger TLVs | variables | CM-specific triggers with encoding defined in Table xxx – Trigger description for assigned carriers. |
| } |  |  |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0 = not support 1= support |

Table 755 —Contents of AAI_MC-RSP message

| M/O | Attributes/Array of attributes | Size (bit) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the MS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 6 | Indicates the carriers to be assigned to the MS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index. | Shall be present when the Global Assign is zero |
| O | Scanning required indicator | 1 | Indicates whether the AMS needs to perform the scanning procedure for the assigned carrier when the carrier is inactive. 0: The AMS doesn’t need to perform scanning. 1: The AMS may perform scanning without any direction from ABS. | Shall be present when the Global Assign is zero |
| O | CM-specific Trigger TLVs | variables | CM-specific triggers with encoding defined in Table xxx – Trigger description for assigned carriers. | Shall be present when the Scanning required indicator is set to 1. |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported |  |

[Modify the texts at line 12 on page 311 as follows:]

16.2.8.2.11.1 Secondary Carrier Activation/De-activation

The activation or deactivation of secondary carrier(s) is decided by the ABS based on QoS requirement, load condition of carriers, channel quality for activated carrier and other factors. The CM-specific trigger/action definition for the assigned carrier is obtained from AAI_MC-RSP message when the scanning required indicator is set to 1. The ABS activates and/or deactivates secondary carrier with the AAI_CM-CMD MAC control message. The ABS sends the AAI_CM-CMD MAC control message on the primary carrier and includes the following information:

[Insert the section at line 1 on page 312 as follows:]

16.2.8.2.11.1.1 CM-specific Trigger condition definitions

The ABS may define trigger conditions for the following actions:

1) Conditions that define when the AMS shall initiate scanning procedure for assigned carriers

2) Conditions that define when the AMS shall report scanning measurement results to the ABS. When RSSI or CINR triggers are specified, an AMS with multiple receive antennas shall use the linear sum of the RSSI or CINR values calculated over all the receive antennas.

CM-specific Trigger definitions are encoded using the description in Table xxx.

Table xxx —CM-specific Trigger Description

| Name | Legnth (bits) | Value |
| --- | --- | --- |
| Number of conditions | 2 | The number of conditions that are included in this trigger (see loop definition below). When more than one condition is included, this trigger is referred to as a complex trigger and is the logical AND combination of all the included conditions. |
| for(i=0; i<= Number of conditions; i++){ |  |  |
| Type/Function/Action | 5 | See Table yyy for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |
| Trigger averaging parameter | 8 | The averaging parameter used for averaging this trigger metric according to equation (1) for target ABS (which is defined in ABS type). If not present, the default trigger averaging parameter in AAI_SCD is used 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |
| } |  |  |

Table yyy—CM-specific Trigger; Type/Function/Action Description

| Name | Size (bits) | Value |
| --- | --- | --- |
| Type | 2 | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric 0x2: RTD metric 0x3: reserved |
| Function | 2 | Computation defining trigger condition: 0x0: Metric of primary carrier is greater than absolute value 0x1: Metric of primary carrier is less than absolute value 0x2: Metric of assigned carrier is greater than absolute value 0x3: Metric of assigned carrier is less than absolute value |
| Action | 1 | Action performed upon reaching trigger condition: 0x0: Initiate the scanning 0x1: Respond on trigger with AAI_SCN-REP |

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------