| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to secondary carrier pre-assignment during MCHO (16.2.8.2.9.2.3) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk, Kiseon Ryu LG Electronics Yuqin Chen ZTE Hyunjeong Kang Samsung Electronics Kelvin Chou MediaTek Inc. Soojung Jung ETRI | E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, kiseon.ryu@lge.com, Chen.yuqin@zte.com.cn hyunjeong.kang@samsung.com kelvin.chou@mediatek.com sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | The contribution proposes the text of multi-carrier operation to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to secondary carrier pre-assignment 
during MCHO (16.2.8.2.9.2.3)

Eunjong Lee, Youngsoo Yuk and Kiseon Ryu

LG Electronics

Yuqin Chen

ZTE

Hyunjeong Kang

Samsung

Kelvin Chou

MediaTek Inc.

Soojung Jung

ETRI

- Introduction

In this contribution, we clarify the secondary carrier pre-assignment for the case that the MC configuration of the target ABS is different from the serving ABS.

- Discussions

In the current MC operation, the serving ABS can pre-assign the secondary carrier(s) to an AMS through Carrier_Preassignment_Indication in AAI_HO-CMD message. When the secondary carrier is pre-assigned, the AMS doesn’t need to exchange its MC capability with target ABS (i.e., AAI_MC-REQ/RSP exchange) as well as the AMS can activate the secondary carrier immediately after MCHO, if the pre-assigned secondary carrier information is transmitted in AAI_HO-CMD message. For this pre-assignment, the serving ABS has to forward the MC capability of the AMS to the target ABS(s) via backbone and receive the information of the pre-assigned secondary carriers before transmitting the AAI_HO-CMD to the AMS, as shown in Figure 423 of IEEE 802.16m/D5.

However, since the MC capability of the AMS had been determined on the basis of the MC configuration supported by the serving ABS, which is indicated in the AAI_MC-ADV message, the target ABS may require updating MC capability of the AMS if the MC configuration is different from the serving ABS. However, the current MCHO is not considering the secondary carrier pre-assignment for this case.

According to the current texts, when the secondary carrier is pre-assigned, the AMS doesn’t need to perform the MC capability negotiation with target ABS (i.e., AAI_MC-REQ/RSP message exchange). In addition, once the secondary carrier(s) is assigned to the AMS, the AMS cannot request updating its assigned secondary carrier(s) during the MC operation. In order to solve these problems, we can firstly consider that the ABS always performs MCHO without the secondary carrier pre-assignment for this case. However, this method leads to a delay in the data communication over multiple carriers because the AMS is able to perform the MC operation only after MC capability negotiation. The ABS can firstly allocate a subset of secondary carriers based on the MC capability information of the AMS which is received from serving ABS although the MC configuration is different from the serving ABS. The AMS can know the difference of the MC configuration between the target and serving ABSs via the AAI_NBR-ADV message.

Therefore, we propose that the AMS is newly negotiates the MC capability with the target ABS even after HO with secondary carrier pre-assignment is done, if the AMS recognizes that the MC configuration of the target ABS is different from the serving ABS via the AAI_MC-ADV message.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Add the texts at line 53 on page 308 as follows:]

16.2.8.2.9.2.3 HO with secondary carrier pre-assignment

The serving ABS may negotiate with the target ABS for secondary carrier pre-assignment, as illustrated in Figure 423. When Carrier_Preassignment_Indication is set to 1 in the AAI_HO-CMD message, the pre-assignment information is forwarded from the Target ABS(s) via backbone to the Serving ABS, and then sent to the AMS by the serving ABS through AAI_HO-CMD message, and part of the pre-assigned secondary carriers (indicated by the Carrier Status Bitmap) may be activated right after network reentry. The target ABS may start transmitting data on such activated secondary carrier(s) if the AMS sends AAI_CM-IND message to the target ABS after network reentry. Before secondary carrier pre-assignment, the serving ABS shall forward the multicarrier capability of the AMS to the target ABS(s) via backbone. If the AMS that have received an AAI_HO-CMD with secondary carrier pre-assignment recognizes that the MC configuration of the target ABS is different from the serving ABS via the AAI_NBR-ADV message, then the AMS may newly negotiate the MC capability with the target ABS through AAI_MC-REQ/RSP messages.

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------