| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Correction on Table 823 - IDcell Partitioning (16.3.6.1) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Jinsoo Choi LG Electronics | js.choi@lge.com |
| Re: | Comments on P802.16m/D5 for LB #31a |  |
| Abstract | The contribution provides the proposed text for correction on IDcell partitioning table to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Correction on Table 823 - IDcell Partitioning (16.3.6.1)

Jinsoo Choi

LG Electronics

- Introduction

In the previous #67 session, the finer IDcell partitioning in AAI_SCD message was accepted [1]. However, there is some incorrect value in the column of the last index of IDcell. This contribution provides the proposed text to correct that with the rationale of modification.

- Modification/Rationale

The second column in the Table 823 contains the last index of IDcell, which indicates the last index of the preamble sequence for certain ABS type. Because the macro ABS cells are hard partitioned with 86 sequences per segment (range of 0 ~ 85), “0000” indication that means no IDcells in the corresponding ABS type should indicate the last index of IDcell as ‘85+256*n’ so the following ABS cells can start with sequence index of ‘86+256*n’.

  - Suggested modification: Change the last index of IDcell with “0000” indication from ‘86+256*n’ to ‘85+256*n’.

Table 823: IDCell partitioning

| Value indicated by SA_Preamble PartitionforBStype in AAI_SCD | The Last index of IDCell |
| --- | --- |
| 0000 | 86+256*n 85+256*n |
| … | … |

- References

[1] IEEE C802.16m-10/0354r2, “Proposed Text of SA-Preamble Partitioning for ABS Type in SCD (16.3.6)”

[2] IEEE C802.16m-07/0034r2, “802.16m System Requirements”

[3] IEEE C802.16m/D5, “Advanced Air Interface (Draft)”

[4] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------------  Text Start  --------------------------------------------------------------

Modify the table 823 in 16.3.6.1.2 section in page 489 as following

Table 823: IDCell partitioning

| Value indicated by SA_Preamble PartitionforBStype in AAI_SCD | The Last index of IDCell |
| --- | --- |
| 0000 | 86+256*n 85+256*n |
| 0001 | 105+256*n |
| 0010 | 115+256*n |
| 0011 | 125+256*n |
| 0100 | 135+256*n |
| 0101 | 145+256*n |
| 0110 | 155+256*n |
| 0111 | 165+256*n |
| 1000 | 175+256*n |
| 1001 | 185+256*n |
| 1010 | 195+256*n |
| 1011 | 205+256*n |
| 1100 | 215+256*n |
| 1101 | 225+256*n |
| 1110 | 235+256*n |
| 1111 | 255+256*n |

-------------------------------------------------------  Text End  ----------------------------------------------------------------