| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Clean-up in DL Control Structure Section (16.3.6) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Jinsoo Choi LG Electronics | js.choi@lge.com |
| Re: | Comments on P802.16m/D5 for LB #31a |  |
| Abstract | The contribution provides the proposed text for clean-up in DL Control Structure section to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Clean-up in DL Control Structure Section (16.3.6)

Jinsoo Choi

LG Electronics

- Introduction

In the DL Control Structure section of IEEE 802.16m/D5, there are still several descriptions that they should be corrected including some erroneous implementations. This contribution provides the proposed text for clean-up.

- Corrections

- Erroneous implementations based on #66 session resolution

  - Remedy-1: Correction of size of S-SFH scheduling bitmap based on C802.16m-10/0075r1

  - Remedy-2: Inserting some missing information fields into S-SFH SP1 IE based on C802.16m-10/0105r2,

  - Remedy-3: Modification of some description based on the #602 comment

- Editorial corrections

  - Remedy-4, 5, 6, 7, 8: Correction of editorial errors

- References

[1] IEEE C802.16m-07/0034r2, “802.16m System Requirements”

[2] IEEE C802.16m/D5, “Advanced Air Interface (Draft)”

[3] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-------------------------------------------------------  Text Start  --------------------------------------------------------------

Remedy-1: In Table 835 in page 525, modify the size (bit) of S-SFH scheduling bitmap from ‘3’ to “2”

Table 835: P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| … | … | … |
| S-SFH Scheduling information | 3 2 | Indicates which S-SFH SP IE is included in S-SFH at this superframe 0b00: S-SFH SP1 IE 0b01: S-SFH SP2 IE 0b10: S-SFH SP3 IE 0b11: no S-SFH |
| … | … | … |
| } |  |  |

Remedy-2: Modify the Table 837 in page 528 as following

Table 837: S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| DL Primary frequency partition location | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |
| Frequency partition location for UL control channels | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |
| … |  |  |

Remedy-3: Modify the sentence from the line 42 to 43 in page 514 as following

The UL data allocations corresponding to an A-MAP region can occupy resources in UL subframes according to the A-MAP relevance and HARQ timing in 16.2.14.2.2.

UL data allocations corresponding to an A-MAP region can occupy resources in any frequency partition within the AAI subframe according to A-MAP relevance and HARQ timing defined in 16.2.14.2.2.

Remedy-4: Change all ‘IDcell’ as ‘IDcell’ in 16.3.6.1.2 section

Remedy-5: In Figure 523, modify the input of channel encoding block ‘Non-user specific A-MAP’ as ‘Non-user specific A-MAP IE’

Remedy-6: To align with other naming in P802.16m/D5, modify the sentence in line 18 of page 520 as following

When the non-user specific A-MAP is in the power-boosted frequency reuse 3 partition, …

Remedy-7: In line 49 of page 522, modify ‘Figure 524’as ‘Figure 525’

Remedy-8: In line 41 of page 554, modify ‘Figure 527’as ‘Figure 528’

-------------------------------------------------------  Text End  ----------------------------------------------------------------