| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Broadcast Burst Resource Indication |  |
| Date Submitted | 2010-04-26 |  |
| Source(s) | Yanfeng Guan, Xianming Chen, Huiying Fang, Lei Zhang, Yuqin Chen ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D4” 16.3.6.5.2.1 Non-user-specific A-MAP IE & 16.3.6.5.2.4 Assignment A-MAP IE |  |
| Abstract | This contribution proposes text on Broadcast Burst Resource Indication. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Broadcast Burst Resource Indication

Yanfeng Guan, Xianming Chen, Huiying Fang, Lei Zhang, Yuqin Chen

ZTE Corporation

- Introduction

In the current 802.16m draft amendment [1], broadcast assignment A-MAP is defined and used for broadcast burst resource indication.

- Problem

When a Multicast or Unicast Assignment A-MAP, such as DL Basic Assignment A-MAP, was not received correctly by an AMS, the corresponding burst could be retransmitted by ABS through HARQ function.

However, when a Broadcast Assignment A-MAP was not received correctly by an AMS, the broadcast burst could not be retransmitted by ABS, as HARQ function could not be used for broadcast burst transmission. Even though the broadcast burst can be time-repeated, but AMS must be able to decode the Broadcast Assignment A-MAP firstly.

In addition, the burst indicated by broadcast assignment A-MAP is for all AMSs, and no robust broadcast burst transmission will result in a big effect on the whole system.

Therefore, it is necessary to make the probability of decoding broadcast assignment A-MAP correctly higher than that of decoding multicast or unicast assignment A-MAP correctly.

Certainly, we can use power-boosted method to guarantee a more robust transmission for Broadcast Assignment A-MAP. But, as MIMO OL Region maybe is located in the same Frequency Partition as Broadcast Assignment A-MAP, different power boosted (including the case of no power boosted) for Broadcast A A-MAP in different ABSs will lead to an instability of the transmission power density in MIMO OL Region. Because power boosting for Broadcast Assignment A-MAP maybe happens in most cases, we must consider it’s effect on the MIMO OL Region. So, we prefer to use a fixed or predefined transmission method for Broadcast Assignment A-MAP, to avoid effect on the OL region interference mitigation as possible.

- Proposed Method

Use Broadcast Assignment A-MAP IE to specify the information used for decoding broadcast message, and Broadcast Assignment A-MAP code rate is determined according to unicast A A-MAP code rate set implicitly as follows:

| Reuse 1 Partition: Multicast or Unicast Assignment A-MAP MCS set | Power-boosted Reuse 3 Partition: Multicast or Unicast Assignment A-MAP MCS set | Reuse 1 Partition: Broadcast Assignment A-MAP | Power-boosted Reuse 3 Partition: Broadcast Assignment A-MAP |
| --- | --- | --- | --- |
| QPSK 1/2，QPSK 1/4 | QPSK 1/2 | QPSK 1/8 | QPSK 1/4 |
| QPSK 1/2，QPSK 1/8 | QPSK 1/4 | QPSK 1/12 | QPSK 1/4 |

No matter Broadcast Assignment A-MAP is located Reuse 1 Partition or Reuse 3 Partition, only twice blind detections for one Broadcast Assignment A-MAP are enough, the complexity is same as before.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D5 Apr. 2010.

- Proposed Text

From Line 45 to 63 on Page 535, 16.3.6.5.2.1 Non-user-specific A-MAP IE

------------------------------------------------------------- Start ----------------------------------------------------------------

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode multicast or unicast assignment A-MAP IE. The number of multicast or unicast assignment A-MAPs in each assignment A-MAP group is indicated in the Non-user-specific A-MAP IE. A-MAP IE also includes HF-A-MAP Index Parameter and HFBCH Index Parameter, which are used to indicate which transmission parameter is used to calculate HF-A-MAP index and HFBCH index respectively. The non-user specific A-MAP IE is shown in Table 841. Non-user specific A-MAP IE has 12 bits in total.

If Broadcast Assignment A-MAP IE exists, it shall follow Non-user-specific A-MAP IE, immediately. The MCS for Broadcast Assignment A-MAP IE is determined according to the MCS for the multicast or unicast assignment A-MAP IE, implicitly. Table X describes the MCS Rule between Broadcast Assignment A-MAP and Multicast or Unicast Assignment A-MAP MCS.

Table X – MCS Rule between Broadcast Assignment A-MAP and Multicast or Unicast Assignment A-MAP

| Reuse 1 Partition: multicast or unicast assignment A-MAP MCS set | Power-boosted Reuse 3 Partition: multicast or unicast assignment A-MAP MCS set | Reuse 1 Partition: broadcast assignment A-MAP | Power-boosted Reuse 3 Partition: broadcast assignment A-MAP |
| --- | --- | --- | --- |
| QPSK 1/2，QPSK 1/4 | QPSK 1/2 | QPSK 1/8 | QPSK 1/4 |
| QPSK 1/2，QPSK 1/8 | QPSK 1/4 | QPSK 1/12 | QPSK 1/4 |

The AMS can identify whether Broadcast Assignment A-MAP exists or not through blind detection method.The number of multicast or unicast assignment A-MAPs in each assignment A-MAP group can be derived from the Assignment A-MAP size in the non-user-specific A-MAP IE through table lookup.

--------------------------------------------------------------- End ----------------------------------------------------------------