| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Concurrent Transmission of Data and Control Channels (16.3.9.4.6) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Zhaohua Lu，Kun Liu，Bo Sun, Dengkui Zhu, Xiangyu Liu ZTE Corporation | Voice : 86-13510279452 E-mail: lu.zhaohua@zte,com.cn sun.bo1@zte.com.cn |
| Re: | Call for LB #31b on “ P802.16m/D5” Proposed text on P802.16m/D5 16.3.9.4.6 |  |
| Abstract | Text Proposal for concurrent transmission rule |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Concurrent Transmission of Data and Control Channels (16.3.9.4.6)

Zhaohua Lu, Kun Liu, Bo Sun, Dengkui Zhu, Xiangyu Liu

ZTE Corporation

# Introduction

If an AMS has not sufficient power to transmit an assigned channel, HARQ channel has the highest transmit priority according to D5, but considering 'ACK' has much more influence on system performance than 'NAK'.

So we suggest adjust HARQ Channel priority based on different content.

The reason is that if the ABS receive nothing on HARQ channel, it usually judge sparingly that the AMS transmit ‘NAK’, so we suggest adjust HARQ Channel priority based on different content.

# References

[1] IEEE P802.16m/D5, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”.

# Text proposal

Instruction to editor;

Black text: existing D5 text

Red and strike out text: for deletion

Blue and underline text: for addition

============================== Start of Proposed Text=================================

Line 6, page 730, include the following texts;

16.3.9.4.6 Concurrent transmission of uplink control channel and data

In case of simultaneous transmission among control channels, uplink transmission power of assigned channels is determined based on the channel list in Table 942 in descending order until the total transmission power reaches maximum power limitation of AMS.

If AMS has not sufficient power to transmit an assigned channel, compared to the power level determined by equation (287), then the AMS transmit this channel with the remaining insufficient power.

Table 942-The priority of uplink transmit channels

Channel Type

HARQ feedback for ACK information or NAK information used to ensure that the MS has received DL persistent allocation A-MAP IE successfully

PFBCH/SFBCH

Synchronized Ranging

Sounding

DATA

Bandwidth Request

HARQ feedback for NAK information except which used to ensure that the MS has received DL persistent allocation A-MAP IE successfully

============================== End of Proposed Text =================================