| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sleep Cycle interruption for UL transmission (16.2.17.2.2) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Kiseon Ryu, Giwon Park, JinSam Kwak LG Electronics, Inc. | +82-31-450-2067 kiseon.ryu@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution is to clarify the sleep cycle operation for an UL transmission |  |
| Purpose | Update P802.16m/D5 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sleep Cycle interruption for UL transmission

Kiseon Ryu, Giwon Park, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

- According to D5, there is no description how an AMS can send UL data in Sleep Window. Sleep mode is optimized in order for the ABS to efficiently send DL traffic to the AMS with minimizing AMS' power consumption. Sending UL traffic should not affect the AMS’ sleep mode operation.

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

[Modify texts as suggested below: Page 373, Line 50]

16.2.17.2.2 Sleep Window operations

During the Sleep Window, the AMS is unavailable to receive any DL data and MAC control signaling from the serving ABS. The AMS may perform power-down or autonomous scan or any other autonomous operations that do not involve the reception of DL transmissions. Handling of control signaling during sleep mode is specified in 16.2.17.2.6. If an AMS has any UL data or MAC control message during sleep mode, the AMS may interrupt the Sleep Cycle operation to request bandwidth for UL transmission in Sleep Window. After sending UL bandwidth request, the AMS shall follow the Listening Window operation specified in 16.2.17.2.3 and 16.2.17.2.3.2.

The protocols and procedures relating to interruptions of normal Sleep Cycle operation are provided in 16.2.17.2.6.

---------------------------------------------------------------Text End -----------------------------------------------------------

- Reference

[1] IEEE P802.16m/D5, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” April 2010.