| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support of Emergency Alert in Sleep Mode (16.2.3) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) Shantidev Mohanty Intel Corporation | cwchen@iii.org.tw shantidev.mohanty@intel.com |
| Re: | In response to IEEE 802.16 Working Group Letter Ballot #31a: Announcement |  |
| Abstract | This contribution proposes text for emergency alert service |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support of Emergency Alert in Sleep Mode (16.2.3)

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Shantidev Mohanty
Intel Corporation

Introduction

Emergency information is transmitted to AMSs when needed. However, current emergency alert design [1] does not consider the interests of sleep AMSs. In order to facilitate emergency information reception for sleep AMSs, an emergency alert indication included in AAI_TRF-IND is proposed in this contribution. Having received the emergency alert indication, the AMS shall stay awake to obtain the emergency information.

Proposed text for the P80216m/D5:

-----------------------------------------------Start of the Text Proposal---------------------------------------------

[Make the following changes to Section 16.2.3.26 in Page 150 beginning Line 58 in P802.16m/D5.]

#### 16.2.3.26 AAI_TRF-IND

Table 713 – Parameters for AAI_TRF-IND

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | MAC control message type | N.A. |
| M | FRMT |  | 1 | This indicates type of Traffic Indication in AAI_TRF-IND message. 0: It indicates the SLPID bitmap-based traffic indication 1: It indicates the SLPID-based traffic indication | N.A. |
| M | Emergency Alert Indication |  | 1 | Used to indicate the presence of emergency information for supporting the emergency alert service 0b0: there is no emergency information 0b1: there is emergency information | N.A. |
| O | M | SLPID Group Indication Bitmap | 32 | It indicates the existence of each SLPID group N-th bit of SLPID-Group Indication Bitmap [MSB corresponds to N = 0] is allocated to SLPID Group that includes AMS with SLPID values from N*32 to N*32+31 0: There is no traffic for any of the 32 AMSs that belong to the SLPID-Group 1: There is traffic for at least one AMS in SLPID-Group. | When FRMT ==0 |
|  | M | Traffic indication bitmap | N*32 | It indicates the traffic indication for 32 AMSs in each SLPID group Each Traffic Indication bitmap comprises multiples of 32-bit long Traffic Indication units. A Traffic Indication unit for 32 SLPIDs is added to AAI_TRF-IND message whenever its SLPID Group is set to 1 32 bits of Traffic Indication Unit (starting from MSB) are allocated to AMS in the ascending order of their SLPID values: 0: Negative indication 1: Positive indication | When FRMT ==0 |
| O | M | Num_of_SLPIDs | 6 | It indicates the number of SLPID included in AAI_TRF-IND message 0~63 | When FRMT ==1 |
|  | M | SLPID | 10 | Each SLPID is used to indicate the positive traffic indication for an AMS 0~1023 | When FRMT ==1 |

-----------------------------------------------End of the Text Proposal---------------------------------------------

[1] IEEE P80216m/D5, “P802.16m/D5: Advanced Air Interface”