| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of RS_ESI message (16.6.2) |  |
| Date Submitted | 2010-5-13 |  |
| Source(s) | Chun-Yen Hsu, Yi-Ting Lin, Chiu-Wen Chen, Kanchei(Ken) Loa, Youn-Tai Lee, Tsung-Yu Tsai, Chun-Che Chien Institute for Information Industry (III) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) | hcy@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot #31a: Announcement |  |
| Abstract | This contribution proposes text for Update of SFH information during ARS operational mode |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of RS_ESI message (16.6.2)

Chun-Yen Hsu, Yi-Ting Lin, Chiu-Wen Chen, Kanchei(Ken) Loa, Youn-Tai Lee, Tsung-Yu Tsai, Chun-Che Chien
Institute for Information Industry (III)

Whai-En Chen, Shih-Yuan Cheng
National Ilan University (NIU)

Introduction

When the essential system information in SFH is changed, an ABS shall send the information through a RS_ESI management message in the 16m DL Relay Zone. If an ARS receives RS_ESI message, it shall transmit AAI_MSG-ACK message. The ARS shall apply the information from the super-frame specified by Super-Frame Number Action in the RS_ESI. This contribution proposes modifications to ensure consistency of the latest update of SFH.

Proposed text for the P80216m/D5:

-----------------------------------------------Start of the Text Proposal------------------------------------------------

[Make the following changes to Table 972 in Page 805 beginning Line 48 in P802.16m/D5]

#### 16.6.2.16.2 Update of SFH information during ARS operational mode

Table 972—RS_ERSI message format

| Name | Size (bit) | Notes |
| --- | --- | --- |
| Message type | 8 |  |
| S-SFH change count | 4 | Change count of S-SFH transmitted in this message |
| S-SFH information bitmap | 3 | If Bit #0 is 0b1, S-SFH SP 1 is included. If Bit #1 is 0b1, S-SFH SP 2 is included. If Bit #2 is 0b1, S-SFH SP 3 is included. |
| If (S-SFH information bitmap Bit #0 == 0b1) { |  |  |
| LSB of superframe number | 4 | Part of superframe number |
| S-SFH SP1 IE () |  | Includes S-SFH SP1 IE in Table 837. The size of S-SFH SP1 IE depends on FFT size. For 2048 FFT, SizeSP1 = SP1size, 2048 FFT For 1024 FFT, SizeSP1 = SP1size, 2048 FFT - 6 For 512 FFT, SizeSP1 = SP1size, 2048 FFT -12 |
| Superframe number | 12 |  |
| Number of UL ACK/NACK channels per HARQ feedback region | 2 | Channel numbers represented by the two bits (0, 1,2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |
| Number of UL ACK/NACK channels per HF-A-MAP egion | 2 | Channel numbers represented by the two bits (0, 1,2, 3) are as follows. For 5 MHz band, 4, 8, 12, 16 For 10 MHz band, 8, 16, 24, 32 For 20 MHz band, 16, 32, 48, 64 |
| Power control channel resource size indicator | 2 |  |
| Non-user specific A-MAP location | 1 | Reuse 1 or reuse 3 |
| A-A-MAP MCS selection | 1 |  |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, DL_CAS_SB0 (4 bits), DL_CAS_MB0 (6 bits), DL_CAS_SBi (3 bits) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, UL_CAS_SB0 (4 bits), UL_CAS_MB0 (6 bits), UL_CAS_SBi (3 bits) |
| Ranging channel formats for non-synchronized AMSs | 1 | Indicates the ranging channel formats number of Table 871. For FDM-based UL PUSC zone support, TBD. |
| Ranging channel allocation periodicity for non-synchronized AMSs | 2 | Indicates the periodicity of ranging channel allocation according to the Table 878. |
| Subframe offset of ranging channel for non-synchronized AMSs | 2 | Indicates the subframe offset (OSF) of ranging channel allocation related to the Table 878. The range of values is 0≦OSF≦3 |
| Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble codes according to the Table 876. For FDM-based UL PUSC zone support, TBD |
| Start code information of ranging channel for non-synchronized AMSs | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble codes (rns0). rns0(kns) = 4 x kns +1 for ranging channel format 0. rns0(kns) = 16 x kns +1 for ranging channel format 1. The range of values is 0≦kns≦15 |
| Ranging preamble code partition information for non-synchronized AMSs | 4 | Indicates the number of initial and handover ranging preamble codes (NIN and NHO) according to the Table 877. For FDM-based UL PUSC zone support, it indicates the number of initial, handover and periodic codes (N, O, and M) according to the Table xxx. |
| ABS EIRP | 7 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit =1, this cell is not allowed for network entry or re-entry |
| Super-Frame Number Action of SP 1 | 4 | LSBs of the super-frame number when the SP 1 is applied |
| } |  |  |
| If (S-SFH information bitmap Bit #1 == 0b1) { |  |  |
| S-SFH SP2 IE () |  | Includes S-SFH SP2 IE in Table 838. The size of S-SFH SP2 IE depends on FFT size. For 2048 FFT, SizeSP2 = SP2size, 2048 FFT For 1024 FFT, SizeSP2 = SP2size, 2048 FFT - 6 For 512 FFT, SizeSP2 = SP2size, 2048 FFT -12 |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table TBD |
| MAC protocol revision | 4 | version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | 12 | For 20 MHz, DL_SAC( 5 bits), DL_FPSC(3 bits), DL_FPC(4 bits) For 5 MHz, DL_SAC( 3 bits), DL_FPSC(1 bit), DL_FPC(3 bits) |
| FFR partitioning info for UL region | 12 | For 20 MHz, UL_SAC( 5 bits), UL_FPSC(3 bits), UL_FPC(4 bits) For 5 MHz, UL_SAC( 3 bits), UL_FPSC(1 bit), UL_FPC(3 bits) |
| AMS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| EIRPIR,min | 5 |  |
| Super-Frame Number Action of SP 2 | 4 | LSBs of the super-frame number when the SP 2 is applied |
| } |  |  |
| If (S-SFH information bitmap Bit #2 == 0b1) { |  |  |
| S-SFH SP3 IE () |  | Includes S-SFH SP3 IE in Table 839. The size of S-SFH SP3 IE depends on FFT size. For 2048 FFT, SizeSP3 = SP3size, 2048 FFT |
| SA-sequence soft partitioning information | 4 |  |
| UL Fast FB Size | 4 | Specifies the size of UL feedback channel per a UL subframe |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| SP scheduling periodicity information | TBD |  |
| HO Ranging backoff start | 4 | Initial backoff window size for MS performing initial ranging during HO process, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) |
| HO Ranging backoff end | 4 | Final backoff window size for MS performing initial ranging during HO process, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff end | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| UL BW REQ channel information | 3 |  |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 |
| Uplink subframe bitmap for sounding | 8 |  |
| Sounding multiplexing type (SMT) for sounding | 1 |  |
| Decimation value D/ Max Cyclic Shift Index P for sounding | 3 |  |
| Super-Frame Number Action of SP 3 | 4 | LSBs of the super-frame number when the SP 3 is applied. |
| } |  |  |

-----------------------------------------------End of the Text Proposal------------------------------------------------

[1] IEEE P80216m/D5, “P802.16m/D5: Advanced Air Interface”