| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Decryption of control connection MAC PDU (16.2.2.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics Kiseon Ryu LGE Jaesun Cha ETRI Lei Wang Interdigital Yuqin ZTE | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> ksryu@lge.com jscha@etri.re.kr leiw@billeigean.com chen.yuqin@zte.com.cn |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot Recirc #31a / Topic: MCEH Format (Section 16.2.2.2.3) |  |
| Abstract | This contribution proposes changes to facilitate Quick Decryption of control connection MAC PDU |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Decryption of control connection MAC PDU

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

Kiseon Ryu

LGE

Jaesun Cha

ETRI

Lei Wang

Interdigital

Yuqin

ZTE

# Introduction

In the IEEE P802.16m/D5 specification [1] encryption status of payload in a control connection MAC PDU is determined based on the EC bit in MCEH whereas the FlowID in MAC header (AGMH or CMH) is used to determine the encryption status of payload in a transport connection MAC PDU.  This mechanism has following issues:

- In control connection MAC PDU because of addition of Type field in MCEH, MCEH can be present anywhere in the extended header group following MAC header, so receiver has to decode MAC header and then the extended headers before it can decrypt the payload. There are several EHs, which are variable in nature. One or more EHs can be present in a MAC PDU. This delays the processing of MAC PDU payload and hence impacts performance

- The EC bit in the MCEH mandates MCEH to be present in all MAC PDUs of control connection. In case of unfragmented management message which do not require polling, we do not need other fields in MCEH. So EC bit in MCEH imposes an overhead of 2 bytes (1 byte EH Length and 1 byte MCEH) in a MAC PDU of control connection.

# Proposal

We propose to reserve an additional Flow ID 0x1 for a control connection. Transmitter sends the FlowID 0x0 in MAC header of control connection MAC PDU if the payload is not encrypted. Transmitter sends the FlowID 0x1 in MAC header of control connection MAC PDU if the payload is encrypted.

# Proposed text

[Change 1: Modify the section 16.2.2.2.3, page 58, lines 51-56, page 59, lines 1-43 as follows]

16.2.2.2.3 MAC Control extended header (MCEH)

The MAC PDU shall include MCEH when the control connection payload in a MAC PDU contains fragmented message or an unfragmented message which requires acknowledgement.  is used on control connection. When message fragments belonging to two different control messages are being sent, the transmitter shall assign different Control Connection Channel ID (CCC ID)s to the MCEH of each MAC PDU. The MCEH format is defined in Table 668.

Table 668: MCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MCEH (){ |  |  |
| Type | 4 | MCEH Type |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| If ( SN Indicator == 0) |  |  |
| { |  |  |
| Reserved | 5 | For byte alignment |
| }else |  |  |
| { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see Table 667) |
| SN | 8 | Payload sequence number |
| reserved | 2 | For byte alignment |
| } |  |  |
| } |  |  |

[Change 2: Modify the section 16.2.4.6, page 220, lines 4-19 as follows]

16.2.4.6 Encryption of MAC PDUs

When transmitting a MAC PDU on a connection that is mapped to an SA, the sender shall perform encryption and data authentication of the MAC PDU payload as specified by that SA. When receiving a MAC PDU on a connection mapped to an SA, the receiver shall perform decryption and data authentication of the MAC PDU payload, as specified by that SA.

The Advanced Generic MAC Header and extended headers shall not be encrypted.  The receiver determines whether the payload in the MAC PDU is encrypted or not from the Flow ID in the AGMH or CMH for the transport connections. In case of control connection MAC PDU, control connection payload in the MAC PDU is encrypted if the Flow ID in AGMH is set to 0x1. Control connection payload in the MAC PDU is not encrypted, if the Flow ID in AGMH is set to 0x0. The encryption information needed to decrypt a payload at the receiving station is present at the beginning and at the end of the connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload and ICV is appended at the end of the connection payload in MAC PDU as shown in Figure 394.

[Change 3: Modify the section 16.2.4.6, page 220, lines 45-55 as follows]

If multiple connection payloads are transmitted in same burst and the connections are mapped to same SA then multiple connection payload may be multiplexed before encryption and multiplexed payload is encrypted together. The receiver shall perform the decryption and data authentication on the multiplexed payload, as specified by the SA. The receiver determines whether the payloads in the MAC PDU is encrypted or not from the Flow ID in the AGMH for the transport connections. The encryption information needed to decrypt the multiplexed payload at the receiving station is present at the beginning of the first connection payload and at the end of the last connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload 1 and ICV is appended at the end of the connection payload n in MAC PDU as shown in Figure 395.

[Change 4: Modify the section 16.2.5.2.2, page 238, lines 28-32 as follows]

The fragment extended header is used only for control flows. The Flow ID EC bit in the AGMH Fragment extended header is used to indicate whether the PDU contains control message encrypted based on security level. Whether each control message is encrypted or not is decided based on the security level which the message is associated with. The indication to the receiver that the PDU is encrypted or not is indicated by the FlowID 0x0 and 0x1 in AGMH respectively.

[Change 5: Modify the section 16.2.5.3.3, page 264, lines 52-56 as follows].

The selective confidentiality protection over control messages is indicated by the Flow ID EC bit in the AGMH MCEH. Contrary to the transport flows where the established SA is applied to all data, the primary SA is selectively applied to the control messages. EC bit in the is used only for control flows to indicate whether PDU contains the control message that is encrypted based on control message type and its usage.

[Change 6: Modify the section 16.2.3, page 70, lines 1-17 as follows]

16.2.3 MAC Control messages

The peer-to-peer protocol of MAC layers in ABS and AMS communicate using the MAC control messages to perform the control plane functions. MAC control messages shall be carried in a MAC PDU to be transported in broadcast, unicast or random access connections. There is a single unicast Control connection. HARQ shall be enabled for MAC control messages sent on the unicast Control connection. Encryption may be enabled for unicast MAC control messages. MAC control messages may be fragmented, but shall not be packed. Table 676 lists the MAC control messages that shall be defined in the ASN.1 format, as shown in <<<Appendix P>>>. The indication to the receiver whether that the PDU is encrypted or not is indicated by the FlowID 0x0 and 0x1 EC=1 in AGMH respectively MCEH. Whether the encryption is applied on a MAC control message or not shall be determined by the message type and MAC procedure context, which is define in Table 676. A messages included in a PDU whose encryption status indicated by the FlowID in AGMH EC bit value does not match the combined message type and corresponding context defined in the Table 676 shall be discarded. Encrypted and non encrypted MAC control messages shall not be sent in the same PDU.

[Change 7: Modify the section 16.2.1.2.2 on page 44 and table 652 on page 45 as follows]

16.2.1.2.2 Flow Identifier (FID)

Each AMS connection is assigned a 4 bit FID that uniquely identifies the connection within the AMS. FIDs identify control connection and transport connections. DL and UL Transport FIDs are allocated from the transport FID space as defined in Table 652. An FID that has been assigned to one DL transport connection shall not be assigned to another DL transport connection belonging to the same AMS. An FID that has been assigned to one UL transport connection shall not be assigned to another UL transport connection belonging to the same AMS. An FID that has been used for a DL transport connection can be assigned to another UL transport connection belonging to the same AMS, or vice versa. Some specific FIDs may be pre-assigned. If the value is 001001 it indicates that the MPDU is signaling header. See Table 652 for the specific allocation of FIDs.

Table 652: Flow Identifiers

| Value | Description |
| --- | --- |
| 0000 | Control FID for unencrypted control connection payload in the MAC PDU(unicast control FID when PDU is allocated by unicast assignment A-MAP IE; broadcast control FID when PDU is allocated by broadcast assignment A-MAP IE) |
| 0001 | Control FID for encrypted control connection payload in the MAC PDU |
| 001001 | FID for Signaling Header |
| 00101 - 1111 | Transport FID |

[Change 8: Modify the section 16.2.2.1.3 on page 47, lines 1-8 as follows]

16.2.2.1.3 MAC Signaling Header

The signaling header shall be sent standalone or concatenated with other MAC PDUs in either DL or UL. If the AMS uses an anonymously assigned UL resource to send the signaling header, the AMS shall include the STID in the contents field of the signaling header. One FID is reserved for MAC signaling header. The value of Flow ID for MAC signaling header is 001001.

[Change 9: In the table 655 on page 47, modify the value of Flow ID field from 0001 to 0010]

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”