| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MCEH Modification (16.2.2.2.3) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot Recirc #31a / Topic: MCEH Format (Section 16.2.2.2.3) |  |
| Abstract | This contribution proposes correction to MCEH |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MCEH Modification (16.2.2.2.3)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

In the IEEE P802.16m/D5 specification [1] ‘Type’ field was introduced in the last meeting. SN size and reserved bits size needs to be corrected for byte alignment.

# Proposed text

[Change1: Modify the table 668, page 59 as follows]

Table 668: MCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MCEH (){ |  |  |
| Type | 4 | MCEH Type |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| If ( SN Indicator == 0) |  |  |
| { |  |  |
| Reserved | 51 | For byte alignment |
| }else |  |  |
| { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see Table 664) |
| SN | 86 | Payload sequence number, SN is maintained per CCC ID, the ‘SN’ value increments by one (modulo 64) sequentially. |
| reserved | 2 | For byte alignment |
| } |  |  |
| } |  |  |

[Change2: In the table 670, page 61 for MAEH format, change the size of ACK_SN field to 6 bits.]

[Change 3: In the table 722, page 164 for AAI_MSG_ACK format, change the size of SN field to 6 bits.]

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”