| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of UL report on Battery Level via Signaling Header(16.2.2.1.3.5) |  |
| Date Submitted | 2010-05-xx |  |
| Source(s) | Youngsoo Yuk, Kiseon Ryu and Jin-Sam Kwak | youngsoo.yuk@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution propose text related on the active mode power management |  |
| Purpose | For TGm discussion and adoption into P802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of UL report on Battery Level via Signaling Header (16.2.2.1.3.5)

Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak

LG Electronics

- Introduction

The current battery level indication has ambiguity because only the ratio is informed without total capacity. We propose some examples to support more clear signaling method. The size of proposed Header is optimized to 2bytes.

--------------------------------------------- Start of the Proposed Text ------------------------------------------------------

- [Remedy 1: modify the text on page #53, Line #13 in P802.16m/D5 by the following text]

- 16.2.2.1.3.5 AMS Battery Level Report Header

The AMS Battery Level Report Header should be used to convey AMS battery level information from AMS to ABS. The specific information provided by AMS Battery Level Report Header is given in Table 661.

Table 661—AMS Battery Report header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AMS Battery Level Report header () { | - | - |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0100 |
| AMS Battery Status | 1 | 0b0: The AMS is plugged into a power source 0b1: The AMS is not plugged into a power source |
| Battery Level Indication | 1 | 0b0: The AMS is able to report detailed battery level. 0b1: The AMS is not able to report detailed battery level. |
| If (Battery Level Indication = 1) { |  |  |
| AMS’s Expected Battery Life | 3 | Expected Battery Life in unit of 10 minutes 0b000: less than 10 min. 0b001: less than 20 min. …. 0b111: The AMS is plugged into a power source |
| AMS’s Power Management Mode | 2 | The AMS’s Power Management Mode 0b00: Maximum Performance mode 0b01: Normal Mode 0b10: Maximum Power Saving Mode 0b11:reserved |
| AMS Battery Level | 3 | The field appears only when the Battery Level Indication is set to 1. 0b001: Battery level is between 75% and 100 % 0b010: Battery level is between 50% and 75 % 0b011: Battery level is between 25% and 50 % 0b100: Battery level is between 5 % and 25 % 0b101: Battery level is below 5 % 0b110 – 0b111: Reserved |
| Reserved | 35 | Shall be filled by 0 |
| } |  |  |
| else { |  |  |
| Reserved | 38 |  |
| } |  |  |
| } |  |  |

[Remedy 2: modify the text on page #408, Line #26 in P802.16m/D5 by the following text]

- 16.2.22 Power Management for the Active Mode

Enhanced power savings for an AMS in active mode may be supported. In this mode, the ABS may allocate resources, adjust operation modes, and set transmission parameters to optimize energy savings at the AMS.

An AMS may report its battery level and power management mode when the battery level changes. The AMS shall cancel the previous battery report as soon as its battery level has returned to a certain threshold, or as soon as the AMS is plugged in a charger.

Power update mechanism as specified in section 8.4.10.3 may be used when an ABS receives an AMS’s battery level report and the ABS supports power management in Active Mode.

------------------------------------------  End of the Proposed Text  ----------------------------------------------------