| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for AAI_MC-REQ/RSP (16.2.3) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Youngsoo Yuk, Eunjong Lee, Kiseon Ryu, Jin Sam Kwak LG Electronics | E-mail: youngsoo.yuk@lge.com |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | This contribution proposes the texts related to AAI_MC-REQ/RSP. |  |
| Purpose | To be adopted by TGm for the P802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for AAI_MC-REQ/RSP (16.2.3)

Youngsoo Yuk, Eunjong Lee, Kiseon Ryu, and Jin Sam Kwak

LG Electronics

- Introduction

In multi-carrier system some AMSs may support all multiple carriers supported by the ABS where the AMSs entered, especially when the number of multiple carriers supported in the ABS is small (e.g., 2~4). In this case we need to reduce the unnecessary overhead of MC-REQ MAC message by inclusion of all physical carrier indices.

- A.	Support of data reception and transmission with single RF over multiple carriers

The MC capability of the AMS to support communicating with multiple carriers with one RF should be differently considered in DL and UL differently.

In case of DL, after filtering of the received signal, FFT with wider bandwidth is performed. The part is not eliminated after filtering can be removed after FFT. There is no technical problem exists in DL.

However, in UL case, the requirement is stricter than that of DL. Figure 1 shows the problem of the aggregation. The spectral mask shape is defined for minimizing the out of band emission, and basically the ratio of the effective bandwidth is the same regardless of the bandwidth. (0.95 in WiMAX). The ratio is related to the hardware complexity or performance of RF devices. Thus, it can be a capability of UL multicarrier aggregation.

With RF having 0.95 of spectral mask, 4 normal PRUs located the edge cannot be used in case of 20MHz (10+10 MHz). To support all normal PRUs, 0.975 of ratio is required but it can be a hardware complexity.

In case of 4 carriers are aggregated, more sharpen TX filter should be equipped. But, for DL, only to support 4096 size of FFT is enough.

Figure 1. The example of the problem on Spectral Mask for Aggregated multicarrier

The spectral mask of the aggregated bandwidth should be different from that of the non-aggregated bandwidth. The Figure 5 shows the difference between two cases. We can define a parameter, “TX spectral Mask shape” as the ration between the effective BW and overall BW. In the legacy system, it was 0.95 regardless of size of bandwidth. However, as shown in Figure 2, to receive all PRUs (even without guard resource) in 20MHz which is composed of two 10MHz carriers, it should be 0.975. With the transmitter achieving spectral requirement of single carrier, 4 PRUs (2 PRUs in each edge) cannot be used. It is related to the number of taps of the transmitter filter in an AMS. Thus, the ABS should consider this capability of an AMS when it decides to grant any PRU in the edge area to the AMS.

Thus the capability of UL support should be informed to ABS during multicarrier capability negotiation.

Possible solutions without sending this capability are

- Not to allow the UL aggregation.

- Send the parameter that maximum number of carriers having same BW the AMS can transmit.

- Send the parameter on the UL spectral mask shape information.

In practical case, asymmetric number of carriers can be activated for DL and UL based on the UL capability.

- Text Proposal

-----------------------------------------------------  Start of the Proposed Text --------------------------------------

[Modify the text on page #204, from Line #1 in P802.16m/D5 by the following text]

- 16.2.3.51 AAI_MC-REQ (multicarrier Request) Message

The Multi-Carrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_Global-Config message and AAI_MC-ADV message from its serving ABS. According to the multi-carrier configuration supported by serving ABS, which is indicated in AAI_MC-ADV, the AMS shall determine the subset of carriers which it can simultaneously support under its hardware capability. Then AMS will send AAI_MC-REQ message to the serving ABS and include sets of physical carrier index to inform ABS this information.

Table 752. AAI_MC-REQ message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Global Support | 1 | Indicates whether the AMS can process all carriers supported in ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in ABS 0b1: The AMS can process all carriers supported in ABS |
| If (Global Support ==0 ) { |  |  |
| Number of Candidate Combinations (N) | 4 |  |
| For (i=0; i<N; i++) { |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  |
| For (j=0; j<Nc; j++) { |  |  |
| Physical carrier Index | 6 | The carrier AMS can simultaneously support |
| } |  |  |
| DL/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b01: The combination can be applied to DL only. |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1 = support |

Table 753—Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the AMS can process all carriers supported in ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in ABS 0b1: The AMS can process all carriers supported in ABS |  |
| O | Number of candidate combinations | 4 | Indicates the number of carrier combination MS can support | Shall be present when Global Support is zero |
| O | Number of candidate assigned carrier | 3 | Indicates the number of carriers MS can support | Shall be present when the number of candidate combinations is not zero |
| O | Physical carrier index | 6 | Indicates the carriers MS can simultaneously support | Shall be present when the number of candidate assigned carrier is not zero |
| O | DU/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b01: The combination can be applied to DL only. | Shall be present when the number of candidate combinations is not zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |

16.2.3.52 AAI_MC-RSP (multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message.

Table 754: AAI_MC-RSP message format

| Field | Size (bits) | Notes |
| --- | --- | --- |
| Global Support | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |
| If (Global Support ==0 ) { |  |  |
| Number of Assigned Carriers (N) | 3 |  |
| For (i=0; i<N; i++) { |  |  |
| Physical carrier Index | 6 |  |
| DL/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b01: The carrier is assigned for DL only. |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1 = support |
| } |  |  |
| } |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1 = support |

Table 755—Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the MS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 6 | Indicates the carriers to be assigned to the MS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be present when the Global Assign is zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A |
| O | DU/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b01: The carrier is assigned for DL only. | Shall be present when the Global Assign is zero |

The AAI_MC-RSP (multicarrier Response) Message is typically sent to the AMS in response to the AAI_MC-REQ message, but it may also be sent by the ABS to an AMS to update the list of assigned carriers in unsolicited manner.

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------