| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of AAI_HO-CMD message for Dynamic Ranging for IEEE802.16m (sections 16.2.3.11) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7188 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for comments for Letter Ballot #31a / Topic: HO |  |
| Abstract | Text proposal for Handover |  |
| Purpose | For member’s review and adoption into P802.16m_D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of AAI_HO-CMD message for Dynamic Ranging for IEEE802.16m (sections 16.2.3.11)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to amend (i.e. missing parameter) the AAI_HO-CMD to support correct dynamic ranging.

- Rationale/Motivation of modifications

The Resource Index for RNG channel needs to be informed in the HO-CMD for support of Dynamic ranging during HO at target ABS when dynamic ranging is coordinated, since the dynamic ranging channel may be allocated in non-static manner.

Also the subframe index indicating the location of the allocated dynamic ranging channel was not left out during some editorial error in D5. Hence, add the subframe index.

- References

[1] IEEE P802.16m/D5, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D5, page 105, line 1, add proposed text in table 692 of section 16.2.3.11 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.11 AAI_HO-CMD

Table 692—AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Resource index | 7 | In WirelessMAN-OFDMA frame with FDM-based UL PUSC Zone, it means DRU index. Others, it means subband index. | Shall be included when dynamic ranging is available at Target ABS |
| O | Subframe index | 3 | Indicates the subframe index of the allocated ranging opportunity. Shall be included when Dedicated Opportunity Code Flag=1 | Shall be included when dynamic ranging is available at Target ABS |

--------------------------------------------------------Text End -----------------------------------------------------