| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jeongki Kim, Wookbong Lee, Youngsoo Yuk LG Electronics | E-mail: jeongki.kim@lge.com, wookbong.lee@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D5 for IEEE 802.16 WG Letter Ballot Recirc #31a |  |
| Abstract | This contribution proposes the texts related to GRA MIMO Mode. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9)

Jeongki Kim, Wookbong Lee, Youngsoo Yuk

LG Electronics

- Introduction

According to 16m D5 [1], if UL MIMO Mode set ID in group resource allocation is 0b10, Mode 2 (CL SU-MIMO with Mt=1, TNS=1) is used as the MIMO mode of the group. However, UL CL SU-MIMO is inefficient for UL group resource allocation because the downlink control overhead will be increased when using PMI indication method or because the pre-coding by mid-amble can be used only for TDD (not FDD). We propose the texts to use OL MU-MIMO with Mt=1, TNS=2 instead of CL SU-MIMO.

- An AMS can know the resource allocation size of each AMS belonging to the AMS’s group by Resource allocation bitmap (RAB) in Group resource allocation A-MAP IE.

- Allocation of group resource and stream index in Mode 3

  - An ABS allocates the first N (Indicated by RAB) LRUs from resource offset to the first scheduled AMS and the second scheduled AMS in a group and the stream index of each AMS is 1 and 2.

  - The stream index 1 and the stream index 2 are allocated to the scheduled AMSs in order of scheduling.

  - If the stream index of N-th AMS is 1, the stream index of (N+1)-th AMS is 2 and vice versa.

  - The resource of N-th AMS is allocated right after the resource of the (N-2) th AMS.

The Table 1 shows an example of group resource allocation when the UL MIMO Mode is Mode 3 and Resource offset is 2(LRU index).

Table 1

| Order of scheduled AMSs | Size of resource allocation | Allocated LRU indices | Stream Index |
| --- | --- | --- | --- |
| AMS1 | 4 | 2 ~5 | 1 |
| AMS2 | 3 | 2~4 | 2 |
| AMS3 | 3 | 6~8 | 1 |
| AMS4 | 4 | 5~8 | 2 |
| AMS5 | 1 | 9 | 1 |
| … | … | … | … |

Figure 1

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: On Page 315, modify table 776 as follows:]

Table 776—UL MIMO mode set candidates

| MIMO Mode set ID | UL Group MIMO mode set | SM Restriction |
| --- | --- | --- |
| 0b00 | Mode 0 | N/A |
| 0b01 | Mode 0, Mode 1 | Mt=2 |
| 0b10 | Mode 2, Mode 3 | Mt=1 |

[Remedy #2: On Page 319, modify table 779 as follows:]

Table 779—MIMO Bitmap Information for UL

| MIMO Mode Set | Existence of Second Bitmap | Length of Bit per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | No | N/A | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | N/A | CL SUOL MU-MIMO with Mt=1, TNS=12 |

[Remedy #3: Modify the section 16.2.9.4 Normal Operation as follows:]

16.2.9.4 Normal Operation

An ABS shall provide contiguous allocations to the AMSs belonging to a group, which have the corresponding bit in the user bitmap set to '1'. The order of resource allocation shall be the same as the order in which they appear in the group's user bitmap. If there is no transmission for a given AMS in a certain AAI subframe, then the ABS shall set the corresponding bit in the user bitmap to '0'.

When an AMS receives a Group Resource Allocation IE in which the corresponding bit in user bitmap is set to '1', then the AMS shall decode the remaining bitmaps to determine other attributes of the allocated resource. An AMS shall determine the location of its allocation by counting the resource allocation sizes of other AMSs appearing before it in the user bitmap. If an AMS does not receive the Group Resource Allocation IE in any of the AAI subframes of a frame in which the IE was expected, then the AMS shall assume no allocations for the group in that frame.

In case that UL MIMO mode set ID is 0b10 (Mode3), the resource of N-the scheduled AMS is allocated by the following rules. Among AMSs set to 1 in user bitmap the stream index of the first AMS is 1 and the stream index of the second AMS is 2. The resource allocation of the first AMS and the second AMS begins from Resource offset in Group resource allocation A-MAP IE. The stream index 1 and the stream index 2 are allocated to the scheduled AMSs in order of scheduling. If the stream index of N-th AMS is 1, then the stream index of (N+1)-th AMS is 2 and vice versa. The resource of N-th AMS is allocated right after the resource of the (N-2) th AMS.

The ACID corresponding to a resource allocated to an MS in a given frame is not explicitly signaled in the Group Resource Allocation A-MAP IE. The ACIDs for an AMS implicitly cycle in the ACID range defined in section 16.2.9.3.1.2. The ACID for an allocation shall be determined using the following formula.

ACID = Mod(Initial_ACID + Mod( floor( Frame_Number / Allocation_Period), N_ACID), 16)(3)

where,

ACID = ACID used for current allocation

Initial_ACID = Initial ACID parameter signaled in the Group Configuration MAC Control Message

Periodicity = Periodicity of group resource allocation in terms of number of frames signaled in the Group Configuration MAC Control Message

N_ACIDs = Number of ACIDs assigned to the AMS for group resource allocation, signaled in the Group Configuration MAC Control Message

Frame_Number = Current frame number, which is given by Equation (4) below

The Frame_Number parameter is determined as

Frame_Number = Superframe_Number * 4 + Frame_Offset(4)

Where Superframe_Number is the current superframe number and Frame_Offset is the offset of the current frame with respect to the start of the corresponding superframe and . The Frame_Number parameter corresponds to the frame in which the Group Resource Allocation A-MAP IE is transmitted.

If the calculated ACID is still performing retransmissions of a previous packet, then the previous packet shall be dropped and the ACID shall be freed for the new allocation. The packet drop shall happen in the frame in which Group Resource Allocation A-MAP IE is received.

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------