| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jeongki Kim, Wookbong Lee, Youngsoo Yuk LG Electronics Rath Vannithamby, Shweta Shrivastava, Roshni Srinivasan rath.vannithamby@intel.com, Shweta.shrivastava@intel.com, Roshni.srinivasan@intel.com Intel Corporation | E-mail: jeongki.kim@lge.com, wookbong.lee@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D5 for IEEE 802.16 WG Letter Ballot Recirc #31a |  |
| Abstract | This contribution proposes the texts related to GRA MIMO Mode. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of UL MIMO Mode Set in Group resource allocation (16.2.9)

Jeongki Kim, Wookbong Lee, Youngsoo Yuk

LG Electronics

Rath Vannithamby, Shweta Shrivastava, Roshni Srinivasan

Intel Corporation

- Introduction

We propose the text to allow the use of OL MU-MIMO with Mt=1, TNS=2 with Group Resource Allocation.

- An AMS can know the resource allocation size of each AMS belonging to the AMS’s group by Resource allocation bitmap (RAB) in Group resource allocation A-MAP IE.

- Allocation of group resource and stream index in Mode 3

  - An ABS allocates PSI=1 or PSI=2 to each scheduled flow in the group. The ABS should assign the indices carefully such that the difference between the total resources of PSI=1 flows and the total resources of PSI=2 flows is minimum.

  - The ABS signals this assigned PSI for each scheduled user via the PSI bitmap included in the GRA A-MAP IE.

  - All PSI=1 flows are allocated contiguous resources (in order of their indices) starting from resource offset.

  - All PSI=2 flows are allocated contiguous resources (in order of their indices) starting from resource offset.

The Table 1 shows an example of group resource allocation when the UL MIMO Mode is Mode 3 and Resource offset is 2(LRU index).

Table 1

| Order of scheduled AMSs | Size of resource allocation | Allocated LRU indices | Stream Index |
| --- | --- | --- | --- |
| AMS1 | 4 | 2 ~5 | 1 |
| AMS2 | 3 | 2~4 | 2 |
| AMS3 | 3 | 6~8 | 1 |
| AMS4 | 4 | 5~8 | 2 |
| AMS5 | 1 | 9 | 1 |
| … | … | … | … |

Figure 1

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

---------------------------------------------------------Start of the Proposed Text ---------------------------------------------------------

[Remedy #1: On Page 315, modify table 776 as follows:]

Table 776—UL MIMO mode set candidates

| MIMO Mode set ID | UL Group MIMO mode set | SM Restriction |
| --- | --- | --- |
| 0b00 | Mode 0 | N/A |
| 0b01 | Mode 0, Mode 1 | Mt=2 |
| 0b10 | Mode 2 | Mt=1 |
| 0b11 | Mode 3 | Mt=1 |

[Remedy #2: On Page 319, modify table 779 as follows:]

Table 779—MIMO Bitmap Information for UL

| MIMO Mode Set | Existence of Second Bitmap | Length of Bit per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | No | N/A | OL SU-MIMO (SFBC with non-adaptive precoder) |
| 0b01 | Yes | 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b1: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 |
| 0b10 | No | N/A | CL SU-MIMO with Mt=1, TNS=1 |
| 0b11 | Yes | 1 | OL MU-MIMO with Mt=1, TNS=2 0b0: PSI=1 0b1: PSI=2 |

[Remedy #3: Modify the section 16.2.9.4 Normal Operation as follows:]

16.2.9.4 Normal Operation

An ABS shall provide contiguous allocations to the AMSs belonging to a group, which have the corresponding bit in the user bitmap set to '1'. The order of resource allocation shall be the same as the order in which they appear in the group's user bitmap. If there is no transmission for a given AMS in a certain AAI subframe, then the ABS shall set the corresponding bit in the user bitmap to '0'.

When an AMS receives a Group Resource Allocation IE in which the corresponding bit in user bitmap is set to '1', then the AMS shall decode the remaining bitmaps to determine other attributes of the allocated resource. An AMS shall determine the location of its allocation by counting the resource allocation sizes of other AMSs appearing before it in the user bitmap. If an AMS does not receive the Group Resource Allocation IE in any of the AAI subframes of a frame in which the IE was expected, then the AMS shall assume no allocations for the group in that frame.

The ACID corresponding to a resource allocated to an MS in a given frame is not explicitly signaled in the Group Resource Allocation A-MAP IE. The ACIDs for an AMS implicitly cycle in the ACID range defined in section 16.2.9.3.1.2. The ACID for an allocation shall be determined using the following formula.

ACID = Mod(Initial_ACID + Mod( floor( Frame_Number / Allocation_Period), N_ACID), 16)(3)

where,

ACID = ACID used for current allocation

Initial_ACID = Initial ACID parameter signaled in the Group Configuration MAC Control Message

Periodicity = Periodicity of group resource allocation in terms of number of frames signaled in the Group Configuration MAC Control Message

N_ACIDs = Number of ACIDs assigned to the AMS for group resource allocation, signaled in the Group Configuration MAC Control Message

Frame_Number = Current frame number, which is given by Equation (4) below

The Frame_Number parameter is determined as

Frame_Number = Superframe_Number * 4 + Frame_Offset(4)

Where Superframe_Number is the current superframe number and Frame_Offset is the offset of the current frame with respect to the start of the corresponding superframe and . The Frame_Number parameter corresponds to the frame in which the Group Resource Allocation A-MAP IE is transmitted.

If the calculated ACID is still performing retransmissions of a previous packet, then the previous packet shall be dropped and the ACID shall be freed for the new allocation. The packet drop shall happen in the frame in which Group Resource Allocation A-MAP IE is received.

16.2.9.4.1 Bitmaps in Group Resource Allocation

:

In addition to the user bitmap, a second bitmap called the MIMO Bitmap is used to indicate the assigned MIMO mode, when multiple MIMO modes and SM parameters are supported in the group. The MIMO Bitmap is only required for certain MIMO mode sets and may not always be transmitted. The existence of MIMO bitmap and the number of bits per scheduled flow in the MIMO Bitmap are listed in Table 778 and Table 779.

In case that UL MIMO mode set ID is 0b11 (Mode3), the resource of N-the scheduled flow is allocated by the following rules. Among flows set to 1 in user bitmap the BS assigns stream index 1 and 2 to the different flows. The assigned stream index is signaled in the PSI bitmap. All PSI=1 flows are assigned contiguous resources in increasing order of their indices starting from resource offset for the group signaled in the Group Resource Allocation A-MAP IE. All PSI=2 flows are also assigned contiguous resources in increasing order of their indices starting from resource offset.

The third bitmap is the Resource Allocation bitmap which uses 2 bits per flow to signal HARQ burst size and 3 bits per flow to signal the Resource Size for the scheduled flow in the AAI subframe or extended AAI subframe that are scheduled in the frame. The resource size refers to the number of LRUs allocated to the flow. The resource size supported in GRA is limited to 16 LRUs. Each group supports eight resource sizes for each burst size supported in the group. The set of resource sizes for each burst size belong to the range [1,16] LRUs. The set of HARQ burst sizes and resource sizes supported in the group is signaled in the Group Configuration MAC control message. For groups corresponding to all MIMO mode sets except UL(0b11), AMS i calculates the starting location of resources of its flow with index i in the AAI subframe as follows.

:

Figure 428–Example of Bitmaps for Group MIMO Mode Set: UL(0b11)

-----------------------------------------------------------End of the Proposed Text   --------------------------------------------------------