| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for updating E-MBS configuration parameter in AAI_SCD message (16.3.6.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, kiseon.ryu@lge.com, youngsoo.yuk@lge.com |
| Re: | Comments on IEEE P802.16m/D5 for IEEE 802.16 WG Letter Ballot Recirc #31a |  |
| Abstract | This contribution proposes the texts related to SCD message update for E-MBS |  |
| Purpose | For E-MBS DG discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for updating E-MBS configuration parameter in AAI_SCD message (16.3.6.2.3)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, and Jin Sam Kwak

- LG Electronics

# Introduction

As described in the Section 16.2.3.30 in 16m Draft [1], an AAI_SCD (System Configuration descriptor) message carries E-MBS configuration parameters (e.g., Zone allocation Bit-MAP, ZF, MSI length, E-MBS AAI frame offset) which a information to determine E-MBS resource region and the AAI_SCD message will be transmitted with very long period compared to MSI length or the period of AAI-E-MBS-CFG.

An Idle mode AMS which subscribes an E-MBS awakes at the superframe which E-MBS MAP is transmitted, receives the E-MBS MAP, checks whether E-MBS bursts for the AMS are transmitted in a MSI, and receives the E-MBS bursts based on the E-MBS connections allocated to the AMS. However, if the Idle mode AMS does not receives the SCD message including the changed E-MBS configuration parameters, the Idle mode AMS cannot decode the E-MBS MAP and the E-MBS bursts until the AMS receives the changed SCD message as shown in Figure 1.

Figure 1

# Proposal

When the E-MBS configuration parameters of SCD message are changed, an E-MBS MAP will contain SCD update indicator set to 1 and the changed E-MBS configuration parameters as shown in Table 1.

Table 1: E-MBS MAP Structure

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| … | … | … |
| SCD update indicator | 1 | Indicates that the E-MBS configuration parameters will be changed in next SCD. If this field is set to 1, the E-MBS configuration parameters which will be included in the next SCD message are included in this E-MBS MAP. |
| If (SCD update indicator ==1) { |  |  |
| Zone_Allocatin Bit-MAP | DSACMAX-1 | See Table 717 AAI_SCD message |
| ZF | 1 | See Table 717 AAI_SCD message |
| MSI Length | 2 | See Table 717 AAI_SCD message |
| E-MBS AAI frame offset | variable | See Table 717 AAI_SCD message |
| Adjustment time offset | 2 | Indicates the time offset at which the E-MBS configuration parameters included in this MAP are adjusted. 0b00: After 1 MSI 0b01: After 2 MSIs 0b10: After 3 MSIs 0b11: After 4 MSIs |
| } |  |  |
| ... | ... | ... |
| Padding | variable | Padding to reach byte boundary |

Figure 2

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Text #1 ---------------------------------------------------------------------------

[Modify Section 16.3.6.2.3 as follows]

16.3.6.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries configuration information for enhanced multicast broadcast service for one E-MBS Zone. The E-MBS MAP is transmitted in the first several RUs of the E-MBS region in the beginning of the MSI. The parameters of the E-MBS region and the burst size of the E-MBS MAP is transmitted in the AAI_E-MBS-CFG MAC control message described in 16.9.3.1. AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance.

Table 834-- E-MBS MAP Structure

| Syntax | Size (bits) | Description/Notes |
| --- | --- | --- |
| E-MBS-MAP () { | - | - |
| AAI_E-MBS-CFG Change Indication | 1 | AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance. If it is set (AAI_E-MBS-CFG Change Indication = 1), parameters in the AAI_E-MBS-CFG message will change and AMS shall decode the next AAI_E-MBS-CFG message. Otherwise, the AAI_E-MBS-CFG message does not change and the AMS resets the Allocation Lifetime as defined in the previous AAI_E-MBS-CFG message and may not decode the next transmission. |
| S-SFH SP2 update indicator | 1 | Indicates whether the changed S-SFH SP2 will be transmitted in this MSI. If this field is set to 1, the time offset which the changed S-SFH SP2 is transmitted will be included in this E-MBS MAP. |
| If (S-SFH SP2 update indicator ==1) { |  |  |
| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset at which the changed S-SFH SP2 is transmitted. The size of this field depends on MSI MSI == 0b00: 1 bits MSI == 0b01: 2 bits MSI == 0b10: 3 bits MSI == 0b11: 4 bits |
| } |  |  |
| SCD update indicator | 1 | Indicates that the E-MBS configuration parameters will be changed in next SCD message transmission. If this field is set to 1, the E-MBS configuration parameters which will be included in the next SCD message and the adjustment time offset are included in this E-MBS MAP. |
| If (SCD update indicator ==1) { |  |  |
| Zone_Allocation Bit-MAP | DSACMAX-1 | See Table 717 AAI_SCD message |
| ZF | 1 | See Table 717 AAI_SCD message |
| MSI Length | 2 | See Table 717 AAI_SCD message |
| E-MBS AAI frame offset | variable | See Table 717 AAI_SCD message |
| Adjustment time offset | 2 | Indicates the time offset at which the E-MBS configuration parameters included in this MAP are adjusted. 0b00: After 1 MSI 0b01: After 2 MSIs 0b10: After 3 MSIs 0b11: After 4 MSIs |
| } |  |  |
| E-MBS_DATA_IE() | - | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

--------------Proposed Text #1 ---------------------------------------------------------------------------