| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Cleanup Text for DL FFR related sections for the IEEE 802.16m/D5 (16.2.3.18 and 16.2.3.19) |  |
| Date Submitted | 2010-04-29 |  |
| Source(s) | Ariel Doubchak, David Zelikovsky Alvarion | ariel.doubchak@alvarion.com david.zelikovsky@alvarion.com |
| Re: | Call for LB #31A on “ P802.16m/D5” Proposed text changes to P802.16m/D5 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D5 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarifications for FFR related messages for the IEEE 802.16m/D5 (16.2.3.18 and 16.2.3.19)

Ariel Doubchak, David Zelikovski

Alvarion

- Introduction

This contribution proposes some clarifications for the AAI_FFR-CMD/AAI_FFR-REP messages:

- Frequency Partition bitmap clarification

- FFR report clarification

- Interference definition clarification (measurement is not necessary inter-cell)

- Reference to SINR and interference measurement definition (use definition in 16.3.14)

Remedy #1:page 131,  modify AAI_FFR-CMD (FFR Command) Message as follows;

-------------------------------------------------Text Change #1 Start -----------------------------------------------------

16.2.3.18 AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message may be transmitted by the ABS to instruct the AMS to perform measurement over specific frequency partition(s). Table XXX lists the parameters to be included into the AAI_FFR-CMD message:

Table XXX—AAI_FFR-CMD message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI-FFR-CMD | NA |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value “1”. The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. Bitmap referring to the frequency partitions [FP0, FP1, FP2, FP3], where the LSB refers to FP0 and the MSB refers to FP3. Each bit indicates if a report on the frequency partition is required where a value of 1 means a report is required. If the frequency partition size is zero, the relevant bit in frequencyPartitionBitMap should be ignored. Frequency Partition Bitmap shall contain at least one bit with value “1”. | NA |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value ‘1’, it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value ‘1’. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| M | frameOffset | 8 | The offset (in units of frames) from the current frame in which AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |  |

-------------------------------------------------Text Change #1 END -----------------------------------------------------

Remedy #2:page 132,  modify AAI_FFR-REP (FFR Report) Message as follows;

-------------------------------------------------Text Change #2 Start -----------------------------------------------------

16.2.3.19 AAI_FFR-REP (FFR Report) Message

Once an AMS receives AAI_FFR-CMD message, an AAI_FFR-REP message is sent by the AMS to report the downlink interference and/or SINR statistics of frequency partition(s) specified in AAI_FFR-CMD message.

Table YYY shows the parameters for AAI_FFR-REP message:

Table YYY—AAI_FFR-REP message format

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Control Message Type |  | 8 | Type of AAI-FFR-REP | NA |
| M | frequencyPartitionBitMap |  | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value “1”. The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. Bitmap referring to the frequency partitions [FP0, FP1, FP2, FP3], where the LSB refers to FP0 and the MSB refers to FP3. Each bit indicates if a report on the frequency partition is required where a value of 1 means a report is required. If the frequency partition size is zero, the relevant bit in frequencyPartitionBitMap should be ignored. Frequency Partition Bitmap shall contain at least one bit with value “1”. | NA |
| M | reportType |  | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value ‘1’, it indicates the specific report type is being sent, otherwise it indicates the specific report type is not being sent. At least 1 bit needs to be set to value ‘1’. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| O | FFR-REPORT | fp0Report |  | The measurement report for FP0 as described in FFR Feedback IE | Present when 0-th LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp1Report |  | The measurement report for FP1 as described in FFR Feedback IE | Present when 1-st LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp2Report |  | The measurement report for FP2 as described in FFR Feedback IE | Present when 2-nd LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp3Report |  | The measurement report for FP3 as described in FFR Feedback IE | Present when 3-th LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-FeedbackIE | interferenceMean | 8 | Interference mean of the frequency partition as defined in 16.3.14 This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
| O |  | interferenceVariance | 4 | Interference variance of the frequency partition as defined in 16.3.14 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
| O |  | sinrMean | 8 | SINR mean of the frequency partition as defined in 16.3.14 -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType is 0b1 |
| O |  | sinrVariance | 4 | SINR variance of the frequency partition as defined in 16.3.14 0dB to 15dB in units of 1dB | Present when 3-rd LSB of reportType is 0b1 |