| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control message structure change |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Joey Chou Intel Wookbong Lee LGE | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes changes to MAC control messsage structure |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- MAC control message structure change

- Joey Chou, Intel

- Wookbong Lee, LGE

- Introduction

This contribution provides changes to MAC message structure to repond to some comments received at the Orlando meeting, for the following messages.

- AAI_SCN-REQ

- AAI_SCN-RSP

- AAI_SCN-REP

- AAI_RNG-REQ

- AAI_RNG-RSP

- AAI_RNG-ACK

- AAI_REG-REQ

- AAI_REG-RSP

- AAI_FFR-REP

- AAI_DSA-REQ

- AAI_DSC-REQ

- AAI_DL-IM

- AAI_SCD

For example, in the table below extracted from AAI-SCD message, the structure uLFeedbackInfo is optional. But, some attributes have to be present when uLFeedbackInfo is present.

| O | O | uLFeedbackInfo | primaryCarrierIndex | 6 | the primary carrier index | Loop of NumPC |
| --- | --- | --- | --- | --- | --- | --- |
|  | M |  | startDLRUIndex |  | the start DLRUs index for feedback channel | Loop of NumPC |
|  | M |  | dLRUNum |  | the number of DLRUs for feedback channel | Loop of NumPC |
|  | M |  | hARQChannelNum |  | the number of HARQ feedback channel per HARQ feedback region | Loop of NumPC |

- Proposed text

II.1 Proposed text 1

16.2.3.13 AAI_SCN-REQ

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Update Table 688 as the following]

Table 688: AAI_SCN-REQ

| M/O | Attributes / Array of attributes |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O | nbrBsIndexArr [0..63] |  | Neighbor BS index |  |  | Variable ( 8 x N) | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Present if N_Recommended_BS_Index > 0 |
| O | nbrBsIndexArray [0..63] |  |  |  |  | Variable |  | Present if N_Recommended_BS_Index > 0 |
| O | A) Neighbor BS index |  |  |  |  | 8 | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Present if N_Recommended_BS_Index > 0 |
| O | fullBsIdArr [0..63] |  |  | Recommended BS ID |  | Variable ( 48 x N) | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Present if N_Recommended_BS_Full > 0 |
| O | fullBsIdArray [0..63] |  |  |  |  | Variable |  | Present if N_Recommended_BS_Full > 0 |
| O | A) Recommended BS ID |  |  |  |  | 48 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Present if N_Recommended_BS_Full > 0 |
| O | preambleIndexArr[0..14] |  |  | Preamble Index |  | Variable ( 6 x N) | Based on the value of N_Recommended_BS_Full, identifiers of the BSs that the MS plans to scan is included. | Present if N_Recommended_Preambe_Index > 0 |
| O | preambleIndexArray [0..14] |  |  |  |  | Variable |  | Present if N_Recommended_Preambe_Index > 0 |
| O | A) Preamble Index |  |  |  |  | 6 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs that the MS plans to scan is included. | Present if N_Recommended_Preambe_Index > 0 |
| O | nbrAbsArr[0..63] | N_Recommended_Carrier_Index |  |  |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighboring ABS [0..63]. | Present when a neighboring ABS is a multicarrier ABS and, N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O |  | carrierIndexArr[0..62] |  |  | Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O | nbrAbsArray [0..63] |  |  |  |  | Variable | Neighbor ABS information |  |
| O | A) N_Recommended_Carrier_Index |  |  |  |  | 6 | Number of carriers to be scanned at each neighboring ABS [0..63]. | Present when a neighboring ABS is a multicarrier ABS and, N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O | B) carrierIndexArray [0..62] |  |  |  |  | Variable |  |  |
| O | B.1) Recommended_Carrier_Index |  |  |  |  | 6 | Recommended physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O | carIndexArr [ 0..14] | Recommended Carrier index at serving ABS |  |  |  | Variable ( 6 x N) | Recommended physical carrier index of the serving ABS to be scanned | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | carrierIndexArray [0..14] |  |  |  |  | Variable |  | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | A) Recommended SABS Carrier index |  |  |  |  | 6 | Recommended physical carrier index of the serving ABS to be scanned |  |
| O | csgIdArr[0..15] | CSG Identifier |  |  |  | Variable (8 x N) | Sent by AMS to aid ABS reselection | Present if CSG identifier is available |
| O | csgIdArray [0..15] |  |  |  |  | Variable |  | Present if CSG identifier is available |
| O | A) CSG Identifier |  |  |  |  | 8 | Sent by AMS to aid ABS reselection |  |

------------------------------------------------- End of proposed text I--------------------------------------------------

II.2 Proposed text 2

16.2.3.14 AAI_SCN-RSP

------------------------------------------------- Start of proposed text II --------------------------------------------------

[Update Table 689 as the following]

Table 689: AAI_SCN-RSP

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | nbrBsIndexArr [0..63] | Neighbor BS index |  | Variable ( 8 x N) | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Present if N_Recommended_BS_Index > 0 |
| O | nbrBsIndexArray [0..63] |  |  | Variable |  | Present if N_Recommended_BS_Index > 0 |
| O | A) Neighbor BS index |  |  | 8 | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. |  |
| O | fullBsIdArr [0..63] | Recommended BS ID |  | Variable ( 48 x N) | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Present if N_Recommended_BS_Full > 0 |
| O | fullBsIdArray [0..63] |  |  | Variable |  | Present if N_Recommended_BS_Full > 0 |
| O | Recommended BS ID |  |  | 48 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. |  |
| O | preambleIndexArr[0..14] | Preamble Index |  | Variable ( 6 x N) | Preamble Index of Recommended BS ID | Present if N_Recommended_Preambe_Index > 0 |
| O |  | Physical carrier index |  |  | As defined in AAI_Global-Config message | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O |  | Channel bandwidth of recommended BS ID |  |  | Channel bandwidth of recommended BS ID in units of 125 KHz. Field length is 1 byte. | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O |  | CP length |  |  | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O | preambleIndexArray [0..14] |  |  | Variable |  | Present if N_Recommended_Preambe_Index > 0 |
| O | A) Preamble Index |  |  | 6 | Preamble Index of Recommended BS ID | Present if N_Recommended_Preambe_Index > 0 |
| O | B) Physical carrier index |  |  |  | As defined in AAI_Global-Config message | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O | C) Channel BW of recommended BS ID |  |  |  | Channel bandwidth of recommended BS ID in units of 125 KHz. Field length is 1 byte. | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O | D) CP length |  |  |  | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when target ABS is not included in AAI_NBR-ADV message |
| O | nbrAbsArr[0..63] | N_Recommended_Carrier_Index |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighboring ABS [0..63]. | Present when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O |  | carrierIndexArr[0..62] | Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O | nbrAbsArray [0..63] |  |  | Variable |  | Present when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O | A) N_Recommended_Carrier_Index |  |  | 6 | Number of carriers to be scanned at each neighboring ABS [0..63]. |  |
| O | B) carrierIndexArray [0..62] |  |  | Variable |  | Present if N_Recommended_Carrier_Index > 0 |
| O | B.1)Recommended_Carrier_Index |  |  | 6 | Recommended physical carrier index for scan at each neighboring ABS. |  |
| O | carIndexArr [ 0..62] | Carrier index at serving ABS |  | Variable ( 6 x N) | Recommended physical carrier index of the serving ABS to be scanned | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | carIndexArr [ 0..62] |  |  | Variable |  | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| O | A) Carrier index at serving ABS |  |  | 6 | Recommended physical carrier index of the serving ABS to be scanned |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

II.3 Proposed text

16.2.3.15 AAI_SCN-REP

------------------------------------------------- Start of proposed text III --------------------------------------------------

[Update Table 670 as the following]

Table 670: AAI_SCN-REP

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | nbrBsIndexArr [0..63] | Neighbor BS index |  | Variable ( 8 x N) | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. | Present only when the scanning report is based on BS index |
| O | nbrBsIndexArray [0..63] |  |  | Variable |  | Present only when the scanning report is based on BS index |
| O | A) Neighbor BS index |  |  | 8 | Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included. |  |
| O | fullBsIdArr [0..63] | Recommended BS ID |  | Variable ( 48 x N) | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Present if N_Recommended_BS_Full > 0 |
| O | fullBsIdArray [0..63] |  |  | Variable |  | Present if N_Recommended_BS_Full > 0 |
| O | A) Recommended BS ID |  |  | 8 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. |  |
|  | faIndexArr[0.14] | faIndex |  | ? | Note: needs definition of FA index |  |
| O | faIndexArray [0.14] |  |  | Variable |  |  |
| O | A) faIndex |  |  | ? | Note: needs definition of FA index |  |
| O | preambleIndexArr[0..14] | Preamble Index |  | Variable ( 10 x N) | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. | Present if N_Recommended_Preambe_Index > 0 |
|  |  | Report value |  | Variable (? X N) | Based on the value of Report metric Note: needs definition of report value |  |
| O | preambleIndexArray [0..14] |  |  | Variable |  | Present if N_Recommended_Preambe_Index > 0 |
| O | A) Preamble Index |  |  | 10 | Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included. |  |
| O | B) Report value |  |  | ? | Based on the value of Report metric Note: needs definition of report value |  |
| O | nbrAbsArr[0..63] | N_Recommended_Carrier_Index |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighboring ABS [0..63]. | Present when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O |  | carrierIndexArr[0..62] | Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighboring ABS. | Present if N_Recommended_Carrier_Index > 0 |
| O | nbrAbsArray [0..63] |  |  | Variable |  | Present when N_Recommended_BS_Index > 0 or N_Recommended_BS_Full > 0 |
| O | A) N_Recommended_Carrier_Index |  |  | 6 | Number of carriers to be scanned at each neighboring ABS [0..63]. |  |
| O | B) carrierIndexArr[0..62] |  |  | Variable |  | Present if N_Recommended_Carrier_Index > 0 |
| O | B.1) Recommended_Carrier_Index |  |  | 6 | Recommended physical carrier index for scan at each neighboring ABS |  |

------------------------------------------------- End of proposed text III --------------------------------------------------

II.4 Proposed text

16.2.3.1 AAI_RNG-REQ

------------------------------------------------- Start of proposed text IV --------------------------------------------------

[Update Table 680 as the following]

Table 680: AAI_RNG-REQ

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | CMAC Tuple | PMK SN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication. |
|  |  | CMAC_PN | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
|  |  | CMAC value | 64 | The CMAC value derived from the whole message. |  |
| O | CMAC Tuple |  | 100 |  | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication |
| O | A) PMK SN |  | 4 | The sequence number of PMK used to derive the current AK. |  |
| O | B) CMAC_PN |  | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
| O | C) CMAC value |  | 64 | The CMAC value derived from the whole message. |  |

------------------------------------------------- End of proposed text IV--------------------------------------------------

II.5 Proposed text

16.2.3.2 AAI_RNG-RSP

------------------------------------------------- Start of proposed text V --------------------------------------------------

[Update Table 681 as the following]

Table 681: AAI_RNG-RSP

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | FID_update[0…TBD] [TBD] | SFID | 32 | SFID which need to update existing flow. | It shall be included if the ABS needs to update AMS's existing flows. FIDs which are not appeared in this field shall be regarded as guranteed by the ABS. |
|  |  | Update or delete | 1 | 0: delete the flow 1: updated QoS Info exists |  |
|  |  | Updated QoS Info | TBD[ editorial Note: refer to DSA message] | All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this field. |  |
| O | FID_update[0…TBD] [TBD] |  | Variable |  | It shall be included if the ABS needs to update AMS's existing flows. FIDs which are not appeared in this field shall be regarded as guranteed by the ABS. |
| O | A) SFID |  | 32 | SFID which need to update existing flow. |  |
| O | B) Update or delete |  | 1 | 0: delete the flow 1: updated QoS Info exists |  |
| O | C) Updated QoS Info |  | TBD[ editorial Note: refer to DSA message] | All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this field. |  |

------------------------------------------------- End of proposed text V--------------------------------------------------

II.6 Proposed text

16.2.3.3 AAI_RNG-ACK

------------------------------------------------- Start of proposed text VI --------------------------------------------------

[Update Table 682 as the following]

Table 682: AAI_RNG-ACK

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Ranging responses[0..32] | Code Index | 6 | Code index received in this ranging opportunity. | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 |
|  |  | Ranging Status | 2 | Used to indicate whether UL messages are received within acceptable limits by ABS. 0b01 = continue 0b10 = abort 0b11 = success | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1 |
|  |  | Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1. In this case, Bit #2 is always set to 0. |
|  |  | Timing offset adjustment | 10 | Time offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and and API bit #0 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #0 = 1 |
|  |  | Power level adjustment | 3 | Power level adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #1 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #1 = 1 |
|  |  | Frequency offset adjustment | 8 | Frequency offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #2 = 1 |
| O | Ranging responses[0..32] |  |  |  |  |
| O | A) Code Index |  | 6 | Code index received in this ranging opportunity. | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 |
| O | B) Ranging Status |  | 2 | Used to indicate whether UL messages are received within acceptable limits by ABS. 0b01 = continue 0b10 = abort 0b11 = success | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1 |
| O | C) Adjustment parameters indication (API) |  | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 Or It shall be included for the specific user when Unicast indication =1. In this case, Bit #2 is always set to 0. |
| O | D) Timing offset adjustment |  | 10 | Time offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and and API bit #0 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #0 = 1 |
| O | E) Power level adjustment |  | 3 | Power level adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #1 = 1 Or It shall be included for the specific user when Unicast indication =1 and API bit #1 = 1 |
|  | F) Frequency offset adjustment |  | 8 | Frequency offset adjustment | It shall be included for the specific user when Unicast indication =0 and RNG-ACK bitmap=1 and number of received codes > 0 and API bit #2 = 1 |

------------------------------------------------- End of proposed text VI--------------------------------------------------

II.7 Proposed text

16.2.3.3 AAI_REG-REQ

------------------------------------------------- Start of proposed text VII --------------------------------------------------

[Update Table 682 as the following]

Table 682: AAI_RNG-REQ

| M/O | Attributes / Array of attributes |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O | AMS capability negotiation parameters | ARQ parameters |  | ARQ_SN_MODULUS |  | 10 | the number of unique SN values |  |
| O |  |  |  | ARQ_WINDOW_SIZE |  | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O |  |  |  | ARQ_SUB_BLOCK_SIZE |  | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  |  | ARQ_BLOCK_LIFETIME |  | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  |  | ARQ_RX_PURGE_TIMEOUT |  | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O |  |  |  | MAXIMUM_ ARQ_BUFFER_SIZE |  | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O |  | Multicarrier capabilities |  |  |  | 2 | 0b00: No MC modes 0b01: Multicarrier aggregation 0b10: Multicarrier switching 0b11: Both multicarrier aggregation and switching |  |
|  |  | Capabilities for Zone switching mode | Zone Switch Mode Support |  |  |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported |  |
|  |  |  | Reserved |  |  |  |  |  |
| O |  | LBS Capabilities | Capability for supporting A-GPS method |  |  | 1 | 0b0: no support 0b1: support |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability |  |  | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
|  |  |  | DL collaborative multi-BS MIMO capability |  |  | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
|  |  |  | DL closed-loop multi-BS macro diversity capability |  |  | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro di¬versity capable |  |
| O |  |  | UL PMI combination capa¬bility |  |  | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |  |
| O |  | E-MBS capabilities |  |  |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O |  | channel BW and Cyclic prefix |  |  |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O |  | frame configuration to support legacy(5MHz) |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | frame configuration to support legacy(10MHz) |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | MAX Tx Power |  |  |  | 24 | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. |  |
| O |  | 3-step BR |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Persistent Allocation support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Group Resource Allocation support |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Co-located coexistence capability support |  |  |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O |  | HO trigger metric |  |  |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O |  | Capabilities for interference mitigation support |  |  | DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O |  |  |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O |  | AMS Scanning Capability |  |  |  | 24 | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: - Number of Center Frequencies AMS can scan during one scan iteration. - Maximum number of preambles per Center Frequency AMS can scan during one iteration. - Number of frames required to complete scanning of all preambles for a given frequency The length of the field is 3 Bytes, which are divided as follows: Bit# 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame Bit# 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bit# 14-15: Minimum supported scan duration • (0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) Bit# 16-23: Maximum supported scan duration • (0x0 = no limit • All other values: maximum scan duration in number of sub-frames)) |  |
| O | AMS capability negotiation parameters |  |  |  |  | variable |  |  |
| O | A) ARQ parameters |  |  |  |  | variable |  |  |
| O | A.1) ARQ_SN_MODULUS |  |  |  |  | 10 | the number of unique SN values |  |
| O | A.2) ARQ_WINDOW_SIZE |  |  |  |  | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O | A.3) ARQ_SUB_BLOCK_SIZE |  |  |  |  | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.4) ARQ_BLOCK_LIFETIME |  |  |  |  | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.5) ARQ_RX_PURGE_TIMEOUT |  |  |  |  | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O | A.6) MAXIMUM_ARQ_BUFFER_SIZE |  |  |  |  | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O | B) Multicarrier capabilities |  |  |  |  | 2 | 0b00: No MC modes 0b01: Multicarrier aggregation 0b10: Multicarrier switching 0b11: Both multicarrier aggregation and switching |  |
| O | C) Capabilities for Zone switching mode |  |  |  |  |  |  |  |
| O | C.1 Zone Switch Mode Support |  |  |  |  |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported |  |
|  | C.2 Reserved |  |  |  |  |  |  |  |
| O | D) LBS Capabilities |  |  |  |  | 1 | 1 |  |
| O | D,1) Capability for supporting A-GPS method |  |  |  |  | 1 | 0b0: no support 0b1: support |  |
| O | E) Capabilities for interference mitigation support |  |  |  |  |  |  |  |
| O | E.1) DL PMI coordination capability |  |  |  |  | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O | E.2) DL collaborative multi-BS MIMO capability |  |  |  |  | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O | E.3) DL closed-loop multi-BS macro diversity capability |  |  |  |  | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro di¬versity capable |  |
| O | E.4) UL PMI combination capa¬bility |  |  |  |  | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable |  |
| O | F) E-MBS capabilities |  |  |  |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O | G) channel BW and Cyclic prefix |  |  |  |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O | H) frame configuration to support legacy(5MHz) |  |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O | I) frame configuration to support legacy(10MHz) |  |  |  |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O | J) OFDMA system support |  |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O | K) MAX Tx Power |  |  |  |  | 24 | Bit 0–7: Maximum transmitted power for QPSK. Bit 8–15: Maximum transmitted power for 16-QAM Bit 15–23: Maximum transmitted power for 64-QAM. |  |
| O | L) 3-step BR |  |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O | M) dynamic ranging for HO |  |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O | N) Persistent Allocation support |  |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O | O) Group Resource Allocation support |  |  |  |  | 1 | If Bit#0 =1,it supports |  |
| O | Q) Co-located coexistence capability support |  |  |  |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O | R) HO trigger metric |  |  |  |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O | S) Capabilities for Handover EBB mode |  |  |  |  | Variable |  |  |
| O | S.1) EBB support |  |  |  |  | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported |  |
| O | S.2) Minimal HO_Reentry_Interleaving_Interval |  |  |  |  |  | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 |  |
| O | T) Capabilities for interference mitigation support |  |  |  |  | Variable |  |  |
| O | T.1) DL PMI coordination capability |  |  |  |  | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable |  |
| O | T.2) DL collaborative multi-BS MIMO capability |  |  |  |  | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable |  |
| O | U) AMS Scanning Capability |  |  |  |  | 24 | This field indicates properties of the AMS that the ABS needs to know for scanning purposes: - Number of Center Frequencies AMS can scan during one scan iteration. - Maximum number of preambles per Center Frequency AMS can scan during one iteration. - Number of frames required to complete scanning of all preambles for a given frequency The length of the field is 3 Bytes, which are divided as follows: Bit# 0-4: Indicates the maximum number of preambles using the same center frequency that AMS can scan within one frame Bit# 5-7: Number of sub-frames the AMS requires to switch from the serving ABS to the first center frequency. This value is measured from the end of the last frame during which the AMS is connected to the S-ABS (and may have to receive/send data) until the beginning of the first frame in which a measurement of a neighboring ABS in a different center frequency can be performed. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 8-10: The number of sub-frames the AMS requires to switch between center frequencies. This is the time duration (in sub-frames) between a sub-frame in which a center frequency was scanned until the beginning of a sub-frame in which a new center frequency can be scanned. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bits# 11-13: The number of sub-frames the AMS requires to switch from the last center frequency back to the serving ABS. This is the time duration (in sub-frames) between a frame in which a center frequency was scanned until the beginning of the first frame in which the AMS is available at the S-ABS and may receive/transmit data. • 0b000 = 0 sub-frames • 0b111 = 7 sub-frames) Bit# 14-15: Minimum supported scan duration • (0b000 = no limit, 0b001= 1 sub-frame, 0b111= 7 sub-frames) Bit# 16-23: Maximum supported scan duration • (0x0 = no limit • All other values: maximum scan duration in number of sub-frames)) |  |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  |  |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  |  |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  |  |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Supported-IP-Versions IE |  |  |  | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support |  |
| O | Convergence sublayer capabilities |  |  |  |  |  |  |  |
| O | A) assification/PHS options and SDU encapsulation support |  |  |  |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O | B) Maximum number of classification rules |  |  |  |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O | C) ROHC support |  |  |  |  | 1 | 0: not support 1: support |  |
| O | D) PHS support |  |  |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O | E) Supported-IP-Versions IE |  |  |  |  | 2 | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4 and IPv6 support |  |
| O | Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE |  |  |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O |  | Requested-Host-Configurations IE |  |  |  | 1 | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |
| O | Host configuration capabilities and parameters |  |  |  |  |  |  |  |
| O | A) Host-Configuration-Capability-Indicator IE |  |  |  |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O | B) Requested-Host-Configurations IE |  |  |  |  | 1 | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |

------------------------------------------------- End of proposed text VII--------------------------------------------------

II.8 Proposed text

16.2.3.3 AAI_REG-RSP

------------------------------------------------- Start of proposed text VIII --------------------------------------------------

[Update Table 682 as the following]

Table 682: AAI_REG-RSP

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
| O |  |  | ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O |  |  | ARQ_SUB_BLOCK_SIZE | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  | ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  |  | ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O |  |  | MAXIMUM_ ARQ_BUFFER_SIZE | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
|  |  | Femto ABS LDM parameters | Start_frame_number |  | Start superframe number |  |
|  |  |  | Available_interval_length |  | Length of available interval in units of four frames |  |
|  |  |  | Unavailable_interval_length |  | Length of unavailable intervals in units of four frames |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |  |
| O |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |  |
| O |  |  | DL closed-loop multi-BS macro diversity capability | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |  |
| O | AMS capability negotiation parameters |  |  | Variable |  |  |
| O | A) ARQ parameters |  |  | Variable |  |  |
| O | A.1) ARQ_SN_MODULUS |  |  | 10 | the number of unique SN values |  |
| O | A.2) ARQ_WINDOW_SIZE |  |  | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O | A.3) ARQ_SUB_BLOCK_SIZE |  |  | TBD | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.4) ARQ_BLOCK_LIFETIME |  |  | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O | A.5) ARQ_RX_PURGE_TIMEOUT |  |  | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O | A.6) MAXIMUM_ARQ_BUFFER_SIZE |  |  | TBD | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O | B) Femto ABS LDM parameters |  |  |  |  |  |
| O | B.1) Start_frame_number |  |  |  | Start superframe number |  |
| O | B.2) Available_interval_length |  |  |  | Length of available interval in units of four frames |  |
| O | B.3) Unavailable_interval_length |  |  |  | Length of unavailable intervals in units of four frames |  |
| O | C) Capabilities for interference mitigation support |  |  |  |  |  |
| O | C.1) DL PMI coordination capability |  |  | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |  |
| O | C.2) DL collaborative multi-BS MIMO capability |  |  | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |  |
| O | C.3) DL closed-loop multi-BS macro diversity capability |  |  | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |  |
| O | C.4) UL PMI combination capability |  |  | 1 | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capable |  |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Resource_Retain_Time |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O |  | IP-Service-Type IE |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved |  |
| O | Convergence sublayer capabilities |  |  |  |  |  |
| O | A) Classification/PHS options and SDU encapsulation support |  |  | TBD | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O | B) Maximum number of classification rules |  |  | TBD | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O | C) ROHC support |  |  | 1 | 0: not support 1: support |  |
| O | D) PHS support |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O | E) Resource_Retain_Time |  |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O | F) IP-Service-Type IE |  |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved |  |
| O | Host configuration capabilities and its parameters | IPv4-Host-Address IE |  | 32 | The allocated IPv4 Host Address for the AMS. |  |
| O |  | IPv6-Home-Network-Prefix IE |  | 64 | The allocated IPv6 Home Network Prefix for the AMS. |  |
| O |  | Additional-Host-Configurations IE |  | TBD | Includes additional host configuration options in DHCP Options format |  |
| O | Host configuration capabilities and its parameters |  |  | Variable |  |  |
| O | A) IPv4-Host-Address IE |  |  | 32 | The allocated IPv4 Host Address for the AMS. |  |
| O | B) IPv6-Home-Network-Prefix IE |  |  | 64 | The allocated IPv6 Home Network Prefix for the AMS. |  |
| O | C) Additional-Host-Configurations IE |  |  | TBD | Includes additional host configuration options in DHCP Options format |  |
| O | Redirection Info | ABSID, preamble index and center frequency for one or more neighbor ABS |  | TBD | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |
| O | Redirection Info |  |  | Variable |  |  |
| O | ABSID, preamble index and center frequency for one or more neighbor ABS |  |  | TBD | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |

------------------------------------------------- End of proposed text VIII--------------------------------------------------

II.9 Proposed text

16.2.3.3 AAI_FFR-REP

------------------------------------------------- Start of proposed text IX --------------------------------------------------

[Update Table 706 as the following]

Table 706: AAI_FFR-REP

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | FFR-REPORT | fp0Report |  |  | Present when 0-th LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp1Report |  |  | Present when 1-st LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp2Report |  |  | Present when 2-nd LSB of frequencyPartitionBitMap is 0b1 |
| O |  | fp3Report |  |  | Present when 3-th LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-REPORT |  |  |  |  |
| O | A) fp0Report |  |  |  | Present when 0-th LSB of frequencyPartitionBitMap is 0b1 |
| O | B) fp1Report |  |  |  | Present when 1-st LSB of frequencyPartitionBitMap is 0b1 |
| O | C) fp2Report |  |  |  | Present when 2-nd LSB of frequencyPartitionBitMap is 0b1 |
| O | D) fp3Report |  |  |  | Present when 3-th LSB of frequencyPartitionBitMap is 0b1 |
| O | FFR-FeedbackIE | interferenceMean | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
| O |  | interferenceVariance | 4 | Interference variance 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
| O |  | sinrMean | 8 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType is 0b1 |
| O |  | sinrVariance | 4 | SINR variance 0dB to 15dB in units of 1dB | Present when 3-rd LSB of reportType is 0b1 |
| O | FFR-FeedbackIE |  |  |  |  |
| O | A) interferenceMean |  | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
| O | B) interferenceVariance |  | 4 | Interference variance 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
| O | C) sinrMean |  | 8 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType is 0b1 |
| O | D) sinrVariance |  | 4 | SINR variance 0dB to 15dB in units of 1dB | Present when 3-rd LSB of reportType is 0b1 |

------------------------------------------------- End of proposed text IX--------------------------------------------------

II.10 Proposed text

16.2.3.45.1 AAI_DSA-REQ

------------------------------------------------- Start of proposed text X --------------------------------------------------

[Update Table 740 as the following]

Table 740: AAI_DSA-REQ

| M/O | Attributes / Array of attributes |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O | QoS parameters |  | Traffic Priority parameter |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 |  |
| O |  |  | Maximum Sustained Traffic Rate parameter |  |  | 32 | Rate (in bits per second) |  |
| O |  |  | Maximum Traffic Burst parameter |  |  | 32 | Burst size (bytes) |  |
| O |  |  | Minimum Reserved Traffic Rate parameter |  |  | 32 | Rate (in bits per second) |  |
| O |  |  | Maximum Latency parameter |  |  | 32 | Milliseconds |  |
| O |  |  | Paging Preference parameter |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O |  |  | Request/Transmission Policy parameter |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O |  |  | UL Grant Scheduling Type parameter |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O |  |  | Tolerated Jitter parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  |  | Unsolicited Grant Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O |  |  | Unsolicited Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O |  |  | Primary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O |  |  | Primary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O |  |  | Secondary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  |  | Secondary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  |  | Adaptation Method |  |  | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only. | Present when UL scheduling type is aGP Service |
| O |  |  | Access Class |  |  | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1.. | Present for UL |
| O |  |  | Differentiated BR Timer |  |  | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s). | Present for UL |
| O |  |  | Quick Access Predefined BR Index Parameter |  |  | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O |  |  | MAC in-order delivery indicator. | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved. |  |
| O |  |  | Vendor Specific QoS Parameter |  |  | variable |  |  |
| O | QoS parameters |  |  |  |  | variable |  |  |
| O | A) Traffic Priority parameter |  |  |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 |  |
| O | B) Maximum Sustained Traffic Rate parameter |  |  |  |  | 32 | Rate (in bits per second) |  |
| O | C) Maximum Traffic Burst parameter |  |  |  |  | 32 | Burst size (bytes) |  |
| O | D) Minimum Reserved Traffic Rate parameter |  |  |  |  | 32 | Rate (in bits per second) |  |
| O | E) Maximum Latency parameter |  |  |  |  | 32 | Milliseconds |  |
| O | F) Paging Preference parameter |  |  |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O | G) Request/Transmission Policy parameter |  |  |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O | H) UL Grant Scheduling Type parameter |  |  |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O | I) Tolerated Jitter parameter |  |  |  |  | 32 | Milliseconds | Present when needed |
| O | J) Unsolicited Grant Interval parameter |  |  |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O | K) Unsolicited Polling Interval parameter |  |  |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O | L) Primary Grant Polling Interval parameter |  |  |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O | M) Primary Grant Size parameter |  |  |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O | N) Secondary Grant Polling Interval parameter |  |  |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | O) Secondary Grant Size parameter |  |  |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | P) Adaptation Method |  |  |  |  | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only. | Present when UL scheduling type is aGP Service |
| O | Q) Access Class |  |  |  |  | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1.. | Present for UL |
| O | R) Differentiated BR Timer |  |  |  |  | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s). | Present for UL |
| O | S) Quick Access Predefined BR Index Parameter |  |  |  |  | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O | T) MAC in-order delivery indicator |  |  |  |  | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved. |  |
| O | U) Vendor Specific QoS Parameter |  |  |  |  | variable |  |  |
| O | ARQ Parameters | ARQ Enable |  |  |  | 8 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted |  |
| O |  | ARQ_WINDOW_SIZE |  |  |  | 16 | >0 and <=(ARQ_BSN_MODULUS/2) |  |
| O |  | ARQ_BLOCK_LIFETIME |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_SYNC_LOSS_TIMEOUT |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_RX_PURGE_TIMEOUT |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O |  | ARQ_SUB_BLOCK_SIZE |  |  |  | 8 |  |  |
| O |  | ARQ_ERROR_DETECTION_TIMEOUT |  |  |  | 16 |  |  |
| O |  | ARQ_FEEDBACK_POLL_RETRY_TIMEOUT |  |  |  | 16 |  |  |
| O |  | RECEIVER_ARQ_ACK_PROCESSING_TIME |  |  |  | 1 | 0–255 |  |
| O | ARQ Parameters |  |  |  |  |  |  |  |
| O | A) ARQ Enable |  |  |  |  | 8 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted |  |
| O | B) ARQ_WINDOW_SIZE |  |  |  |  | 16 | >0 and <=(ARQ_BSN_MODULUS/2) |  |
| O | C) ARQ_BLOCK_LIFETIME |  |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O | D) ARQ_SYNC_LOSS_TIMEOUT |  |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O | E) ARQ_RX_PURGE_TIMEOUT |  |  |  |  | 16 | 0 = Infinite 1–6553500 µs (100 µs granularity) |  |
| O | F) ARQ_SUB_BLOCK_SIZE |  |  |  |  | 8 |  |  |
| O | G) ARQ_ERROR_DETECTION_TIMEOUT |  |  |  |  | 16 |  |  |
| O | H) ARQ_FEEDBACK_POLL_RETRY_TIMEOUT |  |  |  |  | 16 |  |  |
| O | I) RECEIVER_ARQ_ACK_PROCESSING_TIME |  |  |  |  | 1 | 0–255 |  |
| O | Packet Classification Rule parameter |  | Classification Rule Priority field |  |  | 8 | 0–255 |  |
| O |  |  | Protocol field |  |  | 8 | protocol |  |
| O |  |  | IP Masked Source Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O |  |  | IP Masked Destination Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O |  |  | Protocol Source Port Range field |  |  | 32 | sportlow, sporthigh |  |
| O |  |  | Protocol Destination Port Range field |  |  | 32 | dportlow, dporthigh |  |
| O |  |  | IEEE 802.3/Ethernet Destination MAC Address parameter |  |  | 96 | dst, msk |  |
| O |  |  | IEEE 802.3/Ethernet Source MAC Address parameter |  |  | 96 | src, msk |  |
| O |  |  | Ethertype/IEEE 802.2 SAP |  |  | 24 | type, eprot1, eprot2 |  |
| O |  |  | IEEE 802.1D User Priority field |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O |  |  | IEEE 802.1Q VLAN ID field |  |  | 16 | vlan_id1, vlan_id2 |  |
| O |  |  | Associated PHSI field |  |  | 8 | Index value |  |
| O |  |  | Packet Classification Rule Index field |  |  | 16 | Packet Classification Rule Index |  |
| O |  |  | Vendor-specific classification parameters |  |  | variable | Compound |  |
| O |  |  | IPv6 Flow Label field |  |  | 24 | Flow Label |  |
| O |  |  | Classification Action Rule |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O |  |  | IP Type of Service |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O |  |  | PHS DSC Action field |  |  | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O |  |  | PHS Rule field |  | PHSI field | 8 | Index value |  |
| O |  |  |  |  | PHSF field | n | String of bytes suppressed |  |
| O |  |  |  |  | PHSM field | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O |  |  |  |  | PHSS field | 8 | Number of bytes in the suppression string |  |
| O |  |  |  |  | PHSV field | 1 | 0: Verify 1: Don’t verify |  |
| O |  |  |  |  | Vendor-specific PHS parameters | variable | Compound |  |
| O | Packet Classification Rule parameter |  |  |  |  |  |  |  |
| O | A) Classification Rule Priority field |  |  |  |  | 8 | 0–255 |  |
| O | B) Protocol field |  |  |  |  | 8 | protocol |  |
| O | C) IP Masked Source Address parameter |  |  |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O | D) IP Masked Destination Address parameter |  |  |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O | E) Protocol Source Port Range field |  |  |  |  | 32 | sportlow, sporthigh |  |
| O | F) Protocol Destination Port Range field |  |  |  |  | 32 | dportlow, dporthigh |  |
| O | G) IEEE 802.3/Ethernet Destination MAC Address parameter |  |  |  |  | 96 | dst, msk |  |
| O | H) IEEE 802.3/Ethernet Source MAC Address parameter |  |  |  |  | 96 | src, msk |  |
| O | I) Ethertype/IEEE 802.2 SAP |  |  |  |  | 24 | type, eprot1, eprot2 |  |
| O | J) IEEE 802.1D User Priority field |  |  |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O | K) IEEE 802.1Q VLAN ID field |  |  |  |  | 16 | vlan_id1, vlan_id2 |  |
| O | L) Associated PHSI field |  |  |  |  | 8 | Index value |  |
| O | M) Packet Classification Rule Index field |  |  |  |  | 16 | Packet Classification Rule Index |  |
| O | N) Vendor-specific classification parameters |  |  |  |  | variable | Compound |  |
| O | O) IPv6 Flow Label field |  |  |  |  | 24 | Flow Label |  |
| O | Q) Classification Action Rule |  |  |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O | R) IP Type of Service |  |  |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O | S) PHS Rule field |  |  |  |  |  |  |  |
| O | S.1) PHSI field |  |  |  |  | 8 | Index value |  |
| O | S.2) PHSF field |  |  |  |  | n | String of bytes suppressed |  |
| O | S.3) PHSM field |  |  |  |  | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O | S.4) PHSS field |  |  |  |  | 8 | Number of bytes in the suppression string |  |
| O | S.5) PHSV field |  |  |  |  | 1 | 0: Verify 1: Don’t verify |  |
| O | S.6) Vendor-specific PHS parameters |  |  |  |  | variable | Compound |  |
| O | ROHC Parameter |  |  | ROHC Max Context ID |  | 16 | Non-negative integer |  |
| O |  |  |  | Large Context IDs |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O |  |  |  | ROHC MRRU |  | 16 | 0: no segmentation Otherwise: MRRU |  |
| O | ROHC Parameter |  |  |  |  |  |  |  |
| O | A) ROHC Max Context ID |  |  |  |  | 16 | Non-negative integer |  |
| O | B) Large Context IDs |  |  |  |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O | C) ROHC MRRU |  |  |  |  | 16 | 0: no segmentation Otherwise: MRRU |  |

------------------------------------------------- End of proposed text X--------------------------------------------------

II.11 Proposed text

16.2.3.45.4 AAI_DSC-REQ

------------------------------------------------- Start of proposed text XI --------------------------------------------------

[Update table 743 as the following]

Table 743: AAI_DSC-REQ

| M/O | Attributes / Array of attributes |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| O | QoS parameters | Traffic Priority parameter |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 | Present when needed |
| O |  | Maximum Sustained Traffic Rate parameter |  |  | 32 | Rate (in bits per second) | Present when needed |
| O |  | Maximum Traffic Burst parameter |  |  | 32 | Burst size (bytes) | Present when needed |
| O |  | Minimum Reserved Traffic Rate parameter |  |  | 32 | Rate (in bits per second) | Present when needed |
| O |  | Maximum Latency parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  | Fixed-Length Versus Variable-Length SDU Indicator parameter |  |  | 8 | 0: Variable-length SDUs 1: Fixed-length SDUs Default = 0 | Present when needed |
| O |  | Paging Preference parameter |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O |  | Request/Transmission Policy parameter |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not include CRC in the MAC PDU. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O |  | UL Grant Scheduling Type parameter |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O |  | Tolerated Jitter parameter |  |  | 32 | Milliseconds | Present when needed |
| O |  | Unsolicited Grant Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O |  | Unsolicited Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O |  | Primary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O |  | Primary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O |  | Secondary Grant Polling Interval parameter |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O |  | Secondary Grant Size parameter |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | QoS parameters |  |  |  | variable |  |  |
| O | A) Traffic Priority parameter |  |  |  | 3 | 0 to 7—Higher numbers indicate higher priority Default 0 |  |
| O | B) Maximum Sustained Traffic Rate parameter |  |  |  | 32 | Rate (in bits per second) |  |
| O | C) Maximum Traffic Burst parameter |  |  |  | 32 | Burst size (bytes) |  |
| O | D) Minimum Reserved Traffic Rate parameter |  |  |  | 32 | Rate (in bits per second) |  |
| O | E) Maximum Latency parameter |  |  |  | 32 | Milliseconds |  |
| O | F) Paging Preference parameter |  |  |  | 8 | 0: No paging generation 1: Paging generation |  |
| O | G) Request/Transmission Policy parameter |  |  |  | 8 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to’0’ and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to ‘1’, none of the SDUs for this service flow shall have a PHSI field. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 7: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 7 is set to’0’ and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. |  |
| O | H) UL Grant Scheduling Type parameter |  |  |  | 8 | 0: Reserved 1: Undefined (BS implementation-dependenta) 2: BE (default) 3: nrtPS 4: rtPS 5: Extended rtPS 6: UGS 7: aGP Service | Present for UL |
| O | I) Tolerated Jitter parameter |  |  |  | 32 | Milliseconds | Present when needed |
| O | J) Unsolicited Grant Interval parameter |  |  |  | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
| O | K) Unsolicited Polling Interval parameter |  |  |  | 16 | Milliseconds | Present when UL scheduling type is rtPS |
| O | L) Primary Grant Polling Interval parameter |  |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O | M) Primary Grant Size parameter |  |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O | N) Secondary Grant Polling Interval parameter |  |  |  | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | O) Secondary Grant Size parameter |  |  |  | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | P) Adaptation Method |  |  |  | 2 | 00:Implicit adaptation 01:Explicit adaptation, sustained 10: Explicit adaptation, one time only. | Present when UL scheduling type is aGP Service |
| O | Q) Access Class |  |  |  | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1.. | Present for UL |
| O | R) Differentiated BR Timer |  |  |  | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s). | Present for UL |
| O | S) Quick Access Predefined BR Index Parameter |  |  |  | variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O | T) MAC in-order delivery indicator |  |  |  | 1 | Indicate whether or not the order of delivery in non-ARQ connection is preserved by the MAC. 0 : not preserved 1 : preserved. |  |
| O | U) Vendor Specific QoS Parameter |  |  |  | variable |  |  |
| O | Packet Classification Rule parameter | Classification Rule Priority field |  |  | 8 | 0–255 |  |
| O |  | Protocol field |  |  | 8 | protocol |  |
| O |  | IP Masked Source Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O |  | IP Masked Destination Address parameter |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O |  | Protocol Source Port Range field |  |  | 32 | sportlow, sporthigh |  |
| O |  | Protocol Destination Port Range field |  |  | 32 | dportlow, dporthigh |  |
| O |  | IEEE 802.3/Ethernet Destination MAC Address parameter |  |  | 96 | dst, msk |  |
| O |  | IEEE 802.3/Ethernet Source MAC Address parameter |  |  | 96 | src, msk |  |
| O |  | Ethertype/IEEE 802.2 SAP |  |  | 24 | type, eprot1, eprot2 |  |
| O |  | IEEE 802.1D User Priority field |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O |  | IEEE 802.1Q VLAN ID field |  |  | 16 | vlan_id1, vlan_id2 |  |
| O |  | Associated PHSI field |  |  | 8 | Index value |  |
| O |  | Packet Classification Rule Index field |  |  | 16 | Packet Classification Rule Index |  |
| O |  | Vendor-specific classification parameters |  |  | variable | Compound |  |
| O |  | IPv6 Flow Label field |  |  | 24 | Flow Label |  |
| O |  | Classification Action Rule |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O |  | IP Type of Service |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O |  | PHS DSC Action field |  |  | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O |  | PHS Rule field |  | PHSI field | 8 | Index value |  |
| O |  |  |  | PHSF field | n | String of bytes suppressed |  |
| O |  |  |  | PHSM field | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O |  |  |  | PHSS field | 8 | Number of bytes in the suppression string |  |
| O |  |  |  | PHSV field | 1 | 0: Verify 1: Don’t verify |  |
| O |  |  |  | Vendor-specific PHS parameters | variable | Compound |  |
| O | Packet Classification Rule parameter |  |  |  |  |  |  |
| O | A) Classification Rule Priority field |  |  |  | 8 | 0–255 |  |
| O | B) Protocol field |  |  |  | 8 | protocol |  |
| O | C) IP Masked Source Address parameter |  |  |  | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O | D) IP Masked Destination Address parameter |  |  |  | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O | E) Protocol Source Port Range field |  |  |  | 32 | sportlow, sporthigh |  |
| O | F) Protocol Destination Port Range field |  |  |  | 32 | dportlow, dporthigh |  |
| O | G) IEEE 802.3/Ethernet Destination MAC Address parameter |  |  |  | 96 | dst, msk |  |
| O | H) IEEE 802.3/Ethernet Source MAC Address parameter |  |  |  | 96 | src, msk |  |
| O | I) Ethertype/IEEE 802.2 SAP |  |  |  | 24 | type, eprot1, eprot2 |  |
| O | J) IEEE 802.1D User Priority field |  |  |  | 16 | pri-low, pri-high Valid range: 0–7 for pri-low and pri-high |  |
| O | K) IEEE 802.1Q VLAN ID field |  |  |  | 16 | vlan_id1, vlan_id2 |  |
| O | L) Associated PHSI field |  |  |  | 8 | Index value |  |
| O | M) Packet Classification Rule Index field |  |  |  | 16 | Packet Classification Rule Index |  |
| O | N) Vendor-specific classification parameters |  |  |  | variable | Compound |  |
| O | O) IPv6 Flow Label field |  |  |  | 24 | Flow Label |  |
| O | Q) Classification Action Rule |  |  |  | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1–7: Reserved. |  |
| O | R) IP Type of Service |  |  |  | 8 | Bit 0–Bit 1: reserved. Shall be set to 0b00 Bit 2–Bit 7: DSCP value |  |
| O | S) PHS Rule field |  |  |  |  |  |  |
| O | S.1) PHSI field |  |  |  | 8 | Index value |  |
| O | S.2) PHSF field |  |  |  | n | String of bytes suppressed |  |
| O | S.3) PHSM field |  |  |  | n | Bit 0: 0 = Don’t suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don’t suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don’t suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |  |
| O | S.4) PHSS field |  |  |  | 8 | Number of bytes in the suppression string |  |
| O | S.5) PHSV field |  |  |  | 1 | 0: Verify 1: Don’t verify |  |
| O | S.6) Vendor-specific PHS parameters |  |  |  | variable | Compound |  |
| O | ROHC Parameter |  | ROHC Max Context ID |  | 16 | Non-negative integer |  |
| O |  |  | Large Context IDs |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O |  |  | ROHC MRRU |  | 16 | 0: no segmentation Otherwise: MRRU |  |
| O | ROHC Parameter |  |  |  |  |  |  |
| O | A) ROHC Max Context ID |  |  |  | 16 | Non-negative integer |  |
| O | B) Large Context IDs |  |  |  | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2–255: Reserved |  |
| O | C) ROHC MRRU |  |  |  | 16 | 0: no segmentation Otherwise: MRRU |  |

------------------------------------------------- End of proposed text XI--------------------------------------------------

II.12 Proposed text

16.2.3.45.4 AAI_DL-IM

------------------------------------------------- Start of proposed text XII --------------------------------------------------

[Update table 721 as the following]

Table 721—Parameters for AAI_DL-IM Message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | bcSiInfo | twoTxBCSI | 8 | BC_SI in case of two transmit antenna, refer 16.3.7.2.5.5 If LSB#n is '1', the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. | This is transmitted only when multi-BS MIMO is operated |
|  |  | otherTxBCSI | 16 | BC_SI in case of four or eight transmit antenna, refer 16.3.7.2.5.5 If LSB#n is '1', the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. |  |
| O | bcSiInfo |  |  |  |  |
| O | A) twoTxBCSI |  | 8 | BC_SI in case of two transmit antenna, refer 16.3.7.2.5.5 If LSB#n is '1', the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. | This is transmitted only when multi-BS MIMO is operated |
| O | B) otherTxBCSI |  | 16 | BC_SI in case of four or eight transmit antenna, refer 16.3.7.2.5.5 If LSB#n is '1', the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. | This is transmitted only when multi-BS MIMO is operated |
| O | tempBsidSET | bsidSet1 | 8 | tempBsidSET is composed of diversitySetNum number of BS_indices. Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from this BS_index. The Temp_BSID is the order of the ABSs in respect of the BS_index. Temp_BSID = 0 represents the ABS that is referred by the first BS_index in AAI_DL_IM message. | tempBsidSET is only transmitted only when diversitySetNum is higher than 0 |
|  |  | bsidSet2 | 8 |  |  |
|  |  | bsidSet3 | 8 |  |  |
|  |  | bsidSet4 | 8 |  |  |
|  |  | bsidSet5 | 8 |  |  |
|  |  | bsidSet6 | 8 |  |  |
|  |  | bsidSet7 | 8 |  |  |
|  |  | bsidSet8 | 8 |  |  |
|  |  | bsidSet9 | 8 |  |  |
|  |  | bsidSet10 | 8 |  |  |
|  |  | bsidSet11 | 8 |  |  |
|  |  | bsidSet12 | 8 |  |  |
|  |  | bsidSet13 | 8 |  |  |
|  |  | bsidSet14 | 8 |  |  |
|  |  | bsidSet15 | 8 |  |  |
|  |  | bsidSet16 | 8 |  |  |
| o | tempBsidSET |  |  |  |  |
| O | A) bsidSet1 |  | 8 | tempBsidSET is composed of diversitySetNum number of BS_indices. Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from this BS_index. The Temp_BSID is the order of the ABSs in respect of the BS_index. Temp_BSID = 0 represents the ABS that is referred by the first BS_index in AAI_DL_IM message. | tempBsidSET is only transmitted only when diversitySetNum is higher than 0 |
| O | B) bsidSet2 |  | 8 |  |  |
| O | C) bsidSet3 |  | 8 |  |  |
| O | D) bsidSet4 |  | 8 |  |  |
| O | E) bsidSet5 |  | 8 |  |  |
| O | F) bsidSet6 |  | 8 |  |  |
| O | G) bsidSet7 |  | 8 |  |  |
| O | H) bsidSet8 |  | 8 |  |  |
| O | I) bsidSet9 |  | 8 |  |  |
| O | J) bsidSet10 |  | 8 |  |  |
| O | K) bsidSet11 |  | 8 |  |  |
| O | L) bsidSet12 |  | 8 |  |  |
| O | M) bsidSet13 |  | 8 |  |  |
| O | N) bsidSet14 |  | 8 |  |  |
| O | O) bsidSet15 |  | 8 |  |  |
| O | P) bsidSet16 |  | 8 |  |  |

------------------------------------------------- End of proposed text XII--------------------------------------------------

II.13 Proposed text

16.2.3.45.4 AAI_SCD

------------------------------------------------- Start of proposed text XIII --------------------------------------------------

[Update table 717 as the following]

Table 717— AAI_ SCD Message

| M/O |  | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O |  | OL_MIMO_Parameters | OL-Region-ON | 1 | 0 or 1 | OL MIMO parameters |
| O |  |  | OL-Rank1-Configuration | 3 | 0 to 7 | OL MIMO parameters |
| O |  |  | SB-OL-Region-2Size | 4 | 0 to 15 | OL MIMO parameters |
| O |  | OL_MIMO_Parameters |  |  |  |  |
| O |  | A) OL-Region-ON |  | 1 | 0 or 1 | OL MIMO parameters |
| O |  | B) OL-Rank1-Configuration |  | 3 | 0 to 7 | OL MIMO parameters |
| O |  | C) SB-OL-Region-2Size |  | 4 | 0 to 15 | OL MIMO parameters |
| O |  | UlpcDataChannelSet | gammaIotFp0 | 4 | gammaIotFp (IoT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) | The values are broadcasted when the related frequency partitions are used |
| O |  |  | gammaIotFp1 | 4 |  |  |
| O |  |  | gammaIotFp2 | 4 |  |  |
| O |  |  | gammaIotFp3 | 4 |  |  |
| M |  |  | Alpha | 3 | alpha () is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} | NA |
| M |  |  | Beta | 1 | It is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. 0: disable 1: enable | NA |
| M |  |  | dataSinrMin | 4 | dataSinrMin is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{-INF ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} | NA |
| M |  |  | dataSinrMax | 4 | dataSinrMax is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 34, 36, 38, 40} | NA |
| O |  | UlpcDataChannelSet |  |  |  |  |
| O |  | A) gammaIotFp0 |  | 4 | gammaIotFp (IoT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) | The values are broadcasted when the related frequency partitions are used |
| O |  | A) gammaIotFp1 |  | 4 |  |  |
| O |  | B) gammaIotFp2 |  | 4 |  |  |
| O |  | C) gammaIotFp3 |  | 4 |  |  |
| M |  | DA) Alpha |  | 3 | alpha () is the factor according to the number of receive antennas at the ABS. It is 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} | NA |
| M |  | E) Beta |  | 1 | It is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. 0: disable 1: enable | NA |
| M |  | F) dataSinrMin |  | 4 | dataSinrMin is the SINR requirement for the minimum data rate expected by ABS. SINRmin_Data has 4 bits to represent the value in dB among{-INF ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} | NA |
| M |  | G) dataSinrMax |  | 4 | dataSinrMax is the maximum SINR threshold defined by ABS. SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 34, 36, 38, 40} | NA |
| M |  | UlpcControlChannelSet | targetHarqSinr | 4 | It is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB | NA |
| M |  |  | targetSyncRangingSinr | 4 | It is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB | NA |
| M |  |  | targetPfbchSinr | 4 | It is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  |  | targetSfbchBaseSinr | 4 | It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. | NA |
| M |  |  | targetSfbchDeltaSinr | 3 | It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} | NA |
| M |  |  | targetBwRequestSinr | 4 | It is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  |  | gammaIotSounding | 4 | It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. | NA |
| M |  |  | soundingSinrMin | 4 | It is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} | NA |
| M |  |  | soundingSinrMax | 4 | It is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} | NA |
| M |  | UlpcControlChannelSet |  | 35 |  |  |
| M |  | targetHarqSinr |  | 4 | It is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB | NA |
| M |  | targetSyncRangingSinr |  | 4 | It is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB | NA |
| M |  | targetPfbchSinr |  | 4 | It is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  | targetSfbchBaseSinr |  | 4 | It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. | NA |
| M |  | targetSfbchDeltaSinr |  | 3 | It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} | NA |
| M |  | targetBwRequestSinr |  | 4 | It is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB | NA |
| M |  | gammaIotSounding |  | 4 | It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. | NA |
| M |  | soundingSinrMin |  | 4 | It is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} | NA |
| M |  | soundingSinrMax |  | 4 | It is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} | NA |
| O |  | SoundingMultiplexingType | decimationValueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
|  |  |  | maxCyclicShiftIndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) |  |
| O |  | SoundingMultiplexingType |  |  |  |  |
| O |  | A) decimationValueD |  | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
| O |  | B) maxCyclicShiftIndexP |  | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
| O |  | RELAY_Support_Parameters | 16m_Relay_zone_AMS_allocation_indicator | 1 | 0 – 1 | Relay support parameters |
| O |  | RELAY_Support_Parameters |  |  |  |  |
| O |  | A) 16m Relay zone AMS allocation indicator |  | 1 | 0 – 1 | Relay support parameters |
| O |  | E-MBS Configuration Parameters | Zone_Allocation-Bit-MAP | DSACMAX-1 | See section 16.9.3.1. | Present when E-MBS is configured |
|  |  |  | ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. |  |
|  |  |  | MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) |  |
|  |  |  | E-MBS AAI frame offset | variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits |  |
| O |  | E-MBS Configuration Parameters |  |  |  |  |
| O |  | Zone_Allocation-Bit-MAP |  | DSACMAX-1 | See section 16.9.3.1. | Present when E-MBS is configured |
| O |  | ZF |  | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. | Present when E-MBS is configured |
| O |  | MSI Length (NMSI) |  | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) | Present when E-MBS is configured |
| O | O | uLFeedbackInfo | primaryCarrierIndex | 6 | the primary carrier index | Loop of NumPC |
|  | M |  | startDLRUIndex |  | the start DLRUs index for feedback channel | Loop of NumPC |
|  | M |  | dLRUNum |  | the number of DLRUs for feedback channel | Loop of NumPC |
|  | M |  | hARQChannelNum |  | the number of HARQ feedback channel per HARQ feedback region | Loop of NumPC |
| O |  | uLFeedbackInfo |  |  |  |  |
| O |  | A) primaryCarrierIndex |  | 6 | the primary carrier index | Loop of NumPC |
| M |  | B) startDLRUIndex |  |  | the start DLRUs index for feedback channel | Loop of NumPC |
| M |  | C) dLRUNum |  |  | the number of DLRUs for feedback channel | Loop of NumPC |
| M |  | D) hARQChannelNum |  |  | the number of HARQ feedback channel per HARQ feedback region | Loop of NumPC |

------------------------------------------------- End of proposed text XIII--------------------------------------------------