| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Property of Assignment A-MAP IEs (16.3.6) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Email: lei.huang@sg.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a: P802.16m/D5 |  |
| Abstract | The contribution clarifies the property of assignment A-MAP IEs |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Property of Assignment A-MAP IEs

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

According to [1], assignment A-MAP IEs are grouped together based on channel coding rate. In an assignment A-MAP group, all multicast assignment A-MAP IEs shall occupy a contiguous set of MLRUs starting from the beginning of the assignment A-MAP group, followed by unicast assignment A-MAP IEs. If broadcast assignment A-MAP IEs are present in the assignment A-MAP group, the multicast assignment A-MAP IEs are located right after the broadcast assignment A-MAP IEs. So it is necessary to clarify the property of assignment A-MAP IEs. In other words, we need to know which types of assignment A-MAP IE belong to broadcast, multicast and unicast assignment A-MAP IEs, respectively.

- Property of Assignment A-MAP IEs

We suggest that the property of an assignment A-MAP IE is determined according to CRC mask used for the assignment A-MAP IE.

If the CRC mask used for an assignment A-MAP IE refers to either STID or RAID, this assignment A-MAP IE should belong to unicast assignment A-MAP IE. This is because both STID and RAID are specific to a particular AMS. That is to say, DL/UL Basic Assignment A-MAP IE, DL/UL Subband Assignment A-MAP IE, Feedback Allocation A-MAP IE, UL Sounding Command A-MAP IE, CDMA Allocation A-MAP IE, DL/UL Persistent Allocation A-MAP IE, and Feedback Polling A-MAP IE belong to unicast assignment A-MAP IE.

If the CRC mask used for an assignment A-MAP IE refers to group ID, this assignment A-MAP IE should belong to multicast assignment A-MAP IE. This is because group ID is specific to a particular group of AMSs. That is to say, Group Resource Assignment A-MAP IE belongs to multicast assignment A-MAP IE.

In addition, it is easy to understand that Broadcast Assignment A-MAP IE belong to broadcast assignment A-MAP IE. As for BR-ACK A-MAP IE, according to [1], each AMS should try to decode all BR-ACK A-MAP IEs at the DL frame where the BR ACK timer expires after it transmitted a BR preamble sequence. This implies that an AMS may not try to decode any BR-ACK MAP IEs at the DL frame where the BR ACK timer does not expire after it transmitted a BR preamble sequence. In other words, BR-ACK A-MAP IE is actually not a broadcast assignment A-MAP IE. Instead, BR-ACK A-MAP IE can be viewed as a kind of multicast assignment A-MAP IE.

Table 1 show the property of different types of assignment A-MAP IEs.

Table 1: Property of assignment A-MAP IE types

| Assignment A-MAP IE type | CRC mask | Property |
| --- | --- | --- |
| DL/UL Basic Assignment A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| DL/UL Subband Assignment A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| Feedback Allocation A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| UL Sounding Command A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| CDMA Allocation A-MAP IE | 1¦Masking Code, where 15-bit Masking Code is RAID | Unicast |
| DL/UL Persistent Allocation A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| Group Resource Allocation A-MAP IE | 0001¦Masking Code, where the decimal value of 12-bit Masking Code ( i.e., group ID) is in the range of [2 64] | Multicast |
| Feedback Polling A-MAP IE | 0000¦Masking Code, where 12-bit Masking Code is STID | Unicast |
| BR-ACK A-MAP IE | 0001¦Masking Code, where the decimal value of 12-bit Masking Code is 1 | Multicast |
| Broadcast Assignment A-MAP IE | 0001¦Masking Code, where the decimal value of 12-bit Masking Code is 0 | Broadcast |

- Reference

- [1]	IEEE P802.16m/D5, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, Apr 2010

- Proposed Modifications

- Editorial notes

- Current text in P802.16m/D5

Remove text from P802.16m/D5

- Add new text to P802.16m/D5

- -------------------------------------------------------------------------------------------------------------------

- To make the following modification to Table 845 in Page 541

Table 845: Assignment A-MAP IE Types

| A-MAP IE type | Usage | Property |
| --- | --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE | Unicast |
| 0b0001 | UL Basic Assignment A-MAP IE | Unicast |
| 0b0010 | DL Subband Assignment A-MAP IE | Unicast |
| 0b0011 | UL Subband Assignment A-MAP IE | Unicast |
| 0b0100 | Feedback Allocation A-MAP IE | Unicast |
| 0b0101 | UL Sounding Command A-MAP IE | Unicast |
| 0b0110 | CDMA Allocation A-MAP IE | Unicast |
| 0b0111 | DL Persistent Allocation A-MAP IE | Unicast |
| 0b1000 | UL Persistent Allocation A-MAP IE | Unicast |
| 0b1001 | Group Resource Allocation A-MAP IE | Multicast |
| 0b1010 | Feedback Polling A-MAP IE | Unicast |
| 0b1011 | BR-ACK A-MAP IE | Multicast |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast |
| 0b1101 | Reserved | NA. |
| 0b1101 | Reserved | NA. |
| 0b1111 | Extended Assignment A-MAP IE | NA. |

- -------------------------------------------------------------------------------------------------------------------