| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination (16.2.3, 16.2.3.47, 16.5.2.1) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Wooram Shin, Jihyung Kim, Kwang Jae Lim ETRI | E-mail: w.shin@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | The modifications on UL single BS precoding with multi-BS coordination are proposed. |  |
| Purpose | To discuss and adopt the proposed texts in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes for the UL Single BS Precoding with Multi-BS Coordination (16.2.3, 16.2.3.47, 16.5.2.1)

Wooram Shin, Jihyung Kim, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], there need some modifications for the UL single BS precoding with multi-BS coordination in order to correct errors due to change of its message name, non-unified mathematical notation, and inconsistency with control information supported.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

Remedy #1: Change Table 679 in Section 16.2.3 on page 75 as follows:

------------------------------------------------------Start of Proposed Texts------------------------------------------------------

Table 679—MAC Control Messages

| No. | Functional Areas | Message names | Message description | Security | Connection |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| 60 |  | AAI_MultiBS_PMI_COMAAI_UL_MultiBS_ MIMO_SBP | Multi-BS PMI combinationUL multi-BS MIMO for single BS precoding with multi-BS coordination | NULL | Unicast |
| … | … | … | … | … | … |

------------------------------------------------------End of Proposed Texts------------------------------------------------------

Remedy #2: Change the texts and Table 748 in Section 16.2.3.47 on page 200 as follows:

------------------------------------------------------Start of Proposed Texts------------------------------------------------------

16.2.3.47 AAI_MultiBS_PMI_COMAAI_ UL_MultiBS_MIMO_SBP

The AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format is defined in Table 7348—. This message is used by ABS to indicate AMS the PMIminmin and PMI combination ratio (PCR) to AMS for UL single BS precoding with multi-BS coordination.

Table 748—AAI_MultiBS_PMI_COMAAI_UL_MultiBS_MIMO_SBP message format

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_MULTI_BS_PMI_COMAAI_UL_MultiBS_MIMO_SBP | N.A. |
| M | PMIMinmin | 4 to 6 | PMI from the rank-1 base codebook generatesgenerating minimum interference forto the neighboring ABS | N.A. |
| M | PCR | 2 | PMI combination ratio | N.A. |

------------------------------------------------------End of Proposed Texts------------------------------------------------------

Remedy #3: Change the texts and Table 971 in Section 16.5.2.1 on pages 786-787 as follows:

------------------------------------------------------Start of Proposed Texts------------------------------------------------------

16.5.2.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques applicable to UL MIMO modes 2 and 4 withof adaptive codebook precoding with rank-1, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

16.5.2.1.1 DL Signaling

Table 971 shows the control parameters for UL multi-BS MIMO.

Table 971—Control parameters for UL Multi-BS MIMO

| Parameter | Description | Value | Control channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| PMImin | PMI minimizing interference to the neighboring ABS | 4 bits if Nt=2 6 bits if Nt=4 | AAI_UL_MultiBS_MIMO_SBP | Selected from the rank-1 base codebook defined in <<16.3.10.3>> |
| PCR | PMI combination ratio | 0b00: 0.125 0b01: 0.375 0b10: 0.625 0b11: 0.875 | AAI_MultiBS_PMI_COM messeageAAI_UL_MultiBS_MIMO_SBP |  |

16.5.2.1.2 Operation procedure

Single BS precoding with Multi-BS Coordination is performed by combining two PMIs in TDD or FDD UL transmission for mitigating inter-cell interference (ICI) when CL MIMO precoding is applied for the serving and neighboring cells. One of PMIs maximizes the transmission power of the serving cell; another minimizes interference generated to the neighboring cell.

The operation procedure of PMI combination follows the steps below.

- PMI combination may be triggered by an ABS in an unsolicited manner. By using UL sounding signals, the serving and neighboring ABSs may measure channels of AMSs. Upon the channel measurements, the serving and neighboring ABSs may determine PMIPMI maximizing transmission power and PMIminPMImin minimizing interference, respectively. The neighboring ABS may inform the serving ABS of PMIminPMImin through the backhaul network.

Those two PMIs,  and , should be selected based on the following procedure:

(316)

(316)

where HS and HI denote information of channels from the AMS to its serving and corresponding neighboring ABS, respectively, and vi is the i-th codeword of the base codebook.

- The serving ABS shall inform the PMIPMI in the UL Basic Assignment A-MAP IE, or UL Subband Assignement A MAP-IE, or UL CSM Beamforming A-MAP IE, and PMIminPMImin and PCR in the AAI_UL_MultiBS_MIMO_SBP message to the AMS.

The transmitted precoder W combining two precoders  should be generated as follows:

(317)

(317)

------------------------------------------------------End of Proposed Texts------------------------------------------------------

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” April 2010.