| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes of the AAI_SCD Message w.r.t. UL Feedback Information for Carrier Aggregation (16.2.3.30) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Wooram Shin, Soojung Jung, Jae Sun Cha, Sung Cheol Chang, Eunkyung Kim, Chul Sik Yoon, Kwang Jae Lim ETRI | E-mail: w.shin@etri.re.kr, sjjung@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | The modifications on UL feedback channel allocation for asymmetric carrier aggregation are proposed. |  |
| Purpose | To discuss and adopt the proposed texts in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes of the AAI_SCD Message w.r.t. UL Feedback Information for Carrier Aggregation (16.2.3.30)

Wooram Shin, Soojung Jung, Jae Sun Cha, 
Sung Cheol Chang, Eunkyung Kim, Chul Sik Yoon, Kwang Jae Lim

ETRI

# Introduction

In IEEE 802.16m Amendment Draft Standard [1], the AAI_SCD message defines "uLFeedbackInfo" to support UL feedback channel allocation for carrier aggregation, but it seems that there has been an implementation error in converting its ASN.1 format proposed in [2] to a message table format, which does not consider a sequence of "uLFeedbackInfo" whose size is equal to the number of primary carrier.

# Text Proposal for inclusion in the IEEE 802.16m Amendment

Note:

The text in black: the existing text in the 802.16m Amendment Draft Standard

The text in red with strike-out: the removal of existing 802.16m Amendment Draft Standard

The text in blue with under-line: the new text added to the 802.16m Amendment Draft Standard

Change Table 717 in Section 16.2.3.30 on page 159 as follows:

------------------------------------------------------Start of Proposed Texts------------------------------------------------------

Table 717—AAI_SCD Message

| M/O |  | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| … |  | … | … | … | … | … |
| O | O | uLFeedbackInfo[1…8] | primary-CarrierIn-dex | 6 | the primary carrier index | Present when multicarrier is configured with asymmetric carrier aggregation |
|  | M |  | startDL-RUIndex |  | the start DLRUs index for feedback channel |  |
|  | M |  | dLRUNum |  | the number of DLRUs for feedback channel |  |
|  | M |  | hAR-QChan-nelNum |  | the number of HARQ feedback channel per HARQ feedback region |  |

------------------------------------------------------End of Proposed Texts------------------------------------------------------

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks; Part 16: Air Interface for Broadband Wireless Access Systems; Advanced Air Interface,” April 2010.

[2] IEEE C802.16m-10/0281r2, “Feedback mechanism for asymmetric carrier aggregation (16.2.8.1),” March 16, 2010.