| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_SLP-REQ Message Cleanup (16.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #31a on P802.16m/D5 |  |
| Abstract | This contribution proposes cleanup version of AAI_SLP-REQ Message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_SLP-REQ Message Cleanup

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Section 16.2.3.24 as follows]

16.2.3.24 AAI_SLP-REQ

An AMS in Active Mode may use the AAI_SLP-REQ message with Request_Code = 0b01(i.e., Enter Sleep Mode) to request a permission to enter Sleep Mode. The AMS in Sleep Mode can change the Sleep Cycle settings by transmitting AAI_SLP-REQ with Operation = 0b10 (i.e., Change Sleep Cycle settings). The AMS in Sleep Mode can exit from Sleep Mode by transmitting AAI_SLP-REQ with Operation = 0b00 (i.e., Exit from Sleep Mode).

Table 711 – Parameters for AAI_SLP-REQ

| M/O |  | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M |  | Control Message Type |  | 8 | MAC control message type | N.A. |
| M |  | Operation |  | 2 | This indicates operation request type of AAI_SLP-REQ message 0b00: Exit from Sleep Mode 0b01: Enter Sleep Mode 0b10: Change Sleep ModeCycle setting 0b11: Switch Sleep Cycle setting | N.A. |
| O |  | SCID |  | 34 | Sleep Cycle ID 0~15 | When (operation != 0b00) |
| O |  | Start Frame Number |  | 6 | 0~63 | When (operation != 0b00) |
| O | M | M | TIMF | 1 | Traffic Indication Message Flag 0: AAI_TRF-IND message is not sent for the AMS 1: AAI_TRF-IND message is sent to the AMS during every Listening Window 0~1 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | LWEF | 1 | Listening Window Extension Flag 0~1 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | NSCF | 2 | Next sleep cycle indicator 0b00 = Reset to Initial Sleep Cycle 0b01 = min ( 2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved 0b00 ~ 0b11 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | Initial Sleep Cycle | 4 | This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). 0~15 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | Final Sleep Cycle | 10 | This indicates assigned duration for the Final Sleep Cycle (measured in frames). 0~1023 | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | Listening Window | 6 | Assigned duration of AMS’s default Listening Window (measured in frames). This Listening_Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled. | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | M | M | Listening sub-frame bitmap | 18 | The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. The size of the bitmap equals to the number of subframes of a frame. Bit #0 is mapped to the first AAI subframe. Each bit in the bitmap indicates: 0: AMS does not wake up at the specific AAI subframe 1: AMS wakes up at the specific AAI subframe | When (operation == 0b01 ¦¦ operation == 0b10) |
|  | O | O | New Initial Sleep Cycle | 5 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle) | When (operation == 0b01 ¦¦ operation == 0b10) && (NSCF == 0b10) |
|  | O | O | T_AMS | 5 | This timer is for Listening Window Extension of AMS. 0~31 | When (operation == 0b01 ¦¦ operation == 0b10) && (LWEF == 1) |

Parameters shall be as follows

Operation

This indicates operation request type of AAI_SLP-REQ message.

0b00 = AAI_SLP-REQ message is transmitted to exit from sleep Mode

0b01 = AAI_SLP-REQ message is transmitted to enter sleep Mode

0b10 = AAI_SLP-REQ message is transmitted to change the existing sleep Cycle changesetting

0b10 = AAI_SLP-REQ message is transmitted to switch Sleep Cycle setting which has been negotiated since the AMS entered Sleep Mode

SCID

Assigned Sleep Cycle identifier. The ID shall be unique within the AMS. This ID may used in further AAI_SLP-REQ/RSP message for changing/switching the Sleep Cycle setting

LWEF

Listening window Extension Flag. If LWEF = 0, it indicates that the Listening window is of fixed duration. Otherwise, it is extensible.

TIMF

1 = ABS is requested to transmit an AAI_TRF-IND message during the AMS's Listening Window. When the ABS has DL pending unicast traffic for the AMS, the ABS shall inform the AMS of positive traffic indication via AAI_TRF-IND message.

0 = Traffic Indication via AAI_TRF-IND is not required

NISCF

This indicates the inclusion of New Initial Sleep Cycle in AAI_SLP-REQ message.

Start_ Frame_ Number

Start frame number for first Sleep Cycle. This represents the 6 significant bits of frame number in which AMS wants to enter the first sleep state in Sleep Mode. .

Initial_ Sleep_ Cycle

This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). The length of Initial Sleep Cycle shall be equal or longer than the default Listening Window.

Final Sleep Cycle

This indicates assigned duration for the Final Sleep Cycle (measured in frames).

Listening_ Window

Assigned duration of AMS's default Listening Window (measured in frames). This Listening_ Window may be extended as long as there is UL/DL data traffic between AMS and ABS when Listening Window Extension is enabled

Listening sub-frame bitmap

The bitmap indicates the sub-frames in each frame where the AMS needs to remain awake. Most significant bit is mapped to the 1st AAI subframe.

If this value is set to 0xF0xFF, the AMS shall remain awake during entire sub-frames in each frame during Listening Window.

New Initial Sleep Cycle

When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle shallmay be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle).

T_AMS

This timer is required in AMS for Listening Window Extension. If LWEF =1, it shall be included in AAI_SLP-RSQREQ.

---------------------------------------------------------End of the Text----------------------------------------------------------