| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup of Multiplexing Section (16.2.4) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #31a on P802.16m/D5 |  |
| Abstract | This contribution proposes text cleanup on multiplexing. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup of Multiplexing Section

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Section 16.2.4.2 as follows]

16.2.4.2 Multiplexing

Multiple connections' payload associated with same security association can be multiplexed and encrypted together in a MAC PDU. If 'n' connections are multiplexed, one MEH and 0 to 'n' FEH, PEHFPEH and/or MCEH shall be present in a MAC PDU. The AGMH (as defined in 16.2.2.1.1) and the MEH (as defined in 16.2.2.2.4) carries the information about the Flow IDsIds and Lengths of the connection payloads. The FEH, PEHFPEH and MCEH carries the information about the transport and control connection payload respectively. For example multiple connections' payloads which are encrypted using AES CCM can be multiplexed and encrypted together in a MAC PDU. Figure 389 illustrates the multiplexing of two connection payloads which are associated with same security association (i.e., AES CCM).). FEHx/PEHxFPEHx and FEHy/PEHyFPEHy in Figure 389 carry the information about the SDU/SDU fragments in the Payload-X and Payload-Y respectively. Figure 390 illustrates the usage of FEH/PEHFPEH/MCEH together with MEH in a MAC PDU.

Figure 389 – Multiplexing of connection payload associated with same SA

Figure 390 – Usage of FPEHFEH/PEH/MCEH and MEH in MAC PDU

---------------------------------------------------------End of the Text----------------------------------------------------------