| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Contiguous MAC PDU Length Extension |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot Recirc #31a / Topic: EH Group Format (Section 16.2.2.2) |  |
| Abstract | This contribution proposes changes to EH group format to facilitate contiguous large MAC PDU length |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Contiguous MAC PDU Length Extension

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

In the IEEE P802.16m/D5 specification [1] MLEH is used for MAC PDU length extension. The 14 bit extended length (using MLEH) for MAC PDU is not contiguous and adds to processing overhead at the receiver. Also it adds to overhead of 1 byte in the large MAC PDU with no extended headers. Also in certain scenarios the 8 bit EHLen in the EH group may be a limitation when large number of short packets (for e.g. TCP Acks) are packed in the MAC PDU.

# Proposal

The first byte in the EH group format is modified as shown in Figure 1.

# Proposed text

[Change 1: Modify the section 16.2.2.2, page 55, lines 1-47 as follows]

Figure 386—Extended Header Group Format

Table 663 – Extended Header Group Fields

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAC PDU Length extension | 3 | MAC PDU with SPMH: This field is set to zeros MAC PDU with AGMH: If the extended header is present in the MAC PDU, it represents the 3 LSBs of the 14 bit MAC PDU length field. The 11 MSBs of the 14 bit MAC PDU length is present in the AGMH. |
| ELI | 1 | Extended header group length indicator: If set to ‘0’, 4 bit Extended header Group length follows this field, otherwise 12 bit Extended header Group length follows this field. In the MAC PDU with SPMH this field is always set to ‘0’. |
| Extended header Group Length | 4 or 12 | The Extended header Group Length field indicates the total length of the extended headers following the Extended header Group Length field group, including all the extended headers and the Extended header Group length byte. |
| Extended header Type | TBD | Type of extended header as defined in Table 656. |
| Extended header Body | variable | The size of the extended header is determined by extended header type as specified in Table 656. The extended header including the extended header type is byte aligned. |

[Change 2: Modify the section 16.2.2.1.1, page 46, lines 8-26 as follows]

Table 653—AGMH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Advanced Generic MAC Header() { |  |  |
| Flow ID | 4 | Flow Identifier |
| EH | 1 | Extended header group presence indicator; When set to ‘1’, this field indicates that an Extended Header group is present following this AGMH. |
| Length | 11 | This field indicates the length in bytes of MAC PDU including the AGMH and extended header if present. If MLEH is present in a MAC PDU, Length field indicates 11 LSB of length in byte of MAC PDU EH is set to ‘1’, this represent the 11 MSBs of 14 bit MAC PDU length otherwise it represents the 11 bit MAC PDU length. |
| } |  |  |

[Change 3: Delete section “16.2.2.2.10 MAC PDU length extended header (MLEH)”]

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”