| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Modulation and Coding Scheme Configuration for Enhanced Multicast Broadcast Service in IEEE 802.16m/D5 (16.7.3) |  |
| Date Submitted | 2010-5-10 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho ITRI | E-mail: zhengyanxiu@itri.org.tw |
| Re: | Call for LB #31 on “ P802.16m/D5”: Target topic: “16.7.3” |  |
| Abstract | This contribution proposes amendment text describing MCS configuration for enhanced multicast broadcast service in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Modulation and Coding Scheme Configuration for Enhanced Multicast Broadcast Service in IEEE 802.16m/D5 (16.7.3)

Zheng Yan-Xiu, Chang-Lan Tsai, Yu-Chuan Fang, Chung-Lien Ho

ITRI

- Introduction

This contribution proposes amendment text describing the support of the modulation and coding scheme (MCS) configuration for enhanced multicast broadcast service (E-MBS) in IEEE 802.16m. This support facilitates E-MBS MCS selection and reduces operation overhead. Once the E-MBS is referred, the method could be applied immediately.

- E-MBS MCS setup concern and geographical information

In an E-MBS network, the location and channel quality of an AMS is unknown. However, AMSs can move within the network and the channel quality of the AMS becomes unknown. In order to maintain the link reliability, the lowest modulation and coding scheme (MCS) is often used but significantly degrades bandwidth efficiency.

Drive test is generally used to acquire geographical information to verify if the signal quality is enough to support the service. However, drive test is expensive. If the network does not change, one-shot drive test might be affordable for an operator. For E-MBS, the E-MBS zone might be reconfigured to support different geographical location. Once the zone is re-configured, the drive test shall be performed again to measure the received signal quality.

Furthermore, the drive test of E-MBS is complex procedure. When a car performs the measurement, the network shall configure multiple ABS sending signals at the same time and a car shall drive around the zone. If the zone is large, the test takes time to measure the signal strength. If the time is long, it consumes radio resource. Comparing to single cell scenario, E-MBS deployment is hard to find an economic way.

- E-MBS MCS setup concern and geographical information

AMS can serve the measurement and an operator can save his operation cost. The operator can commend a group of AMSs within an E-MBS zone to perform measurement. The AMS can measure the signal strength to report. Once the network receives enough reports, operator can setup the associated MCS based on this geographical information. The higher layer can commends these through higher layer message but an AMS need a reference signal to measure.

There are two kinds of reference signal could be used, data and pilot. Pilot is used in all scenarios and mobile station has to measure the channel based on the pilot. Performing measurement based on the pilot would be an easy way. We also can measure through the data but an AMS shall have a general purpose measurement device to measure the signal strength of the data. Furthermore, pilot is also should be measured in advance. Pilot measurement can be specified in the standard to support this function. Except for this, the drive test procedure would be out of scope and does not need to be specified in the standard.

- Conclusion

E-MBS specific pilot measurement should be specified as an AMS function to minimize operation cost on E-MBS MCS configuration.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D5, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D5

===============   Start of Proposed Text ==========================

[Note to Editor: Add the following text after line 24 in Page 818 in 16.7.3]

- 16.7.3.3 Support of E-MBS Configuration for System Deployment

An SON server may initiate E-MBS measurement procedure before an E-MBS server launches E-MBS service. An SON server may ask ABS to send AAI_L2-XFER message to request an AMS to perform the measurement. AMS measures E-MBS Zone specific pilot patterns in 16.3.5.4.4 and reports such as RSSI, CINR, location, etc. The SON server could collect these measurements and use to calculate the suitable MCS for the E-MBS zone. The E-MBS server can launch E-MBS service based the calculated MCS.

=================== End of Proposed Text =======================