| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Operation in Multi-carrier Operation in 802.16m (16.2.8.2.10.2) |  |
| Date Submitted | 2010-4-30 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho ITRI | E-mail: zhengyanxiu@itri.org.tw |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.8.2.10.2” |  |
| Abstract | This contribution proposes amendment text describing paging operation in multi-carrier system for ABS Type in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Paging Operation in Multi-carrier Operation in 802.16m (16.2.8.2.10.2)

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho

ITRI

- Introduction

This contribution proposes amendment text describing the restriction for the paging carrier selection when a 16m ABS supports multiple carriers. The restriction avoids potential frequent location update due to mobile station capability.

- Problem Formulation

In our multi-carrier paging philosophy, each AMS can receive its own paging information on one carrier. In order to reduce the frequency of location update procedure, in the D5, HASH function is proposed for an AMS to find the associated paging carrier. If all AMSs have capability to receive paging on all carriers, the problem is perfectly resolved. However, AMS might not capability to receive all carriers and location update is forced to be performed and network loading becomes high.

Fig. 1: An example of multi-carrier paging.

Figure 1 illustrates an example. If there are two basestations, BS 1 and BS 2, BS1 operates on carrier 1, carrier 3, carrier 5 and carrier 7, and BS2 operates on carrier 2, carrier 4, carrier 6 and carrier 8. Both basestations have four MSs receiving paging information on these four carriers. If these four MSs handoff from BS1 to BS2, MS2 moves from 3.5 GHz to 2.5 GHz and MS4 moves from 2.5 GHz to 3.5 GHz. If MS2 only can not operate on 3.5 GHz or MS4 only can not operate on 2.5 GHz, location update shall be performed and network side registry should be updated. It is a disaster

In fact, in IEEE 802.16 L802.16-09/010Xr5, it highlights the band class for 802.16. In different countries, different band class could be used with different band class support. Therefore, we should consider restricting the paging carriers belonging to a band and frequent location update can be avoided in advance.

Different band has different channel characteristics, if we consider to group paging carriers across multiple bands, they might be coverage hole on higher frequency and paging information can not be received as well.

Table 1: Band Class Definition IEEE 802.16 L802.16-09/010Xr5.

| Band Class | UL AMS Transmit Frequency (MHz) | DL AMS Receive Frequency (MHz) | Duplex Mode |
| --- | --- | --- | --- |
| 1 | 2300-2400 | 2300-2400 | TDD |
| 2 | 2305-2320, 2345-2360 | 2305-2320, 2345-2360 | TDD |
|  | 2345-2360 | 2305-2320 | FDD |
| 3 | 2496-2690 | 2496-2690 | TDD |
|  | 2496-2572 | 2614-2690 | FDD |
| 4 | 3300-3400 | 3300-3400 | TDD |
| 5L | 3400-3600 | 3400-3600 | TDD |
|  | 3400-3500 | 3500-3600 | FDD |
| 5H | 3600-3800 | 3600-3800 | TDD |
| 6 | 1710-1770 | 2110-2170 | FDD |
|  | 1920-1980 | 2110-2170 | FDD |
|  | 1710-1755 | 2110-2155 | FDD |
|  | 1710-1785 | 1805-1880 | FDD |
|  | 1850-1910 | 1930-1990 | FDD |
|  | 1710-1785, 1920-1980 | 1805-1880, 2110-2170 | FDD |
|  | 1850-1910, 1710-1770 | 1930-1990, 2110-2170 | FDD |
| 7 | 698-862 | 698-862 | TDD |
|  | 776-787 | 746-757 | FDD |
|  | 788-793, 793-798 | 758-763, 763-768 | FDD |
|  | 788-798 | 758-768 | FDD |
|  | 698-862 | 698-862 | TDD/FDD |
|  | 824-849 | 869-894 | FDD |
|  | 880-915 | 925-960 | FDD |
|  | 698-716, 776-793 | 728-746, 746-763 | FDD |
| 8 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | 1785-1805, 1880-1920, 1910-193, 2010-2025, 1900-1920 | TDD |
| 9 | 450-470 | 450-470 | TDD |
|  | 450.0-457.5 | 462.5-470.0 | FDD |

- Conclusion

If an AMS camps on a paging group, all carriers shall be accessible to an AMS in case of redundant location update. For example: Paging carriers shall be confined on single band to prevent potential frequent location update due to different mobile station capability.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D5, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D5

===============   Start of Proposed Text ==========================

[Note to Editor: Add the following text after line 1 in Page 311 in 16.2.8.2.10.2]

An AMS may perform Location Update process to acquire its preferred carrier for the idle mode support when AMS cannot find the paging carrier. If different carriers have different coverage area, then the preferred carrier is used for transmitting paging message. In this case, the AMS does not follow equation (2) to derive paging carrier index. In order to avoid frequent location update process, all paging carriers belonging to a paging group shall be accessible for all AMS camping on the paging group.

=================== End of Proposed Text =======================