| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reduction on A-MAP Processing Overhead during Multi-Carrier Aggregation (16.2.8.1) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D5 |  |
| Abstract | One of the major motivations to have unpaired multi-carrier aggregation is to reduce AMS processing overhead. However, AMS will still need to process all the A-MAP in each active secondary carrier according to P802.16m/D5. This will result in huge processing overhead. It is necessary to have some way to reduce the processing overhead for A-MAP blind decoding over activated secondary carriers. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reduction on A-MAP Processing Overhead during Multi-Carrier Aggregation

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Introduction

There is a problem in current multi-carrier aggregation mode defined in P802.16m/D5. AMS will need to perform blind decoding over the A-MAP IEs transmitted over each activated secondary carriers. This will result in huge AMS processing overhead but unnecessary. The problem comes from the lack of information on when the A-MAP IE contain the resource allocation grant to AMS over the secondary carriers.

Instead of performing cross-carrier packet scheduling, ABS can simply provide the limited information regarding to A-MAP IE location which AMS need to decode over the secondary carriers. This will be sufficient to reduce the unnecessary blind decoding over the activated secondary carriers. Otherwise the power consumption by multi-carrier AMS will be huge and make it difficult to be applied over mobile handheld device.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#43, page#297 in P802.16m/D5]

In addition to partially configured carriers, the ABS may also configured DL only part of an FDD fully configured carrier to be paired and supported by the UL feedback channels on the AMS's primary carrier. When assigning one DL FDD carrier to AMS without a paired UL FDD carrier, ABS will inform AMS the A-MAP IEs in which sub-frames will contain the resource allocation information for the AMS in activated secondary carriers.

---------------------------------------------------------Start of the Text-----------------------------------------------------------