| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on General Multi-Carrier Deployment Scenarios (16.3.3.6.1) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D5 |  |
| Abstract | In P802.16m/D5, the only example on legacy support with multiple RF carriers deployment is that the legacy zone will only coexist with 16m zone by TDM manner in one carrier. The basic reason is that the legacy network operator will only gradually increase the portion of 16m zone when more and more user terminals are upgraded to dual-mode. It is not a general scenario that the legacy zone coexists with 16m over multiple carriers simultaneously. The standard text should reflect this point. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on General Multi-Carrier Deployment Scenarios

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Alex Hsu, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#54, page#436 in P802.16m/D5]

16.3.3.6.1 Support for WirelessMAN-OFDMA with multicarrier operation

In the multicarrier mode supporting WirelessMAN-OFDMA, each carrier can have either a basic frame structure (16.3.3.1) or a basic frame structure configured to support the WirelessMAN-OFDMA (16.3.3.5). Figure 484 illustrates an general scenario example of the frame structure in the multicarrier mode supporting WirelessMAN-OFDMA over one carrier. Support both AAI zone and WirelessMAN-OFDMA zone over more than one carrier is optional. In the multicarrier mode, to support WirelessMAN-OFDMA, the uplink can be also configured as TDM as defined in 16.3.3.5.

---------------------------------------------------------Start of the Text-----------------------------------------------------------