| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Multicarrier MAC Messages (16.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #31a on the Draft Amendment (IEEE P802.16m/D5) |  |
| Abstract | Clarification on the Multicarrier MAC Messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Multicarrier MAC Messages (16.2.3)

Kelvin Chou

MediaTek Inc.

Text Proposal 1

In P802.16m/D5, Section 16.2.3, insert Section 16.2.3.57 as follows and delete the entire section 16.2.8.2.3.1.1

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.57 AAI_MC-ADV (Multicarrier Advertisement) Message

The MC ABS shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table XXX — AAI_MC-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Multicarrier configuration change count | 4 | Incremented by 1 upon each update |  |
| M | Serving BS carrier number | 3 | Indicates the number of carriers the ABS carriers |  |
| M | Serving BS Uniformity Flag | 1 | 0: All Carriers supported by serving ABS have the same Protocol Version, SFH_Info 1: otherwise |  |
| M | Physical Carrier Index of current carrier | 6 | The carrier that ABS broadcast this message; the physical carrier index refers to AAI_Global-Config message |  |
| M | MAC protocol version | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m |  |
| O | Physical carrier index | 6 | Physical carrier index in AAI_Global-Config message | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 |
| O | Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier or not |  |
| O | SA-Preamble index | 10 | SA-Preamble index of the carrier |  |
| O | MAC Protocol Version (not current carrier) | 8 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 and Serving BS Uniformity Flag = 1 |
| O | SFH encoding format | 2 | 0b0: full Subpkt information 0b1: delta encoding, w.r.t. the current carrier |  |
| O | Control bitmap | 3 | Each bit maps to one SFH subpacket or extended broadcast information |  |
| O | SFH Subpkt 1 | 88 |  | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 and Serving BS Uniformity Flag = 1 and SFH_encoding_format=00 and Control_bitmap[0]=1 |
| O | SFH Subpkt 2 | 88 |  | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 and Serving BS Uniformity Flag = 1 and SFH_encoding_format=00 and Control_bitmap[1]=1 |
| O | SFH Subpkt 3 | 88 |  | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 and Serving BS Uniformity Flag = 1 and SFH_encoding_format=00 and Control_bitmap[2]=1 |
| O | Delta information | Variable | Delta encoding w.r.t. the current carrier | Shall be present for each carrier except for the current carrier when Serving BS carrier number > 1 and Serving BS Uniformity Flag = 1 and SFH_encoding_format=01 |

------------------------------------------------------  Text End  ---------------------------------------------------------------