| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Non-user specific A-MAP Extension (Section 16.3.6.5) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Hyunkyu Yu, Ji-Yun Seol, Jaeweon Cho Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-10/0032, “IEEE 802.16 Working Group Letter Ballot Recirc #31a” Target topic: “IEEE P802.16m/D5 section 16.3.6.5”. |  |
| Abstract | The contribution clarifies the non-user specific A-MAP extension. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for Non-user specific A-MAP Extension (Section 16.3.6.5)

Hyunkyu Yu, Ji-Yun Seol, Jaeweon Cho

Samsung Electronics

- Introduction

In the previous session, it was resolved that the transmission of broadcast MAC control message is signaled through broadcast assignment A-MAP IE. However, D5 still has description about the transmission of broadcast MAC control message through NUS A-MAP extension. It is required to remove the relevant text.

Also, if we want to reserve the extended non-user specific A-MAP IE for future extension, the transmission structure and the location need to be clearly described.

- Proposed Text Changes

[Remedy 1: Change the following text, at line 15 in page 536, section 16.3.6.5.2.1 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.1 Non-user-specific A-MAP IE

…

Table 841 – Non-user specific A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Assignment A-MAP size | 8 | Indicate the number of assignment A-MAPs in each assignment A-MAP group using Equation (207) to Equation (224) |
| … | … | … |
| Non-user specific A-MAP extension flag | 1 | If non-user specific A-MAP extension flag is set, it indicates that non-user specific A-MAP is extended. The extended non-user specific part uses the same PHY structure, i.e. channel encoding, modulation, and MIMO encoding, as the non-user specific A-MAP |
| Reserved | 1 | Reserved bits |

…

(224)

If the non-user specific A-MAP extension flag in the non-user specific A-MAP is set, the non-user specific A-MAP extension may be used to specify the information used to decode the PGID Info, AAI-TRF-IND, AAI-PAG-ADV, and other broadcast messages. The PHY structure for this extension is the same as the non-user specific A-MAP.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy 2: Remove the following text, at line 27 in page 374, section 16.2.17.2.3.1 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.17.2.3.1 Traffic Indication

Traffic Indication is sent for one or a group of AMS using the AAI_TRF-IND message.

The AAI_TRF-IND message shall be transmitted as described in 16.3.6.5.2.1.

…

-------------------------------------------------End proposed text----------------------------------------------------------------