| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Assignment A-MAP CRC (Section 16.3.6.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Hyunkyu Yu, Ji-Yun Seol, Jerry Pi, Sudhir Ramakrishna Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-10/0032, “IEEE 802.16 Working Group Letter Ballot Recirc #31a” Target topic: “IEEE P802.16m/D5 section 16.3.6.3”. |  |
| Abstract | The contribution clarifies the assignment A-MAP CRC procedure. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for Assignment A-MAP CRC (Section 16.3.6.3)

Hyunkyu Yu, Ji-Yun Seol, Jerry Pi, Sudhir Ramakrishna

Samsung Electronics

- Introduction

In D5, it is not clear whether an assignment A-MAP CRC uses CRC16-CCITT as defined in ITU-T recommendation X.25 or not. The generator polynomial is the same as that for CRC16-CCITT, but one’s complement operation is omitted in the current description. Thus, I recommend to simply describe the assignment A-MAP CRC by referring CRC16-CCITT.

- Proposed Text Changes

[Change the following text, at line 59 in page 523, section 16.3.6.3.2.4 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.3.2.4 Assignment A-MAP

…

A 16-bit CRC is generated based on the randomized contents of the assignment A-MAP IE. Denote the assignment A-MAP IE by m(x) = bN-1xN-1 + bN-2xN-2 + … + b1x + b0, where bN-1 is the MSB of the assignment A-MAP IE and b0 is the LSB of the assignment A-MAP IE. The 16-bit CRC is calculated as the remainder of dividing m(x) ∙ x16 by the 16-bit CRC generator polynomial g(x) = x16 + x12 + x5 + 1. The resulted CRC is denoted by p(x) = p15x15 + p14x14 + … + p1x + p0 where p15 is the MSB of the CRC and p0 is the LSB of the CRC.

The 16-bit CRC mask, denoted by q(x), is generated according to 16.3.6.5.2.4. The 16-bit CRC mask is then applied to the 16-bit CRC by bitwise XOR operation.

The masked CRC is then appended to the assignment A-MAP IE, resulting in a bit sequence of m(x) ∙ x16 + p(x)q(x). The resulting sequence of bits shall be encoded by the TBCC described in 16.3.11.2.

Coded bits can be repeated to improve the robustness of an A-A-MAP channel based on the link condition of a particular AMS.

A 16-bit CRC, for which CRC16-CCITT as defined in ITU-T recommendation X.25 is used, is generated based on the randomized contents of the assignment A-MAP IE. Denote the assignment A-MAP IE by m(x), the 16-bit CRC by p(x), and the 16-bit CRC mask by q(x) which is generated according to 16.3.6.5.2.4. The 16-bit CRC mask is applied to the 16-bit CRC by bitwise XOR operation. The masked CRC is then appended to the assignment A-MAP IE, resulting in a bit sequence of m(x) ∙ x16 + p(x)q(x).

The resulting sequence of bits shall be encoded by the TBCC described in 16.3.11.2.

…

-------------------------------------------------End proposed text----------------------------------------------------------------