| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Broadcast A-A-MAP IE (Section 16.3.6.5) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Hyunkyu Yu, Ji-Yun Seol, Hyunjeong Kang, Jaeweon Cho Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | IEEE 802.16-10/0032, “IEEE 802.16 Working Group Letter Ballot Recirc #31a” Target topic: “IEEE P802.16m/D5 section 16.3.6.5”. |  |
| Abstract | The contribution proposes clarification text on the broadcast assignment A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for Broadcast A-A-MAP IE (Section 16.3.6.5)

Hyunkyu Yu, Ji-Yun Seol, Hyunjeong Kang, Jaeweon Cho

Samsung Electronics

- Introduction

This contribution clarifies unclear procedures for the time domain repetition in the broadcast assignment A-MAP IE.

- Restrict the burst size to 600 bytes.

- Clarify the periodicity of time domain repetition and the relevant procedure.

- Proposed Text Changes

[Change the following text, at line 20 in page 592, section 16.3.6.5.2.4.13, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 862 describes the fields in a broadcast assignment A-MAP IE.

Table 862 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast-A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel allocation information |
| If (Function Index == 0){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 44 39 entries of Table 956 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Periodicity of Repetition | 1 | 0b0: 1 frame 0b1: 2 frames |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shallwill be transmitted one more time 0b10: the same burst shallwill be transmitted two more times 0b11: the same burst shallwill be transmitted three more times |
| Reserved | 13 | Reserved bits |
| } else{ |  |  |
| Reserved | TBD16 | Reserved bits |
| } |  |  |
| } else if (Function Index == 1){ |  |  |
| Number of Ranging Opportunities (N) | 2 | TBD |
| For (i=0; i<N; i++){ |  |  |
| Subframe index | 3 |  |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE.

The broadcast burst signaled by the Broadcast Assignment A-MAP IE is always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

The periodicity of the time domain repetition is one frame. During the period when a broadcast burst is transmitted repetitively in time, other broadcast bursts can only be transmitted without time domain repetition. For each broadcast burst transmission, there is a broadcast A-A-MAP IE to indicate the resource index and remaining repetition number. For each broadcast burst transmission with time domain repetition, the ABS shall transmit a broadcast A-A-MAP IE with decreasing the value of the remaining repetition number by 1 per each transmission. That is, if the ABS performs the first transmission at the k-th frame with signaling the remaining repetition number as N-1, the transmission shall be completed at the (k+N-1)-th frame or (k+2N-2)-th frame according to periodicity. The value of repetition shall be decreased by 1 per each transmission. For other fields, only the resource index can be changed if required during the period when the time domain repetition is performed. SPID is always 0 for broadcast burst transmission.

-------------------------------------------------End proposed text----------------------------------------------------------------