| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS Operation Supporting Carrier Switch Mode in IEEE P802.16m (16.9.2) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D5,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #31a: Announcement, IEEE 802.16-10/0032 |  |
| Abstract | E-MBS operation to support carrier switch mode on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS Operation Supporting Carrier Switch Mode in IEEE P802.16m (16.9.2)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS).

According to the IEEE 802.16m Amendment Draft Standard [1], E-MBS data can be transmitted via an alternative carrier, i.e., other than AMS’s primary carrier where service flows are configured the AMS is redirected to relevant carrier through DSA. During the transmission of AAI_E-MBS-CFG message and the E-MBS data to which AMS is subscribed, the AMS with only one transceiver may not be available for signaling exchange with ABS on the primary carrier. The AMS with multiple transceivers may be able to receive E-MBS data while communicating with ABS on the primary carrier.

In order to receive the AAI_E-MBS-CFG message and the E-MBS data, the AMS with only one transceiver should switch its carrier to the relevant carrier. Rest time of the transmission of AAI_E-MBS-CFG message and the E-MBS data to which AMS is subscribed, the AMS either return to the primary carrier or stay in the relevant carrier. When the ABS tries to transmit some non E-MBS data in the primary carrier but the AMS has not returned to the primary carrier, the ABS cannot transmit any traffic via the primary carrier. On the other hand, the AMS does not have to return to the primary carrier when there is no traffic to be transmitted via the primary carrier.

Therefore, in this contribution, carrier switching time between primary carrier and E-MBS carrier is proposed to support E-MBS efficiently.

# Example of Carrier Switch between Primary Carrier and E-MBS Carrier

Figure 1 shows an example of carrier switch between primary carrier and the E-MBS carrier. The AMS, which subscribed to the some specific E-MBS content, should switch to the E-MBS carrier at the moment of transmission of AAI_E-MBS-CFG message, E-MBS MAP, and the corresponding E-MBS data to receive those. As shown Figure 1, the AMS is subscribed to the E-MBS content m. The AMS switches to the E-MBS carrier at t0 and t9 to receive the AAI_E-MBS-CFG message and E-MBS MAP. After receiving the AAI_E-MBS-CFG message and E-MBS MAP, the AMS switches to the primary carrier at t4 and t13 to receive some traffic via its primary carrier. In order to receive the E-MBS data, the AMS should switch to the E-MBS carrier at t7 and it should stay in the E-MBS carrier during t7 through t8. During t4 through t7, i.e., during AMS stays in the primary carrier, the AMS can communicate with the ABS via the primary carrier. If there is no traffic to communicate each other, the AMS does not return to the primary carrier at t4 and it may return to the primary carrier at t8.

Figure 1 – an example of carrier switch between primary carrier and the E-MBS carrier

# Carrier Switch between Primary Carrier and E-MBS Carrier

- Start time to switch to the E-MBS Carrier

  - time to transmit the AAI_E-MBS-CFG message

  - time to transmit the E-MBS MAP message

  - time to transmit the E-MBS content

- return time to the primary carrier

  - after receiving the AAI_E-MBS-CFG message

  - after receiving the E-MBS MAP message

  - after receiving the E-MBS content

# Parameters to support Carrier Switch between Primary carrier and E-MBS carrier

- start time to switch to the E-MBS carrier (not necessary)

  - to receive AAI_E-MBS-CFG message, E-MBS MAP message, E-MBS content

  - AMS already knows the time t0, t0 + 20ms, t7 in Figure 1.

- ABS notifies AMS to return after receiving E-MBS MAP or E-MBS content

  - t4 ~ AMS should return to the primary carrier after receiving E-MBS MAP. AMS should switch to the E-MBS carrier at t7 to receive the E-MBS contents. AMS should return to the primary carrier at t8 after receiving the E-MBS contents.

  - t8 ~ AMS may not return to the primary carrier after receiving E-MBS MAP. AMS should return to the primary carrier at t8 after receiving the E-MBS contents.

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Modify the line49-51, page 827 in 16.9.2.1 of 802.16m/D5]

A Startreturn time parameter is included in AAI-DSA message for carrier switching mode to indicate the time the AMS performs carrier switching tofrom the E-MBS carrier for detecting and after decoding any E-MBS related traffic such as the AAI_E-MBS_CFG message, E-MBS MAP and E-MBS contents.

[Remedy2: add the following into the line 34, page 180 in 16.2.3.45.1 of 802.16m/D5]

- return time: return time which the AMS switches from the E-MBS carrier after decoding any E-MBS related traffic, only included in ABS initiated DSA-REQ. If return time is set to 0, the AMS returns after receiving E-MBS MAP or E-MBS contents immediately. If return time is set to 1, the AMS returns after receiving E-MBS contents only and it may not return after receiving E-MBS MAP.

[Remedy3: add the following into the line 56, page 188 in 16.2.3.45.2 of 802.16m/D5]

- return time: return time which the AMS switches from the E-MBS carrier after decoding any E-MBS related traffic, only included in ABS initiated DSA-REQ. If return time is 0, the AMS returns after receiving E-MBS MAP or E-MBS contents immediately. If return time is set to 1, the AMS returns after receiving E-MBS contents only and it may not return after receiving E-MBS MAP.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” April 2010